3GPP TSG-SA WG6 Meeting #55	S6-231937

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231767)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0309 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Example of MCData services which are not handled by SIP core |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The example provided in clause 7.12.2 regarding the MCData services which are not handled by the SIP core may not be an accurate one. The clause itself may lead to misunderstanding. Therefore, the CR proposes to remove this clause, and rather focus on the procedures themselves. Clause 7.12.2 is updated A note is added in the affected clauses to distinguish between the procedures when the SIP core is involved, and those when is not. For the procedures that do not interact with the SIP core, the Rx interface between MCData server and PCRF shall be used, i.e., the reference point MCData-5, as described in clause 7.5.2.3.3 and 7.5.2.2.4 where QoS is required. For the procedures such as in clause 7.5.2.2.2 and clause 7.5.2.3.2, QoS is not considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 7.12.2 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion may occur that IP connectivity service does not require SIP core. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.12.2	MCData services not handled by SIP core for network resources management

Network resources requests are sent over the Rx interface to the PCRF. The Rx interface can be used either from the SIP core or the MCData server. In some information flows the SIP core is not involved in the MCData service procedures, and the Rx interface from the MCData server must be used to trigger resources request as described in clause 9.2.2.3.3 from 3GPP TS 23.280 [5]. Examples of such procedures are file upload and download via HTTP as described in clause 7.5.2.2.2 and clause 7.5.2.3.2, respectively.

The resources management procedures related to clause 7.5.2.2.2 and clause 7.5.2.3.2 may be applied same as other MCData services related procedures, as described in 3GPP TS 23.280 [5], that is:

-	Request for unicast resources at session establishment (in this case service initiation);

-	Request for modification of unicast resources; and

-	Request for resources with shared priority.

# * * * * End of changes* * * *