3GPP TSG-SA WG6 Meeting #55	S6-231961

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231769)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0398 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing EN related to implicit affiliation to ad hoc group alert participants |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The EN Editor's note: Sending an implicit affiliation indication to the alerted participant in the ad hoc group emergency alert request is FFS. is added to indicate whether the MC service server sends implicit affiliation to the alerted participants in an ad hoc group associated to an emergency alert. However, via implicit affiliation the participants will be associated to the ad hoc group. This CR removes the EN and emphasizes that added MC service clients to an ongoing ad hoc group emergency alert (via late entry) is implicitly affiliated to that ad hoc group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - removing the EN in clause 10.10.3.3.1 - update clause 10.10.3.3.3 such that newly added MC service clients to an ongoing ad hoc group emergency alert is implicitly affiliated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The participants of an ad hoc group emergency alert are not affiliated nor associated to the ad hoc group established for the emergency alert. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.3.3.1, 10.10.3.3.2, 10.10.3.3.4 and 10.10.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on S6-231513, so the latter needs to be implemented before this presented CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.10.3.3.1	Ad hoc group emergency alert initiation

Figure 10.10.3.3.1-1 illustrates the procedure for the MC service client initiating an ad hoc group emergency alert initiated by an authorized MC service user.

Pre-conditions:

1.	The MC service user at MC service client 1 wants to send an ad hoc group emergency alert to the MC service users who are satisfying certain criteria.

2.	All MC service clients belong to the same MC system.

Figure 10.10.3.3.1-1: Ad hoc group emergency alert initiation

1.	The MC service user at the MC service client 1 initiates an ad hoc group emergency alert to an ad hoc group with participants satisfying specific criteria. The MC service user may add additional information describing the reason for sending the alert.

2.	MC service client 1 initiates the ad hoc group emergency alert by sending the ad hoc group emergency alert request containing the details of the criteria to be applied by the MC service server for determining the participants list or the list of participants. MC service client 1 may set its MC service emergency state. The MC service user at MC service client 1 may select a functional alias used for the MC service group emergency alert. If MC service client 1 is in MC service emergency state, the MC service emergency state is retained by the MC service client 1 until explicitly cancelled.

3.	MC service server checks whether the MC service user of MC service client 1 is authorized for initiation of ad hoc group emergency alerts and whether the request is supported. The MC service server checks whether the provided functional alias, if present, can be used and has been activated for the user.

4.	The MC service server determines the list of participants to be invited for the ad hoc group emergency alert based on the information present in the information element Criteria for determining the participants. This information element could carry either criteria or indicator identifying the criteria or combination of both. The MC service server stores the ad hoc group ID together with the list of participants for the duration of the ad hoc group emergency alert. The MC service server considers the alerted participants as implicitly affiliated to the ad hoc group.

5.	The MC service server sends the ad hoc group emergency alert response to the MC service user 1 to confirm the ad hoc group emergency alert request is granted. The MC service server adds the ad hoc group ID (generated by the MC service server in the case where the MC service ad hoc group ID created by the MC service client 1 is not acceptable or the case where the MC service ad hoc group ID was not provided by MC service client 1).

6.	The MC service server sends an ad hoc group emergency alert request including the ad hoc group ID towards the MC service clients 2 and 3.

7.	MC service users are notified of the ad hoc group emergency alert. The functional alias of the MC service user initiating the ad hoc group emergency alert and additional information related to the alert may be displayed.

8.	The receiving MC service clients send the ad hoc group emergency alert response to the MC service server to acknowledge the ad hoc group emergency alert. The MC service server considers the alerted participants as implicitly affiliated to the ad hoc group.

9.	It is configured, whether any MC service client may setup a subsequent ad hoc group call, using the ad hoc group ID from the ad hoc group emergency alert, according to 3GPP TS 23.379 [16].

NOTE 1:	MC service client 1 may start the subsequent ad hoc group call after step 5.

NOTE 2:	Sending the emergency alert without making a request to also start an emergency call does not put the group into the in-progress emergency state.

NOTE 3:	Sending the emergency alert does not put the other participants in the group into an MC service emergency state.

The MC service server continuously checks whether other MC service clients meet or if participating MC service clients no longer meet the criteria for the ad hoc group emergency alert.

# * * * * Second change * * * *

##### 10.10.3.3.2	Ad hoc group emergency alert cancel

Figure 10.10.3.3.2-1 illustrates the procedure for the MC service client cancelling an ad hoc group emergency alert associated with an ad hoc group i.e., MC service users on MC service client 1, MC service client 2 and MC service client 3 belong to the same ad hoc group.

Pre-conditions:

1.	The MC service client 1 is authorised to cancel the ad hoc group emergency alert (as configured in the user profile data according to 3GPP TS 23.379 [16], 3GPP TS 23.281 [4] or 3GPP TS 23.282 [5]).

2.	The initiating MC service client 1 is a participant of the ad hoc group associated with the ongoing emergency alert.

Figure 10.10.3.3.2-1: Ad hoc group emergency alert cancel

1.	The MC service user at the MC service client 1 initiates an ad hoc group emergency alert cancel to inform the MC service server that alert is to be cancelled.

NOTE:	The ad hoc group emergency alert cancel request can carry an indication to also request that the in-progress emergency of the group is to be cancelled. The MC service server can accept or deny the request to cancel the in-progress emergency state of the group, separately from accepting or denying the request to cancel the emergency alert at MC service client 1.

2.	MC service client 1 sends an ad hoc group emergency alert cancel request to the MC service server. If the cancel request is for another MC service user that initiated this emergency alert, that user’s MC service ID shall be included in the request.

3.	MC service server uses the ad hoc group ID to select the participants of that ad hoc group.

4.	The MC service server sends the ad hoc group emergency alert cancel response to the MC service client 1 to confirm the ad hoc group emergency alert cancel request.

5.	The MC service server sends an ad hoc group emergency alert cancel request towards the participants of the ad hoc group associated with the ad hoc group emergency alert.

6.	MC service users are notified of the ad hoc group emergency alert cancellation and the MC service emergency state is cleared

7.	The receiving MC service clients send the ad hoc group emergency alert cancel response to the MC service server to acknowledge the ad hoc group emergency alert cancel. The MC service server deaffiliates the MC service client 2 and MC service client 3 from the ad hoc group associated to the emergency alert.

If there is no other communication in the ad hoc group or once the communication in the ad hoc group is terminated, the ad hoc group ceases to exist.

# * * * * Third change * * * *

##### 10.10.3.3.3	Entering an ongoing ad hoc group emergency alert

During an ongoing ad hoc group emergency alert, the MC service server continuously checks if additional MC service users meet the conditions of the ad hoc group emergency alert.

As illustrated in Figure 10.10.3.3.3-1 MC service client is added to the ad hoc group as the MC service client meets the criteria for receiving an ongoing ad hoc group emergency alert.

Pre-conditions:

1.	An ad hoc group emergency alert has been initiated and the related ad hoc group exists.

2.	The criteria for entering an ad hoc group emergency alert are known to the MC service server.

Figure 10.10.3.3.3-1: Entering an ongoing ad hoc group emergency alert

1.	MC service server acquires the latest information of the MC service user at the MC service client and checks whether the criteria for initiating an ad hoc group emergency alert to the MC service client are met.

2.	The MC service server sends an ad hoc group emergency alert request notification to MC service client to notify that it has moved into potential danger. The MC service client is added to the emergency ad hoc group.

3.	MC service client notifies the MC service user about the ad hoc group emergency alert.

4.	The receiving MC service client sends the ad hoc group emergency alert response to the MC service server to acknowledge the ad hoc group emergency alert. The MC service server considers the MC service client 1 to be implicitly affiliated to the ad hoc group associated to the emergency alert.

5.	If there is a related ad hoc group emergency call ongoing, the MC service server adds the MC service client to the list of ad hoc group call participants according to 3GPP TS 23.379 [16].

Editor's note:	It is FFS whether a notification to the ad hoc group emergency alert participants is sent about entering the alert.

# * * * * Fourth change * * * *

##### 10.10.3.3.4	Leaving an ad hoc group emergency alert

During an ongoing ad hoc group emergency alert, the MC service server continuously checks if MC service users still meet the conditions of the emergency alert.

As illustrated in Figure 10.10.3.3.4-1 MC service client is removed from the ad hoc group associated with an ongoing ad hoc group emergency alert as the MC service client does not meet the criteria for the alert any longer and the emergency alert is removed from the MC service client.

Pre-conditions:

1.	An ad hoc group emergency alert has been initiated and the related ad hoc group exists.

2.	The MC service server is aware of the criteria for leaving an ad hoc emergency alert.

Figure 10.10.3.3.4-1: Leaving an ad hoc group emergency alert

1.	MC service server acquires the latest information of the MC service user at the MC service client and checks whether the criteria for the ad hoc group emergency alert are still met.

2.	The MC service server sends an ad hoc group emergency alert cancel request to MC service client. The MC service client is removed from the ad hoc group.

3.	MC service client notifies the MC service user that the emergency alert is cancelled.

4.	The receiving MC service client sends the ad hoc group emergency alert cancel response to the MC service server to acknowledge that the ad hoc group emergency alert is cancelled for this MC service user. The MC service server deaffiliates the MC service client 1 from the ad hoc group associated to the emergency alert.

NOTE:	If a communication is ongoing in the ad hoc group for the MC service user, the MC service server will wait to remove the MC service user from the ad hoc group until the communication is terminated.

Editor's note:	It is FFS whether a notification to the ad hoc group emergency alert participants is sent about leaving the alert.

# * * * * End of changes * * * *