3GPP TSG-SA WG6 Meeting #55	S6-231979

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231710)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0179 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct MCVideo ad hoc group call information tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are various errors in 7.19.2 information flows that need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corret the MCVideo ad hoc group ID IE description in table 7.19.2.2 that it can be provided by either the group call initiator or assigned by the MCVideo server which makes the description consistent across all MC services specs. - Void tables 7.19.2.3 and 7.19.2.6. The call request and response messages are always between the MCVideo server and MCvideo client, not between MCVideo servers. - Correct the MCVideo ID IE description in table 7.19.2.4 When MCVideo server is sending a group call request to the members the MCVideo ID needs to be the targeted group member not the group call initiating party. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will give the wrong guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.2.2, 7.19.2.3. 7.19.2.4, 7.19.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.19.2.2	Ad hoc group call request return (MCVideo server – MCVideo client)

Table 7.19.2.2-1 describes the information flow ad hoc group call request return from the MCVideo server to the MCVideo client. This response could be intermediate response to provide the server assigned MCVideo ad hoc group ID.

Table 7.19.2.2-1 Ad hoc group call request return information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ad hoc group ID (NOTE) | O | The MCVideo group ID to be associated with the ad hoc group call which is either provided by the group call initiator or assigned by the MCVideo server.This information element shall be present if the authorization result is success. |
| Pre-configured group ID (NOTE) | O | The preconfigured MCVideo group ID whose group configuration is to be used for the ad hoc group call (e.g. information for end-to-end security). |
| Authorization result | M | Indicate if authorization is success or failure |
| NOTE: These IEs are included only when the Authorization Result IE indicates authorization is successful. |  |  |

* * * Next Change * * * *

#### 7.19.2.3	Void

* * * Next Change * * * *

#### 7.19.2.4	Ad hoc group call request (MCVideo server – MCVideo client)

Table 7.19.2.4-1 describes the information flow ad hoc group call request from the MCVideo server to the MCVideo client.

Table 7.19.2.4-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | M | The MCVideo ID of the ad hoc group member |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID to be associated with the ad hoc group call |
| SDP offer | M | Media parameters of MCVideo server |
| Broadcast indicator (NOTE) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (NOTE) | O | Indicates that the ad hoc group call request is an MCVideo imminent peril call |
| Emergency Indicator (NOTE) | O | Indicates that the ad hoc group call request is an MCVideo emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| NOTE: If used, only one of these information elements is present. |  |  |

* * * Next Change * * * *

#### 7.19.2.6	Void