3GPP TSG-SA6 Meeting #55	S6-231989

# Berlin, Germany, 22nd May 2023 - 26th May 2023                      (revision of S6-231950)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0348 | Rev | 2 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Call Release Reason code in the PES based Call Disconnect procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions, UKHO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An MCPTT server may need to release one or more MCPTT call legs during MCPTT call setup or during an active MCPTT call because of various reasons such as failure to establish required unicast bearers. In such error cases the MCPTT server may disconnect the impacted call leg(s) and inform the impacted MCPTT client(s) about the failure. While the existing specification provides details on MCPTT call leg disconnect procedures, additional information on the specific cause for call leg release is not provided. It’s highly desirable to provide additional information of call release to the impacted MCPTT client(s) so that the MCPTT client/user may take appropriate actions based on such information. When an MCPTT call setup using Pre-established Session, the impacted call leg is released by the MCPTT server using the call disconnect message as per Section 10.5.3.2.2 in TS 23.379. This change request proposes to addition of the Reason code value in the call disconnect message to provide additional information to the MCPTT client about call release. The Reason code will be generic field that can be used to indicate additional information to the client including normal release conditions or error conditions such as bearer failure cases described above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 10.5.2.6, added Information Element for Reason code. - In 10.5.3.2.2, added description on inclusion of Reason code in call disconnect |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible for MCPTT client/user to know the exact reason for call failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.2.6, 10.5.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Incorporated editorial review corrections Rev 2: Changed correction over corrections |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

#### 10.5.2.6	Pre-established session call disconnect request

Table 10.5.2.1.6-1 describes the information flow pre-established session call disconnect request from the MCPTT server to the MCPTT client, for the procedure defined in subclause 10.5.3.2.2 of the present document.

Table 10.5.2.6-1: Pre-established session call disconnect request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT group ID | O | Identity of the MCPTT group (see NOTE) |
| MCPTT ID | O | MCPTT ID of the private call MCPTT user (see NOTE) |
| Session ID | M | This element identifies the specific session ID used for pre-established sessions. |
| Reason code | O | Information about call disconnect reason |
| NOTE: Depending on type of connection either MCPTT group ID or MCPTT ID shall be present |  |  |

* * * 2nd Change * * * *

##### 10.5.3.2.2	Call disconnect over unicast

Figure 10.5.3.2.2-1 shows the high level procedure where a disconnect information flow is sent by the MCPTT server to the MCPTT client A to indicate that the MCPTT private call or group call using a pre-established session has been released.

Figure 10.5.3.2.2-1: Disconnect procedure

1.	Floor control is ongoing within a private or group call established using pre-established session.

2.	MCPTT server has determined to release MCPTT client A from the call e.g., due to call release.

3.	Pre-established session call disconnect request message is sent by the MCPTT server to indicate to MCPTT client A that the MCPTT private call or group call using a pre-established session has been released. MCPTT server may include a Reason code parameter to indicate additional information to the MCPTT client about the call disconnect reason. Pre-established session between MCPTT client A and MCPTT server remains for further use.

* * * End of Changes * * * *