3GPP TSG-SA WG6 Meeting #57	S6-232840

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0476 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | v | Radio Access Network |  | Core Network | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Common EAS bundle |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, the EEL supports the Common EAS selection for a given application service. As a given application service require an EAS bundle (i.e., list of EASs interacting each other for a complete service), it should be specified how to discovery and select an EAS bundle commonly for a set of UEs, noting that this issue corresponds to the objective 1-a-(iv) of the WID. To thid end, this CR proposes two changes to define and use Common EAS Bundle information as follows: - to include Common EAS Bundle information in the Application Group Profile, common EAS bundle information that consists of Bundle ID, Bundle Type, List of EAS IDs, Main EAS ID, and EAS bundle requirement. - to include Common EAS Bundle information in the EAS bundle requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Common EAS bundle information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common EAS bundle discovery and selection remains unsupported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.10, 8.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

8.2.10	EAS bundle requirements

The following IEs describe the EAS bundle requirements:

NOTE:	Information elements in the EAS Bundle Requirements are provided by the ASP.

Table 8.2.10-1: EAS bundle requirements

| Information element | Status | Description |
| --- | --- | --- |
| Coordinated EAS discovery | O | Indicates if coordinated EAS discovery is required i.e., if EAS discovery request for one of the bundled EAS is processed, then EAS discovery response should include information of all the EASs belonging to the bundle. |
| Coordinated ACR | O | Indicates if coordinated ACR is required i.e., if EAS ACR is initiated for one of the bundled EAS, then ACR should be initiated for all the EASs belonging to the bundle. The IE may further indicate what actions must be taken if ACR for one or more bundled EAS fails e.g. ACR for all other EAS that are part of the bundle must be cancelled or not. |
| Affinity | O | Indicates the affinity requirement of the EAS bundle. The IE can be set to "strong" indicating that the EASs must be in the same EDN, "preferred" indicating that it is nice to have EASs in the same EDN but not essential or "weak" indicating that its not essential for EASs to be in the same EDN. |
| Common EAS bundle | O | Indicates if the selection of Common EAS bundle is required for a given group of UE. The IE also includes Common EAS information that contains Bundle ID, Bundle type, Main EASID, and list of EAS IDs. |

8.2.11	Application Group profile

An Application Group profile includes information about the Application Group used to determine common EAS.

Table 8.2.11-1: Application Group profile

| Information element | Status | Description |
| --- | --- | --- |
| Application Group ID | M | Application group identifier as defined in 7.2.11. |
| EAS ID | M | Identifier of EAS |
| Expected Group Geographical Service Area. | O | The expected location(s) (e.g. route) of the group of UEs during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| Common EAS bundle information | O | List of EAS bundle information for the commonly selected EAS |
| > Bundle ID | M | Bundle ID as described in clause 7.2.10. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| > Main EASID | O | Indicate which EAS in a common EAS bundle takes the main EAS service role. |
| > list of EAS ID | O | List of EAS ID to be invoked by the EAS for EAS driven associated EAS. |

Editor's note: It is FFS which additional IEs are required in the Application Group Profile.

Editor's note: It is FFS how the EES can determine common EAS requirements if different values than the ones provided by the “AC Service KPIs” of the corresponding AC Profile are required for optimal group performance

>>>> End of Changes <<<<