3GPP TSG-SA WG6 Meeting #62	S6-243550

Maastricht, The Netherlands, 19th – 23rd August 2024	(revision of S6-243206)

Source:	Ericsson

Title:	Application Enablement Layer AIML Capabilities for Supporting XR Services

Spec:	3GPP TS 23.700-23 V1.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

As described in Key Issue #4, the study aspects on application enablement layer capabilities usage to supporting the XR services include:

- Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

- Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

The application enablement layer capabilities of the AIML Enablement server (studied in TR 23.700-82) can be considered to use for supporting XR services. For example, AI/ML may be required for image/object recognition (as listed in the description part of KI#4), image/video processing, etc. Use cases can be found in TR 23.874 and TR 23.876 about AI/ML support XR relevant services, e.g., split AI/ML image/object recognition, split image/video processing. AIMLE server can provide assistance informaiton to the SEALDD server for deliver/distribute XR data to the split AI/ML endpoints.

This paper proposes to use the application enablement layer capabilities provided by the AIML Enablement server to support XR services for AI/ML based XR data delivery/distribution.

2. Reason for Change

Application enablement layer capabilities provided by the AIML Enablement server (studied in TR 23.700-82) can be used to support XR services for AI/ML based XR data delivery/distribution, how the application enablement AIML capability could be used for supporting the AI/ML based XR data delivery/distribution need be studied.

3. Conclusions

This pCR proposes to use the application enablement layer capabilities provided by the AIML Enablement server to support XR services for AI/ML based XR data delivery/distribution.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 V1.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS23.433: "Service Enabler Architecture Layer for Verticals (SEAL);Data Delivery enabler for vertical applications;  Stage 2".

[7]	3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[8]	3GPP TR 26.806: "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects".

[9]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[10]	3GPP TR 22.847: "Study on supporting tactile and multi-modality".

[11]	3GPP TS 23.558:"Architecture for enabling Edge Applications".

[12]	3GPP TS 23.434:"Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[13]	3GPP TR 22.856:"Feasibility Study on Localized Mobile Metaverse Services".

[14]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[15]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC) ".

[16]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[17]	3GPP TS28.538: "Management and orchestration; Edge Computing Management (ECM)".

[18]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[19]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[Y1]	3GPP TR 23.700-82: "Study on application layer support for AI/ML services".

[Y2]	3GPP TR 23.874: "Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS".

[Y3]	3GPP TR 23.876: "Study on AI/ML Model Transfer Phase 2".

[Y4]	3GPP TR 23.482: "Functional architecture and information flows for AIML Enablement Service".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

## 7.0	7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 | X |  |  |  | X |  |  | X |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 | X |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |  |
| Sol #9 |  |  |  |  | X |  |  |  |
| Sol #10 |  |  |  |  |  | X |  |  |
| Sol #11 |  |  |  |  | X |  |  |  |
| Sol #12 |  |  |  |  |  | X |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |
| Sol #X |  |  |  | X |  |  |  |  |

* * * Next Change * * * *

## 7.X	Solution #X: Application Enablement Layer AIML Capabilities for Supporting XR Services

### 7.X.1	General

The application enablement layer capabilities of the AIML Enablement server (studied in 3GPP TR 23.700-82 [Y1]) can be considered to use for supporting XR services. For example, AI/ML may be required for image/object recognition (as listed in the description part of KI#4), image/video processing, etc. Use cases can be found in 3GPP TR 23.874 [Y2] and 3GPP TR 23.876 [Y3] about AI/ML support XR relevant services, e.g., split AI/ML image/object recognition, split image/video processing. AIMLE server can provide assistance information to the SEALDD server for deliver/distribute XR data to the split AI/ML endpoints.

The following clauses specify procedures, information flows and APIs for usage of application enablement layer AIML capabilities to support XR Services.

### 7.X.2	Procedure

Pre-conditions:

-	The AIMLE Server is known by the consumer and the SEALDD Server.

-	The AI/ML services provided by the AIMLE Server is allowed to exposure to the consumer and the SEALDD Server.

-	The consumer decides AI/ML based XR service, e.g. split image/video processing (distribute image/video processing tasks to multiple split endpoints for processing).

-	Up to requirements of high layer application, SEALDD Server may trigger interaction with AIMLE Server for assistance information.

Figure 7.X.2-1: Application enablement layer AIML capabilities to support XR Services

- The consumer (e.g. VAL server, VAL client via SEALDD client) sends request to AIMLE server for assistance information on AI/ML operations for XR service, e.g. split endpoints for split AI/ML based image/video processing. The consumer gets assistance information, e.g. split AI/ML endpoints.

- The consumer sends request to the SEALDD server for supporting the AI/ML based XR service (e.g. deliver/distribute XR data to the split AI/ML endpoints). The request message contains message as defined in table 7.X.3.2-1.

- Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server responses to the VAL server. The SEALDD server derives information for the request, for example, based on the message in the request to decide whether assistance from 5GC and/or AIMLE server is needed or not, and determine downstream entities and services needed based on the information derived.

NOTE 1: The SEALDD server may decide to interact with AIMLE server for assistance information as described in step 3, if such assistance information is required according to the requirements of high layer application and not contained in the request message from the consumer in step 2. Compared with provided by the consumer, the benefits that the SEALDD server requests to AIMLE server for assistance information directly include e.g. receive assistance directly from AIMLE server with relevant low delay, support fast update of configuration with the updated assistance information directly from the AIMLE server, allow flexible request of assistance information for data delivery purposes (e.g. achievable QoS, suggestion of QoS) as the SEALDD server can adjust its request to the AIMLE server based on need.

Then the SEALDD server sends request to the AIMLE server for assistance information on AI/ML operations for the XR data delivery/distribution. For example, for split AI/ML image/video processing, the request contains the information of split AI/ML endpoints (from the request in step 1), data information (e.g. data type, data size, range of data size) to be delivered/distributed to the split endpoints, may contain QoS requirements on the data delivery/distribution, e.g. desired data delivery/distribution time. The information flow for the request reuses the solutions introduced in 3GPP TS 23.700-82 [Y1] and the mechinisms will be specified in 3GPP TS 23.482 [Y4].

The AIMLE server generates assistance information and responses to the SEALDD server with the required assistance information, e.g.

-	Time point/time window for the data delivery/distribution to the split AI/ML endpoints and satisfy the required QoS. The mechinism on support AI/ML splitting operations in enablement layer can be reused.

-	The achievable QoS with current configuration of session between UE and AF (e.g. the consumer). The mechinism on inference (by using/training and then using ML model) can be reused to generate the assistance information.

-	Suggestion of QoS values for each flow of the session in order to satisfy application requirement. The mechinism on inference (by using/training and then using ML model) can be reused to generate the assistance information.

NOTE 2:	The workflow relevant to AIMLE server for above assistance information and API design will be discussed in the normative phase.

- The SEALDD server may request and receive assistance from 5GC, as described in step 3, e.g. assistance information from NWDAF (or via NEF) about analytics on E2E data volume transfer time. In the request from SEALDD server to NWDAF for E2E data volume transfer time analytics, a pre-configured data size may be used (the pre-configured data size may be the data size provided in step 1, or be decided by the SEALDD server according to the data size range in step 1 and based on its local policy). The analytics subsceiption may be updated when the actual data size changed (the actual data size may be obtained via measurement described in Sdd_TransmissionQuality Management API).

- The SEALDD server triggers to establish connection among the consumer, SEALDD server, SEALDD client(s), and the split AI/ML endpoints (i.e. VAL client(s)) for the data delivery/distribution of the XR service. The assistance information received in steps 4 and 5 can be used by the SEALDD server separately or jointly based on need and local policy. For example:

-	The time point/time window can be used to decide the time for data delivery/distribution and satisfy QoS requirement.

-	The achievable QoS can be used to decide whether update of the current configuration is needed or not.

-	The suggestion of QoS values can be used to decide the configuration of each flow of the session.

-	The analytics from NWDAF can be used to check whether the required QoS can achieve, or adjustment is needed.

NOTE 3:	The connection establishment and data delivery/distribution procedures could use the existing solutions.

- Operations are performed at the consumer, SEALDD server, SEALDD client(s), and VAL client(s) for the AI/ML based XR data delivery/distribution, e.g. image/video/multi-modal data delivery/distribution from the consumer to the split AI/ML endpoints.

### 7.X.3	Information flows

#### 7.X.3.1	General

The following information flows are specified for supporting AI/ML based XR service request based on clause 7.X.2.

#### 7.X.3.2	Supporting AI/ML based XR service request

Table 7.X.3.2-1 describes the information flow from the consumer (e.g. VAL Server) as a request or update request for the XR service.

Table 7.X.3.2-1: Supporting AI/ML based XR service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer (e.g. VAL server ID, SEALDD client ID, VAL client ID). |
| VAL service ID | O | Identity of the VAL service. |
| VAL UE ID(s) | O | Identifier(s) of the VAL UE(s) for the XR service. |
| Data information | M | The information of the data needs to be delivered or distributed for the XR service. |
| >Data type | M | The type of data, could be e.g. image, video, multi-model data (e.g. audio, video, positioning, haptic data). |
| >Data size information | M | The maximum/minimum/average size of the data or a range of size for the data to be delivered/distributed. |
| QoS requirement | O | The QoS requirement, e.g. latency for data delivery/distribution. |
| Assistance information | O | The assistance information for deliver/distribute the data for the XR service, e.g. time point/time window for the data delivery/distribution to the split AI/ML endpoints. |
| NOTE: This information element shall not be updated. |  |  |

NOTE 1:	The XR service information in the request is used by the SEALDD server e.g. derive information and request assistance information in steps 3-5 of clause 7.X.2, check user consent for data collection.

NOTE 2:	Whether contains the IE data size will be discussed during normative phase.

#### 7.X.3.3	Supporting AI/ML based XR service response

Table 7.X.3.3-1 describes the information elements for the supporting AI/ML based XR service response from the SEALDD server to the consumer.

Table 7.X.3.3-1: Supporting AI/ML based XR service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the XR service request (positive or negative acknowledgement). |

### 7.X.4	Solution evaluation

This solution addresses KI#4 and introduces procedure on utilize of the application enablement layer AIML capabilities provided to support XR services, for the following aspects:

- Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

- Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

NOTE:	The dependency with AIMLE service will be discussed during normative phase.

# *** End of Changes ***