3GPP TSG-SA WG6 Meeting #62	S6-243130

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24xxxx)

Source:	Ericsson, FirstNet

Title:	Pseudo-CR on functional model enhancements to enable MC communication over UE-satellite-UE link

Spec:	3GPP TR 23.700-01 v 1.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

3GPP TR 23.700-29 is now 100% completed with the following conclusions related to key issue of UE-satellite-UE support:

-	The participating UEs are served by the same HPLMN

-	For call setup procedures, the IMS network (P-CSCF) determines the possibility of enabling UE-satellite-UE link based on the satellite ID and the access network information

-	It is assumed that: gNB is onboard, UPF is onboard (upon the confirmation of 3GPP SA3-LI), and AGW may or may not be onboard.

Therefore, this pCR introduces a solution for enabling UE-satellite-UE communication for MC services. It also discusses the possible enhancements on the existing functional model to enable the UE-satellite-UE link for private communication.

2. Reason for Change

This pCR introduces a solution addressing key issue #4 regarding the support of UE-satellite-UE communication.

3. Conclusions

This pCR follows the conclusion of 3GPP TR 23.700-29 and proposes a solution addressing key issue #4 regarding the support of UE-satellite-UE communication.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v 1.1.0

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TR 22.822: "Study on using satellite access in 5G; Stage 1".

[4]	3GPP TR 22.865: "Study on satellite access - Phase 3".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[9]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications"

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1"

[12]	3GPP TR 38.821: "Solutions for NR to support Non-Terrestrial Networks (NTN)"

[13]	3GPP TR38.811: "Study on New Radio (NR) to support non-terrestrial networks"

[14]	3GPP TS 23.554: "Application architecture for MSGin5G Service"

[15]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture;"

[16]	R2-2107453: "On LEO satellite flyover timing and discontinuous coverage", 3GPP TSG-RAN WG2 Meeting #115-e, August 9th - 27th, 2021

[17]	R2-2206115: "ASN.1 proposal for satellite assistance information for prediction of discontinuous coverage", 3GPP TSG-RAN WG2 Meeting #118-e, May 9th – May 20th, 2022

[23.289]	3GPP TS 23.289: "Mission Critical services over 5G System; Stage 2"

[22.261]	3GPP TS 22.261: "Service requirements over the 5G system"

[22.346]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1"

* * * Second Change * * * *

### 7.1.x	Solution #x: Functional model enhancements to enable MC communication over UE-satellite-UE link

#### 7.1.x.1	Solution description

This solution addresses key issue #4 and describes how to enable UE-satellite-UE communication for MC service UEs. It focuses on enhancements related to the functional model application plane of the MCPTT service to enable MC communication over UE-satellite-UE link. Similar enhancements can be done to MCVideo and MCData services.

The assumptions covered within this solution are as follows:

- It is considered that two MC service UEs need to establish a private communication, e.g., a private MCPTT call. The two MC service UEs are within the same home MC system and within the coverage of the same NGSO satellite (i.e., LEO or MEO).

- The average time duration of the MCPTT call is within the time the two UEs are both served by the same satellite, i.e., the start and the end of the MCPTT call happen as the two MC service UEs are within the coverage of the same satellite.

NOTE:	The average duration of the MC service UEs being served by one satellite depends on different parameters, including the size of the satellite beamer. E.g., for a satellite beamer size of 1000 km in diameter and for a stationary MC service UE (or moving with neglected speed), the MC service UE needs 132.28 seconds (about 2.2 minutes) to move to another satellite.

- The solution does not look into splitting the MC media plane between the direct path to the ground network and the UE-satellite-UE link. The assumption is to have only one path available at a time.

#### 7.1.x.2	Functional model to enable UE-satellite-UE

The UE-satellite-UE communication refers to the link between UEs under coverage of one or more serving satellites with the media plane traffic going through the satellite(s). Only the signalling plane traffic goes through the ground network. This is illustrated in figure 7.1.x.2-1.

Figure 7.1.x.2-1: Network architecture diagram as illustrated in 3GPP TS 23.700-29 [15]

As discussed in 3GPP TR 23.700-29 [15], the SIP core/IMS network determines whether it is possible to enable media plane traffic over the UE-satellite-UE link, e.g., to optimize the media path. This decision depends on several conditions, e.g., operator policy, NR satellite type, UEs capabilities and their subscription data.

Furthermore, UE-satellite-UE link plays a significant role during limited backhaul operation in isolated scenarios (IOPS), in ensuring media plane connectivity via satellite where the signalling plane is connected through the ground network, as described in 3GPP TS 22.346 [22.346] and 3GPP TS.22.261 [22.261].

Figure 7.1.x.2-2: Functional model to enable UE-satellite-UE link

To enable the media plane traffic going through the UE-satellite-UE link, the UPF and gNB are on board of the satellite as described in 3GPP TR 23.700-29 [15]. Furthermore, both media distribution function and floor control server are on board of the satellite, and they are controlled by the MC service server located in the ground. This is illustrated in figure 7.1.x.2-2. The interface(s) between both the media distribution function and floor control server and the MC service server is left for implementation.

The UE-satellite-UE link can be utilized for private communication. Session establishment and modification procedures follows the existing procedures in 3GPP TS 23.289 [23.289].

Editor’s Note:	 The possibility of having further functional entities, such as SIP core/IMS related entities, depends on SA2.

Editor’s Note:	Aspects related to security and encryption are subject to discussion within SA3.

#### 7.1.x.3	Solution evaluation

This solution discusses enabling UE-satellite-UE link for MC services. The solution proposes the necessary application plane additions to the existing functional model to enable the media traffic sent via the satellite link to the UE, and not through the ground network. The determination of media traffic over the satellite is up to the IMS network at the ground.

The solution considers a set of assumptions that are associated to enabling the media plane over the UE-satellite-UE link for two MC service users require to establish a private communication, e.g., MCPTT call that starts and ends as both MC service UEs are within the coverage of one NGSO satellite. The scenario also depends on the satellite beamer coverage as it influences the time duration the UE is served by a single satellite before performing handover to the adjacent satellite.

The solution has dependency on the work done by SA2 on the same topic.

* * * End of Change * * * *