3GPP TSG-SA WG6 Meeting #62	S6-243169

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6a240291)

Source:	Lenovo

Title:	New Solution to KI # 5

Spec:	3GPP TR 23.700-21 v1.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This solution provides a mechanism for addressing KI#5 on avatar discovery aspects.

2. Reason for Change

This solution provides a mechanism for addressing KI#5 on avatar discovery aspects.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.1.0.

* * * First Change * * * *

## 7.x	Solution #x: Discovery and requirements translation for 5G-enabled Traffic Flow Simulation and Situational Awareness

### 7.x.1	Solution description

In Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness use case (as discussed in TR 22.856), real-time information and data about the real objects can be delivered to the virtual objects of the road infrastructure and traffic participants including vulnerable road users can form a smart transport metaverse as shown in Figure 7.x.1.1.  Then real-time processing and computing can be conducted to support traffic simulation and also situational awareness and real time path guidance and real-time safety, or security alerts can be generated for ICVs as well as the driver and passengers.

Figure 7.x.1.1 Use case of 5G-enabled Traffic Flow Simulation and Situational Awareness [TR 22.856]

To support such use case, it is essential to discover the the identities of the virtual UEs (as counterparts of the physical UEs) and identify the sessions required as well as the per session requirement in both physical and virtual world.

This solution maps to KI# 5. This solution provides a mechanism applicable to mobile metaverse services (especially for 5G-enabled Traffic Flow Simulation and Situational Awareness as discussed in SA1) to support:

- the discovery of avatar and physical users for a mobile metaverse service and a given service area.

- the translation of metaverse-specific service requirements to provisioning policies and parameters for the involved application sessions within the mobile metaverse service.

NOTE: In this solution, the sessions include UE1 and UE2 and avatar counterparts of UE1 and UE2 (called as Avatar UEs for simplicity; however, referring to the digital users or virtual objects or digital twins corresponding to the UEs). Such avatar UEs may be deployed in one or more DNs.

Figure 7.x.1-2: Procedure for discovery and requirements translation

- The Mobile Metaverse Specific Server (MMSS) at the DN side sends an application service requirement message to the NRM server (including mobile metaverse enablement capabilities) for configuring the communication service/network requirements for a mobile metaverse service. This requirement comprises a set of application performance (QoE, QoS) and availability targets for the mobile metaverse service, as well as the target area and time of validity.

- The NRM Server translates the application requirement to a network communication requirement for the mobile metaverse service. Such requirement includes the communication means / methods, the traffic patterns (which can be the application traffic schedules and/or the PDU set information for the application sessions), the network topological area for which the service applies.

- The NRM Server, based on the translated network requirements, obtains information (identities/addresses, metaverse-related capabilities, and location info) for all the registered/connected UEs within the network coverage area where the metaverse service plans to be deployed. For location information, such retrieval can be based on SEAL LMS service for receiving the list of UEs and their locations in each area/zone (see clause 9.3.12 in 3GPP TS 23.434), or by querying this information from the UEs (via NRM clients) for the area of interest (see clause 9.3.4-5 in 3GPP TS 23.434).

- The NRM server upon obtaining the information on UEs and their status in a given area, detects the UEs which are metaverse-capable and are connected to the 3GPP network. Metaverse-capability is detected based on whether there is a mobile metaverse specific application at the target VAL UEs and can be defined as a capability of a VAL UE having an avatar counterpart (as virtual object or digital twin) in the metaverse world.

NOTE 1: For non-metaverse capable UEs, the NRM server may trigger the creation of a digital twin of the VAL UE to allow the interaction between this VAL UE and the virtual object (e.g. for sensor data sharing, traffic assistance). The triggering procedure and the creation of a digital twin / avatar of the VAL UE is out of scope of this solution.

- Then, the NRM server identifies the application sessions required for the end-to-end service and translate the network requirements to different per session performance requirements for expected communications among the discovered avatar - physical UEs (also considering their relative location in both digital and physical world). Such identification is based on the communication needs among Meta App residing in the VAL UEs (in the physical world), as well as communication between a Meta App in the VAL UE and the VAL server where the avatar of the VAL UE is located (e.g., EDN) for supporting the Traffic Flow Simulation and Situational Awareness use case. As example, sessions will be established for all VAL UEs (among Meta Apps in VAL UEs) and between each per of VAL UE and its avatar, within the target area of interest.

NOTE 2: For the application QoS requirements setting and coordination, this step may re-use NRM capability for coordinated QoS provisioning operation in network assisted UE-to-UE communications as in clause 14.3.5.3 of TS 23.434. This will be further explored in normative phase.

- The NRM server sends the service provisioning parameters and policies for the metaverse-specific sessions and the given area to the involved UEs and MMSS. The provisioning parameters may include per session QoS requirements for each metaverse session received/sent at/by the UE, where each metaverse session may be composed of multi-modal type of communication conveying audio, video or haptic/sensor information from/between the UE.

### 7.x.2	Architecture Impacts

This solution requires an enhanced capability at the SEAL NRM for supporting the discovery and requirements translation for metaverse services.

### 7.x.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #x.

-	MM-specific service requirements API (request / response model or subscribe/notify; API provider: SEAL NRM server; known consumer: MMSS, VAL server; corresponding to step 1).

-	MM-specific provisioning policy API (command/ notify; API provider: SEAL NRM server; known consumer: VAL UE, VAL server; corresponding to step 6).

### 7.x.4	Solution evaluation

This solution provides a mechanism for discovery and requirement translation for mobile metaverse services. This solution is only applicable to metaverse applications like “Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness” as discussed in SA1 since the sessions are for both physical and avatar UEs.

The solution is feasible and doesn’t have any dependency to other WGs.

NOTE: Possible re-use /extension of SEALDD may be considered (if necessary) for supporting some steps of this procedure.

* * * Next Change * * * *