3GPP TSG-SA WG6 Meeting #62	S6-243308

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0578 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client-triggered location reporting procedure update to remove on-demand indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The location reporting trigger information flow and Client-triggered location reporting procedure is for setting the trigger for location reporting, however it also includes on-demand case of reporting location information immediately. There is dedicated procedure for the on-demand request, hence its redundant to keep it in trigger setting procedure. The triggers in the location reporting triggers are optional but should be mandatory or conditionally present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.9.2.4: The Location reporting trigger updated to remove an indication for immediate report (on-demand) and updated IE’s to properly indicate that only triggering conditions are mandatory in the request. 10.9.3.3: Client-triggered location reporting procedure – Updated for following 1> The step 2 is updated to support only for event-triggered location reporting procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information elements are used for the client triggered location reporting and also creates confusion on which request is used for on-demand (dedicated or as part of location reporting trigger) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.4, 10.9.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.4	Location reporting trigger

Table 10.9.2.4-1 describes the information flow from the location management client to the location management server for triggering a location reporting procedure.

Table 10.9.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M (see NOTE 1) | Identity of the requesting authorized MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias | O | Functional alias that corresponds to the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID | O (see NOTE 1) (see NOTE 3) | Identity of the requested MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias | O (see NOTE 3) | Functional alias that corresponds to the requested MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID). |
| Requested non-emergency location information | O | Identifies what location information is requested, except for emergency calls or imminent peril calls or emergency alerts or in emergency state |
| Requested emergency location information | O | Identifies what location information is requested, for emergency calls or imminent peril calls or emergency alerts or in emergency state |
| Triggering criteria in non- emergency cases | O (see NOTE 2) | Identifies when the client will send the location report in non-emergency cases |
| Triggering criteria in emergency cases | O (see NOTE 2) | Identifies when the client will send the location report in emergency cases |
| Minimum time between consecutive reports | O | Defaults to 0 if absent and 0 for emergency calls, imminent peril calls and emergency alerts |
| NOTE 1: The identity of the requesting MC service user and the requested MC service user shall belong to the same MC service. E.g. if requesting MC service user is using a MCPTT ID, then the requested MC service user identity shall be an MCPTT ID. NOTE 2: At least one of these rows shall be present. NOTE 3: Either the MC service ID or the functional alias must be present. |  |  |

* * * * Next Change * * * *

#### 10.9.3.3	Client-triggered location reporting procedure

NOTE 1: This procedure is valid for single MC system operation only.

Figure 10.9.3.3-1 illustrates the high level procedure of client-triggered location reporting.

Figure 10.9.3.3-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized MC service user) sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1.

2.	Location management server checks whether location management client 2 is authorized to send a location reporting trigger for location management client 1's location information. Depending on the information specified by the location reporting trigger, the location management server initiates an event-triggered location reporting procedure for the location of location management client 1 as specified in the clause 10.9.3.1.

* * * * End of changes * * * *