3GPP TSG-SA WG6 Meeting #63	S6-244179

Hyderabad, India, 14th – 18th Oct 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0330 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Satellite access with discontinuous coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-01 v1.2.0, the KI#3 for Supporting Satellite access with discontinuous coverage has been concluded as follows: Solution #AE3 resolves open issue about application enabler (SEAL Configuration management server) obtaining the satellite coverage availability information and to retrieve unavailability period for UE (either from UE or from the 5GS), for supporting satellite discontinuous coverage. Further the solution also resolves open issue about leveraging UE unavailability period information for minimizing the service impact or adjusting the application behaviour by the SEALDD server. There is dependency with SA2 for obtaining exposure of loss of coverage event information. Solution #AE3 can be considered for normative work. So it’s needed to specify the functions, procedures and information flows for the KI#3 and Sol#AE3 in TS for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add functions of obtaining the satellite coverage availability information and unavailability period for UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The service will be impacted when it accesses the network via the satellites. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.x(new), 11.3.2.a(new) 11.3.2.b(new), 11.3.2.c(new), 11.3.2.d(new), 11.3.2.e(new), 11.3.2.f(new), 11.3.2.g(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

11.3.x	Satellite coverage availability information configuration

11.3.x.1	General

The procedures of satellite coverage availability information configuration provisioning and exposing are described in the following sub-clauses.

11.3.x.2	Procedures of the satellite coverage availability information provisioning and exposing

The procedure for VAL UE obtaining the Satellite coverage availability information configuration is illustrated in figure 11.3.x.2-1.

Pre-conditions:

-	The configuration management server has obtained the satellite related information (e.g. ephemeris) from the satellite information provider (e.g. satellite service provider or the operators).

-	The configuration management server is aware of VAL UE’s location.

Figure 11.3.x.2-1: The satellite coverage availability information provisioning and exposing procedure

1.	The configuration management server generates the satellite coverage availability information based on the obtained ephemeris for satellites and the VAL UE’s location data. If the configuration management server receives the different satellite coverage availability information from different satellite information providers for the same satellite, it may consolidate them in a unified format. The example of satellite coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [10].

2.	The configuration management server sends the satellite coverage availability information configuration request to the configuration management client to indicate whether the coverage is available for a particular location and time when the VAL UE uses the satellite access.

3.	The configuration management client acknowledges the configuration management server if it supports the satellite access.

4.	Based on the obtained ephemeris for satellites and the satellite coverage availability information, the configuration management server could retrieve the unavailability period (e.g., unavailability period duration and start time for the unavailability period) to guide the VAL UE/VAL server to minimize the service impact (e.g. indicate when to trigger the UL/DL service flow in case the VAL UE is using the satellite access).

5.	The configuration management server sends the UE unavailability period information configuration request to the configuration management client including the unavailability period per satellite for a particular location.

6.	The configuration management client acknowledges the configuration management server if it supports the satellite access. And the configuration management client may consider the UE unavailability period information (e.g. Not send UL messages during the unavailability period) when the VAL UE accesses the satellites.

7.	The VAL server may subscribe the UE unavailability period information which includes the UE identity, the trigger conditions, etc.

8.	The configuration management server notifies the VAL server the UE unavailability period information when it’s available to indicate the VAL server how to adjust the application behaviour to minimize the service impact (e.g., extend the data buffering, pending the downlink data, etc.).

* * * Next Change * * * *

11.3.2.a	Satellite coverage availability information configuration request

Table 11.3.2.a-1 describes the information flow from the configuration management server to the configuration management client to configure the satellite coverage availability information.

Table 11.3.2.a-1: Satellite coverage availability information configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Satellite coverage availability information(NOTE1) | M | Indicates when the satellite coverage is available for a particular location and time when the VAL UE using the satellite access. |
| NOTE 1: The example of satellite coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [10]. |  |  |

* * * Next Change * * * *

11.3.2.b	Satellite coverage availability information configuration response

Table 11.3.2.b-1 describes the information flow from the configuration management client to the configuration management server for the response of the satellite coverage availability information configuration request.

Table 9.3.2.a-1: Satellite coverage availability information configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Result | M | Indicates the request result. |

* * * Next Change * * * *

11.3.2.c	UE unavailability period information configuration request

Table 11.3.2.c-1 describes the information flow from the configuration management server to the configuration management client to configure the UE unavailability period information.

Table 11.3.2.c-1: UE unavailability period information configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| UE unavailability period information (NOTE1) | M | Indicates the unavailability period duration and start time for a particular location when the VAL UE using the satellite access. |
| > UE unavailability period duration | O | Indicates the unavailability period duration for a particular location when the VAL UE using the satellite access. |
| > UE unavailability period start time | O | Indicates the start time of the unavailability period for a particular location when the VAL UE using the satellite access. |
| NOTE 1: The definition of unavailability period information can refer to the clause 5.4.1.4 of 3GPP TS 23.501 [10]. |  |  |

* * * Next Change * * * *

11.3.2.d	UE unavailability period information configuration response

Table 11.3.2.d-1 describes the information flow from the configuration management client to the configuration management server for the response of the UE unavailability period information configuration request.

Table 9.3.2.a-1: UE unavailability period information configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Result | M | Indicates the request result. |

* * * Next Change * * * *

11.3.2.e	UE unavailability period information subscription request

Table 11.3.2.e-1 describes the information flow from the VAL server or configuration management client to the configuration management server to subscribe the UE unavailability period information.

Table 11.3.2.e-1: UE unavailability period information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Triggering conditions | M | Identifies when the NRM client will trigger/send the notification. |
| Time between consecutive reports | M | Indicates the interval time between consecutive reports |

* * * Next Change * * * *

11.3.2.f	UE unavailability period information subscription response

Table 11.3.2.f-1 describes the information flow from the configuration management server to the VAL server or the configuration management client to respond the subscription of the UE unavailability period information.

Table 11.3.2.f-1: UE unavailability period information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Subscription status | M | It indicates the subscription result |

* * * Next Change * * * *

11.3.2.g	UE unavailability period information notification

Table 11.3.2.g-1 describes the information flow from the configuration management server to notify the UE unavailability period information to the VAL server or the configuration management client.

Table 11.3.2.g-1: UE unavailability period information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| UE unavailability period information (NOTE1) | M | Indicates the unavailability information for a particular location when the VAL UE using the satellite access. |
| > UE unavailability period duration | O | Indicates the unavailability period duration for a particular location when the VAL UE using the satellite access. |
| > UE unavailability period start time | O | Indicates the start time of the unavailability period for a particular location when the VAL UE using the satellite access. |
| Triggering conditions | M | Identifies the conditions that triggered the sending of the notification. |
| Timestamp | O | Timestamp of the notification report. |
| NOTE 1: The definition of unavailability period information can refer to the clause 5.4.1.4 of 3GPP TS 23.501 [10]. |  |  |

* * * End of Changes * * * *