3GPP TSG-SA WG6 Meeting #63	S6-244209

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on DC application and profile information retrieval

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the DC application and profile information retrieval procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#7 and KI#2, the MMTel Enabler layer is responsible for the DC application management, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.2.4	DC application and profile information retrieval

#### 8.2.4.1	General

DC application and profile information retrieval enables a Controlling Application Server to retrieve the DC application and profile information with the MMTel Enabler Server.

#### 8.2.4.2	Procedure

Figure 8.2.4.2-1 illustrates the DC application and profile information retrieval procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.4.2-1: DC application and profile information retrieval

1.	The Controlling Application Server sends a DC Application and Profile information retrieval request to the MMTel Enabler Server. The request message includes information elements as specified in Clause 8.2.4.3.1.

2.	Upon receiving the request, the MMTel Enabler Server authorizes the Controlling Application Server if needed. If successful, the MMTel Enabler Server gets the DC application information from the DCAR. The northbound interfaces for DC Application management on DCAR is implementation-based.

3.	The MMTel Enabler Server sends a DC Application and Profile information retrieval response to the Controlling Application Server. The response message includes information elements as specified in Clause 8.2.4.3.2.

#### 8.2.4.3	Information flows

##### 8.2.4.3.1	DC Application and profile information retrieval request

Table 8.2.4.3.1-1 describes information elements for the DC Application and profile information retrieval request from the Controlling Application Server to the MMTel Enabler server.

Table 8.2.4.3.1-1: Information elements in DC Application and Profile information retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Application ID list | M | A list of Application ID for the request. |

##### 8.2.4.3.2	DC Application and profile information retrieval response

The elements of the DC Application and profile information retrieval response from the MMTel Enabler Server are the same as Table 8.2.1.3.1-2.

* * * End of Changes * * * *