3GPP TSG-SA WG6 Meeting #63	S6-244210

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Third-Party Call service API

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the Third-Party Call service exposure procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#1, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.4.2	Third-Party Call service API

#### 8.4.2.1	General

The MMTel Enabler Server exposes the Third-Party Call service API to the Vertical service provider in order to support establishing a call between two UEs. The Third-Party Call service API exposed by the MMTel Enabler Server relies on the IMS core network capabilities.

#### 8.4.2.2	Procedure

##### 8.4.2.2.1	Establishing a Third-Party Call with audio, video and Data Channel

Figure 8.4.2.2.1-1 illustrates the operation to establish a Third-Party Call between the Vertical service provider and the MMTel Enabler Server.

Pre-conditions:

1.	The Vertical service provider is authorized to use Third-Party Call service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the Session Management related API and the IMS Event exposure related API of IMS AS.

3.	The MMTel Enabler Server is authorized to use OMA THIRD PARTY CALL API.

Figure 8.4.2.2.1-1: Establishing a Third-Party Call

1.	The Vertical service provider sends a Third-Party Call establishment request to the MMTel Enabler Server. The Third-Party Call establishment request shall include the Participant and Notification address. It may also include Media information, DC media information and Application Profile requested.

2.	Upon receiving the request, the MMTel Enabler Server can utilize the OMA OneAPI to establish a Third-Party Call session with audio or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall or request to establish an IMS Audio/Video Call session via IMSAS. If DC media information is included in step 1, the MMTel Enabler Server utilizes the DC update capability of the IMS core network to add DC media to the existing Third-Party Call Session as specified in 3GPP TS 23.228 [x].

3.	The MMTel Enabler Server sends a Third-Party Call establishment response to the Vertical service provider. The response includes the Result and Session ID, it may include a Failure Reason if the Result is failure.

##### 8.4.2.2.2	Adding/removing Data Channel to an Existing Third-Party Call

Figure 8.4.2.2.2-1 illustrates adding/removing Data Channel to an existing Third-Party Call operation between the Vertical service provider and the MMTel Enabler Server.

Pre-conditions:

1.	The Vertical service provider is authorized to use Third-Party Call service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the Session Management related API and the IMS Event exposure related API of IMS AS.

3.	A Third-Party Call session without Data Channel has established.

Figure 8.4.2.2.2-1: Update a Third-Party Call

1.	The Vertical service provider sends a Third-Party Call update request to the MMTel Enabler Server. The Third-Party Call update request shall include the Session ID and DC media information. It may also include Application Profile requested.

2.	Upon receiving the request, the MMTel Enabler Server utilizes the capability of the IMS core network to add/remove DC media to the existing Third-Party Call Session as specified in 3GPP TS 23.228 [x].

3.	The MMTel Enabler Server sends a Third-Party Call update response to the Vertical service provider. The response includes the Result and Session ID, it may include a Failure Reason if the Result is failure.

#### 8.4.2.3	Information flows

##### 8.4.2.3.1	Third-Party Call establishment request

Table 8.4.2.3.1-1: Third-Party Call establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Participant | M | Array of call participants. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call |
| Application Profile requested | O | The DC application profile expected to be used in this call |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

##### 8.4.2.3.2	Third-Party Call establishment response

Table 8.4.2.3.2-1: Third-Party Call establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the Third-Party Call establishment request was successful |
| > Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Failure response | O | Indicates the Third-Party Call establishment request was failed |
| > Cause | O | Indicates the cause of request failure |

##### 8.4.2.3.3	Third-Party Call update request

Table 8.4.2.3.3-1: Third-Party Call update request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| DC media information | M | Identifier of whether DC media is expected to be used in this call |
| Application Profile requested | O | The DC application profile expected to be used in this call |

##### 8.4.2.3.4	Third-Party Call update response

Table 8.4.2.3.4-1: Third-Party Call update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the Third-Party Call update request was successful |
| > Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Failure response | O | Indicates the Third-Party Call update request was failed |
| > Cause | O | Indicates the cause of request failure |

* * * End of Changes * * * *