3GPP TSG-SA WG6 Meeting #63	S6-244258

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0101 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on low latency signalling transmission connection establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the signalling transmission connection establishment, in order to provide friendly API to the VAL server, the paper propose to split the signalling transmission connection establishment procedure into best effort signalling transmission connection establishment procedure, low latency signanlling transmission connection establishment procedure. In this paper, low latency signanlling transmission connection establishment procedure should be defined in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added low latency signanlling transmission connection establishment procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is difficult for VAL server to invoke SEALDD functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1, 9.2.3.A1, 9.2.3.A2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.2.A	SEALDD enabled low latency signalling transmission connection establishment procedure

Figure 9.2.2.1-1 illustrate the procedure for low latency signalling transmission connection establishment. When VAL server has the low latency requirement on signalling transmission, then the VAL server can invoke low latency signalling transmission connection establishment function of the SEALDD.

This procedure is similar to the SEALDD enabled best effort signalling transmission connection establishment procedure, the differences are as follows:

Figure 9.2.2.1-1: SEALDD enabled low latency signalling transmission connection establishment procedure

1.	When VAL server has low latency requirement on application signalling transmission, then the VAL server decides to use SEALDD service for low latency application signalling transfer and allocates address as SEALDD-S connection information for receiving the application signalling traffic from SEALDD server. The VAL server sends SEALDD enabled low latency signalling transmission connection establishment request to the SEALDD server. The service request includes the VAL server ID, VAL service ID to identify the VAL application signalling traffic, the SEALDD-S connection information of the VAL server, and the latency requirement of the signalling transmission.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD service response.

Step3-8 are same as procedure defined in clause 9.2.2.1, the only difference is that the SEALDD server and SEALDD client establish low latency signalling transmission connection to transmit application signalling based on the latency requirement of the signalling transmission.

9.2.3.A1	SEALDD enabled low latency signalling transmission connection establishment request

Table 9.2.3.A-1 describes the information flow from the VAL server to the SEALDD server for requesting the low latency signalling transmission connection establishment service.

Table 9.2.3.1-1: SEALDD enabled low latency signalling transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL)/port and/or URL of the VAL server to receive the application packets signalling traffic from the SEALDD server |
| Signalling transmission requirement | M | Application signalling transmission requirement provided by VAL server |
| > Requested 5GS Delay |  | M |  | The Requested 5GS Delay provided by the VAL server |  |

9.2.3.A2	SEALDD enabled low latency signalling transmission connection establishment response

Table 9.2.3.A2-1 describes the information flow from the SEALDD server to the VAL server for responding to the low latency signalling transmission connection establishment transmission.

Table 9.2.3.A2-1: SEALDD enabled low latency signalling transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S connection information | O | Address information (e.g., IP address and/or port, URL)/port and/or URL of the SEALDD server to receive the packets signalling traffic from the VAL server for application traffic transfer |
| Cause | O See NOTE | Indicates the reason for the failure. |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * END of Change * * * *