3GPP TSG-SA WG6 Meeting #63	S6-244276

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

Source:	Huawei, HiSilicon

Title:	pCR on Update Usage of CAPIF in different phases of an Application in cluase 6.6

Spec:	3GPP TR 23.946 v0.6.0

Agenda item:	8.2

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to update clause 6.5	Usage of CAPIF in different phases of an Application.

2. Reason for Change

(1) A CR S6-243726/CR0197 was agreed to update the business model and introduce a new annex to illustrate how different API invokers use the CAPIF. This content should be reflected in the 23.946.

(2) The existing figure 6.5.-1, only the application developer is involved in the application software lifecycle and business interaction with CAPIF, however the other stakeholders like application service provider, channel partner are missing.

(3) The service API discover may be used in the application software development preparation phase and the application execution pahse.

(4) The EN “Editor's Note: The API discovery can occur before onboarding, further elaboration of the procedure is FFS.” is moved as the consumer can browse the brief service API information to make the decision whether to select the CAPIF provider and service provider and onboard.

(5) This content is more fit the clause 5 rather than clause 6.

So the figure and the description should be updated accordingly.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.946 0.6.0.

# * * * First Change * * * *

5.4	Usage of CAPIF in different phases of an application business lifecycle

This clause shows an example of CAPIF usage in application service system. In this example, there are following stakeholders/roles: application service provider, application developer, and CAPIF provider and service API provider as defined in 3GPP TS 23.222 [2].

Figure 6.5-1 shows the whole service flow of CAPIF usage by the 3rd party during the application business lifecycle. The application business lifycycle includes 3 phases:

Phase 1.	Application development preparation phase: Before the application developer develops the application, the developer determines the candidate service APIs to be used for the application. The developer selects the CAPIF provider and the service API provider. Then the developer searches the interested service API(s) information, e.g. available service API, service API name, and so on via the utilizes the application management client (act as API invoker), which invokes the CAPIF_Discover_Service_API to obtain that service API information from the CCF. Then, the developer creates an account and gets authorization to access CAPIF core function. This procedure follows onboarding of API invoker as specified in subclause 8.1 in 3GPP TS 23.222 [2]. The developer may find the service API provider at any time later.

Phase 2.	Application development and testing phase: The application developer obtains the detailed service API information including API usage instructions which enables the application developer to program the service API into the application. In addition, the developer may also obtain the security credential of the service API before programming it (option #1). Later, the application developer can test the application which invokes the corresponding CAPIF API and service API (e.g. in a sandbox mode).

Phase 3.	Application deployment and execution phase: After the development, the application is commercial released. The host application (acting as API invoker) obtains the authorization from CAPIF core function to invoker the service API as specified in clause 8.11 of 3GPP TS 23.222 [2]. The host application may obtain the security credential of the service API invocation (Option #2). Upon receiving the access token from CAPIF core function, the application initiates the service API invocation, as specified in subclause 8.16 in TS 23.222 [2]. In RNAA scenario, the API invoker obtains authorization with the CAPIF core function as specified in clause 8.31 of 3GPP TS 23.222 [2], and then requests service API invocation.

NOTE:	There are two options about obtaining the security credential of the service API invocation. Option #1 is fetching it during phase 1 or phase 2 and program it into the host application. In this option, the security can not be dynamically freshed at any time. Option #2 is fetching it during the host application is runing. In this option, the security can be dynamically freshed at any time.

Figure 6.5-1: Usage of CAPIF in different phases of an application business lifecycle

# * * * Next Change * * * *