3GPP TSG-SA WG6 Meeting #63	S6-244433

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244285)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new solution of IOPS network deployment aspect to KI#1

Spec:	3GPP TR 23.700-09 v0.2.0

Agenda item:	8.8

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to add solution to KI#1.

2. Reason for Change

In key issue #1, the following bullet are to be studied.

-	Review and evaluate existing work in the current IOPS technical specification 3GPP TS 23.180 [7] and determine if the existing baseline in TS 23.180 [7] can be used in the generic IOPS solution.

-	Provide a summary of the solutions from 3GPP TS 23.180 [7] that are suitable for a generic IOPS model and can be used with the required changes and which cannot be re-used for generic IOPS and must be re-written.

The existing 23.280 based on the IOPS network architecture defined in 23.401 annex K, which is actually an informative deployment option. However, in order to make the mechanism defined in 23.180 generic, the 5G IOPS network deployment is required.

Introduce a Annex to have similar IOPS network deployment as 4G IOPS that a local 5GC is used due a backhual failure.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-09 v0.2.0.

* * * First Change * * * *

6.a	Solution #a: Converting the functional model and deployment section of TS 23.180

6.aa.1 General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180. It describes which changes are needed to make the application of functional model to deployments of 3GPP TS 23.180 applicable for all supported 3GPP access methods.

NOTE:	in clause 6.aa.2 and 6.aa.3, the original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution.

6.aa.2		Addressing the functional model and deployment section of 3GPP TS 23.180

9	Application of functional model to deployments

9.1	Overview

The IOPS architectural model includes application functions at the IOPS MC system and UEs to support MC services in the IOPS mode of operation during a backhaul failure. The IOPS MC system provides MC services support to the MC service UE via a single PLMN dedicated to the IOPS mode of operation.

The architectural model to support MC services in the IOPS mode of operation consists of a signalling control plane and an application plane. The signalling control plane provides the necessary signalling support for the related IOPS application layer transactions, e.g. the registration and discovery of UEs on the IOPS MC system. The IOPS application plane provides the necessary support for the transport of the IOPS operation related application data as well as the IP packets containing the MC service application data to be distributed via the IOPS MC system.

The MC service application data includes all signalling control data and application data (control and media) required to provide MC services between MC service clients. The IP related transmissions are established over the IOPS MC system via IP unicast and multicast transmissions.

9.2	IOPS architectural model diagram

Figure 9.2-1 shows the IOPS architectural model for the IOPS MC system solution in case of a backhaul failure.

Figure 9.2-1 Architectural model in the IOPS mode of operation

9.2.1	IOPS application services support layer

The IOPS MC system is composed of the following functional entities for the support of MC services in the IOPS mode of operation:

-	an IOPS MC connectivity function as described in clause 7.4.1.3, an IOPS distribution function as described in clause 7.4.1.4 and a signalling application server as described in clause 7.4.2.2.

9.2.2	Local IOPS 3GPP system

A local IOPS 3GPP system can comprise either:

-	a local core network and a single isolated IOPS-capable base station, which may be co-located or have connectivity to the local core network; or

-	a local core network and two or more IOPS-capable base station, which have connectivity to a single local core network.

NOTE:	For 4G IOPS, the implementation and deployment guidelines using a Local EPC approach is described in 3GPP TS 23.401 [2].

The local IOPS 3GPP system provides point-to-point and/or point-to-multipoint transmission services with QoS in the IOPS mode of operation, e.g., as described in 3GPP TS 23.401 [2] for EPS.

9.2.3	UE 1

The UE 1 is an MC service UE supporting the IOPS mode of operation. It supports transmission services and applications to provide MC services over the IOPS MC system. The UE 1 is composed of the following functional entities:

-	for MC services over the IOPS MC system, MC service client(s) as described in clause 7.4.1.5;

-	for IOPS application plane, an IOPS connectivity client as described in clause 7.4.1.2; and

-	for IOPS signalling control, a signalling user agent as described in subclause 7.4.2.1.

9.2.4	UE 2

The UE 2 represents one or more UEs with the same functionality as UE 1.