3GPP TSG-SA WG6 Meeting #63	S6-244434

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244091)

Source:	Nokia

Title:	Pseudo-CR on describing the IOPS discovery request message access generic

Spec:	3GPP TR 23.700-09

Agenda item:	8.8

Document for:	Approval

Contact:	martin.oettl@nokia.com

1. Introduction

In Rel-17 work on Mission critical services support in the Isolated Operation for Public Safety (IOPS) mode of operation has led to solutions described in 3GPP TS 23.180.

This original IOPS technical specification (3GPP TS 23.180) was defined to support 4G networks, but as other mission critical services it was described rather agnostic to the type of network access.

This pCR is related to KI#1 to make the existing IOPS procedures and information flows, described in 3GPP TS 23.180, access generic. The original text has been copied from TS 23.180, i.e. it is not new text, and the change marks highlight suggested required modifications.

2. Reason for Change

Study work is continued.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-09, v0.2.0.

* * * First Change * * * *

### 6.x.x	Solution #x: Clause 10.2.2.3 (IOPS discovery request) of TS 23.180

#### 6.x.x.1	General

This clause is related to KI#1 and addresses clause 10.2.2.3 of 3GPP TS 23.180.

#### 6.x.x.2	Solution

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.2.2.3	IOPS discovery request

Table 10.2.2.3-1 describes the information flow for the IOPS discovery request from the IOPS connectivity client to the IOPS MC connectivity function.

Table 10.2.2.3-1: IOPS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service ID | M | The IOPS MC service ID of the requesting MC user |
| IP connectivity information | M | It includes the requesting MC service UE's IP address (NOTE 1) |
| List of IOPS MC service group ID (NOTE 2) | M | It includes the IOPS group (IOPS MC service group ID and its associated IOPS group IP multicast address) or list of IOPS groups the requesting MC user is configured with |
| Availability of connectivity information | O | It indicates if the connectivity information of the requesting MC user can be made available to other users on the system |
| List of IOPS MC service IDs (NOTE 2 and NOTE 3) | O | The specific list of IOPS MC service IDs that the requesting MC user's connectivity information can be shared with |
| Priority state | M | Indicates whether the requesting MC user is requesting higher priority or not |
| NOTE 1: The MC service UE's IP address is assigned by the local IOPS 3GPP system based on the PDN connection or PDU session established by the MC user for the IOPS IP connectivity functionality. NOTE 2: The list of IOPS MC service IDs or IOPS MC service group IDs shall be of the same MC service as the MC user requesting to be discovered. NOTE 3: If present, the user's connectivity information availability can be only made available to the provided specific list of IOPS MC service IDs. |  |  |

#### 6.x.x.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

#### 6.x.x.4	Solution evaluation

The above propopsal convertes the current scope section of 3GPP TS 23.180 into an access generic IOPS solution.

* * * End Of Change * * * *