3GPP TSG-SA WG6 Meeting #63	S6-244545

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244324, S6-243759)

Source:	Samsung

Title:	New solution for user consent in nested API invocations

Spec:	3GPP TR 23.700-22 v1.0.0

Agenda item:	8.7

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan (basavarajjp@samsung.com)

1. Introduction

This contribution  proposes new solution for Key Issue #1, for user consent management in nested API invocations, upon service API invocation. This pCR is continutation to the pCR (S6-243759), implementing the comments discussed in SA6#62 meeting.

2. Reason for Change

The open issue#1 in key issue #1 is to address how consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function. Obtaining authorization information with resource owner consent upon service API invocation is not handled in CAPIF TS 23.222. And, this is also applicable in the cases of nested API invocations.

This pCR proposes new solution for obtaining authorization with resource owner consent upon service API invocation, which is applicable for both nested API invocations and generically when API invoker invokes service API directly on AEF.

The behaviour of AEF is undefined when the resource owner consent information is not available at Service API invocation. The behaviour needs to decided and specified by stage-2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v1.0.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |
| Sol #15 | X |  |  |  |  |  |  |
| Sol #16 | X |  |  |  |  |  |  |
| Sol #17 |  |  |  |  | X |  |  |
| Sol #X | X |  |  |  |  |  |  |

* * * Next Change * * * *

## 6.X	Solution #X: User consent upon nested service API invocation

### 6.X.1	Solution description

#### 6.X.1.1	General

This solution maps to KI#1 on managing the resource owner consent in nested API invocations scenarios. The solution addresses the scenario of obtaining authorization information with resource owner / user consent upon nested service API invocations. The solution proposed for obtaining authorization information with resource owner / user consent upon service API invocation is applicable both the cases of API invoker obtaining authorization information with resource owner consent upon service API invocation on AEF or an AEF obtaining authorization information with resource owner consent upon a nested API invocation on another AEF.

For the solution related to obtaining authorization information with resource owner consent upon nested service API invocation, the AEF invokes the service API directly to downstream AEF and the downstream AEF responds with a message that authorization information with user consent is required. Based on the message from downstream AEF the requesting AEF obtains the required authorization information with resource owner consent using the procedure specified in TS 23.222 [2]. Also, the procedure in clause 8.16, TS 23.222 [2] can be updated to handle the case for obtaining authorization with resource owner consent upon service API invocation.

#### 6.X.1.2	Obtain authorization with user consent upon service API invocation

This solution is an enhancement to clause 8.16, TS 23.222 [2] (Service API invocation with AEF authorization) or the solution can be new a procedure alternative to clause 8.16, TS 23.222 [2] to handle the case of obtaining authorization with resource owner consent upon service API invocation. The solution mainly proposes a response message from API exposing function, when the resource owner consent is missing in the API invoker’s service API invocation on API exposing function. The API invoker obtains the resource owner consent for the related service API, using the exising RNAA mechanisms specified in TS 23.222.

Figure 6.X.1.2-1: Procedure for obtaining authorization information upon service API invocation

Figure 6.X.1.2-1 illustrates the steps for obtaining authorization information upon service API invocation, as given below:

- The API exposing function 1 invokes with service API directly on API exposing function 2 without the authorization information, as specified in step 1 of  clause 8.16.3, 3GPP TS 23.222 [2].

- API exposing function 2, determines that the required authorization information with resource owner consent is not available to authorize the service API request from API exposing function 1.

- API exposing function 2 responds to API exposing function 1, with response message indicating the failure status of the request message, and indicates the reason of failure that the Authorization information with resource owner consent is not available for resource owner (s) in the service API request. The information included in the response message is shown in Table 6.X.1.2-1.

Table 6.X.1.2-1: Response message

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | Indicates that the request message has failed. |
| Authorization information with resource owner consent required | M | Indicates that the required authorization information with resource owner consent is required. |
| CCF Information | O | Information related to the CCF. For example, CCF URL, Identifier of the CCF, etc. If this information is not present, the API exposing function 1 may contact the CCF that it is configured with. |

- Based on the information received from API exposing function 2, the API exposing function 1 obtains the authorization information with resource owner consent for the required resource owners, using the procedure in clause 8.31 of 3GPP TS 23.222 [2].

In the above solution, API exposing function 1 can be API invoker and the API exposing function 2 can be API exposing function 1. In such case, when the API invoker directly invokes the service API on API exposing function, then the existing RNAA mechanisms specified in TS 23.222 can be used to obtain the resource owner consent.

### 6.X.2	Architecture Impacts

The existing CAPIF architecture as specified in 3GPP TS 23.222 [2], is used for the interactions between API Invoker and CCF defined in this solution. No new functional entities or interfaces are proposed in this solution.

### 6.X.3	Corresponding APIs

The solutions proposed impact the procedures and the related APIs, specified in clause 8.16 of 3GPP TS 23.222 [2]. During normative phase, if new procedures are agreed as way forward than updating the existing APIs and procedures, then correspondingly new APIs may need to be specified.

### 6.X.4	Solution evaluation

This solution addresses the open issue #1 of key issue #1 by enabling for obtaining authorization with resource owner consent upon service API invocation, which is applicable for both nested API invocations and generically when API invoker invokes service API directly on AEF. This solution handles the scenario when an AEF or the API invoker invokes the service API, with out the resource owner consent. The solution proposes new response message from AEF, to the entity invoking service API where authorization (with resource owner consent) is required. This response initiates for obtaining the authorization with user consent based on existing procedures in TS 23.222 [2].

* * * Next Change * * * *

## 9.1	Evaluation of key issue#1

Clause 5.1 captures the key issue on managing resource owner consent. The following open issues are listed:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored?

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

Solution#2 specified in clause 6.3 proposes a solution to address open issue#1 in nested API scenario. This solution requires coordination and feedback on the security aspects for the appropriate user consent at AEF.

Solution#3 specified in clause 6.4 proposes a solution to address open issue#3 on finer granular access control for service API by enhancing the existing access control by introducing the resource level access.

Solution#9 specified in clause 6.10 proposes a solution to address open issue#1 and open issue#2 by enabling more granular information about API invoker being used in the context of RNAA.

Solution#10 specified in clause 6.11 proposes a solution to address open issue#3 by emphasing the need for authorization based on granular service access (e.g., per service API or per service operation or per service resource). The impact on stage 2 aspects of this procedure are covered by Solution#3 and the specific impact for authorization requires further study.

Solution#11 specified in clause 6.12 proposes a solution to address open issue#1 on communication between resource owner and Authorization function (CCF) for user consent revocation.

Solution#16 specified in clause 6.17 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for obtaining user consent.

Solution#12 specified in clause 6.13 addresses open issue#1 to manage consent of resource owner through communication between ROF and CCF (Authorization Function) for processing user consent requests related to one or more API invokers.

Solution #13 specified in clause 6.14 addresses open issue#1 on obtaining resource owner consent via the CAPIF-8 reference point, whereby a request for such consent can be on behalf of one or more API invokers and relate to one or more API exposing functions to reduce the required number of interactions between the CCF (Authorization Function) and resource owner via the ROF.

Solution#13 specified in clause 6.14 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for bulk user consent requests processing for pre-configured API invokers.

Solution#14 specified in clause 6.15 proposes a solution to address open issue#2 on inclusion of purpose for data processing in the mechanim for API invoker requesting/obtaining authorization information.

Solution#15 specified in clause 6.16 proposes a solution to address open issue#1 by describing the role of Authorization function in storage of authorization related information from the resource owner.

Solution #x specified in clause 6.X proposes a solution to address open issue#1 by introducing a new response message to the requesting entity (API invoker or AEF) from the AEF, upon the service API invocation without the resource consent information. The response message triggers the requesting entity to obtain resource owner consent for service API invocation.

Editor's note:	Further evaluation of KI#1 considering the related solutions is FFS.

* * * Next Change * * * *

## 10.2	Conclusions of key issue #1

Solution#3 and Solution#9 can be considered as candidate for normative work for addressing the aspects of granular access and granular API invoker information.

Solution#11, Solution#16, Solution#12 and Solution#13 can be considered as candidate for normative work for addressing the communication between resource owner and Authorization function (CCF) over CAPIF-8. The authentication and authorization procedures for CAPIF-8 are required.

Solution#14 can be considered as candidate for normative work for addressing the inclusion of purpose for data processing in the mechanism for API invoker requesting/obtaining authorization information.

Solution#15 can be considered as candidate for normative work for enhancing the description of the Authorization function role.

Solution#2 requires coordination and feedback on security aspects for the appropriate user consent at AEF to be considered for stage 2 impacts in normative work.

Solution#X can be considered as candidate for normative work for enhancing the procedure to obtain resource owner consent upon service API invocation at AEF.

NOTE 1:	Security aspects are in scope of SA3 and further coordination with SA3 is required.

NOTE 2:	Solution#10 is related to security aspect and is in SA3 scope.

NOTE 3:	A consistent terminology for the terms "user consent", "resource owner consent", "resource owner authorization" is to be defined during normative work.

* * * End of Changes * * * *