3GPP TSG-SA WG6 Meeting #63	S6-244598

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244296)

Source:	Ericsson

Title:	Procedure for Assisting ML Model Training

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added updates for assisting ML model training to TS 23.482.

2. Reason for Change

Based on the requset from the consumer (e.g. VAL server), AIMLE server can assist the ML model training at VAL server and among VAL client(s) for HFL and VFL, etc. This paper proposes to add updates for assisting ML model training to TS 23.482.

3. Conclusions

This paper proposes to add updates for assisting ML model training to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * * First Change * * * *

#### 8.3.3.2	ML model training response

Table 8.3.3.2-1 describes the information flow of ML model training response from the AIMLE server to the VAL server.

Table 8.3.3.2-1: ML model training response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the ML model training request was successful. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for training. |
| > Indication of ML model training complete | O | Indication of the ML model training (e.g. VFL) is completed, and the trained ML model(s) can be used for inference, but the ML model(s) cannot be shared. |
| Failure response | O (NOTE) | Indicates that the ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: At least one of these information elements shall be present |  |  |

* * * Next Change * * * *

## 9.x	ML model training API

### 9.x.1	General

Table 9.x.1-1 illustrates the API for ML model training. This API enables the VAL server to communicate with the AIMLE server for ML model training.

Table 9.x.1-1: Aimles_MLModelTraining API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_MLModelTraining | Request | Request/Response | VAL Server |

### 9.x.2	Aimles_MLModelTraining_Request operation

API operation name: Aimles_MLModelTraining_Request

Description: The consumer requests for ML Model training.

Inputs: See clause 8.3.3.1.

Outputs: See clause 8.3.3.2.

See clause 8.3.2 for details of usage of this operation.

# *** End of Changes ***