3GPP TSG-SA WG6 Meeting #63	S6-244639

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244206)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on DC application and profile configuration

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the DC application and profile configuration procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#7 and KI#2, the MMTel Enabler layer is responsible for the DC application management, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.2.1	DC application and profile configuration

#### 8.2.1.1	General

DC application and profile configuration enables a Controlling Application Server to configure the DC application and profile with the MMTel Enabler Server.

#### 8.2.1.2	Procedure

Figure 8.2.1.2-1 illustrates the DC application and profile configuration procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

3.	Both DC application and profile have been uploaded to Controlling Application Server.

Figure 8.2.1.2-1: DC application and profile configration

1.	The Controlling Application Server sends a DC Application and Profile configuration request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.1.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server allocates an Application ID and creates the DC Application Profile for each DC Application included in the DC Application and Profile configuration request, and maintain the mapping of Application ID and DC Application Profile.

3.	The MMTel Enabler Server configures the DC Application and Profile on DCAR.

4.	The MMTel Enabler Server sends a DC Application and Profile configuration response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.1.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

#### 8.2.1.3	Information flows

##### 8.2.1.3.1	DC Application and profile configuration request

Table 8.2.1.3.1-1 describes information elements for the DC Application and profile configuration request from the Controlling Application Server to the MMTel Enabler server.

Table 8.2.1.3.1-1: Information elements in DC Application and Profile configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application are listed in Table 8.2.1.3.1-2. |

Table 8.2.1.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider |
| Application Name (see NOTE 1) | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version (see NOTE 1) | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: - Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. - Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. - Precall+Incall: the Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario (see NOTE 2) | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: - Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. - Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. - Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: - Service area: the application is allowed to be used in a certain area. - Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint (see NOTE 2) | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection (see NOTE 4) | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| PersonalDataCollectionInfoURL (see NOTE 4) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 1) | O | The Code package of the Data Channel Application. |
| NOTE 1: This IE shall be present when when configuring the DC application. NOTE 2: This IE shall be present when when configuring the DC application profile. NOTE 3: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. The relationship with PersonalDataCollection should be considered, which is out of 3GPP scope. NOTE 4: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

##### 8.2.1.3.2	DC Application and profile configuration response

Table 8.2.1.3.2-1 describes information elements for the DC Application and profile configuration response from the MMTel Enabler Server.

Table 8.2.1.3.2-1: Information elements for DC Application and Profile configuration response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application and profile configuration response list | M | A list of DC Application and profile configuration response. This list contains a response for each DC Application. The detailed information is listed in Table 8.2.1.3.2-2. |

Table 8.2.1.3.2-2: Information elements in DC Application and profile configuration response list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider. |
| Successful response | O | Indicates the success of the configuration of this DC Application and Profile |
| > Application ID | M | Identifier of the Data Channel Application assigned by the MMTel Enabler Server |
| Failure response | O | Indicates the failure of the configuration of this DC Application and Profile |
| > Reason | O | Indicates the reason for failure. |

* * * End of Changes * * * *