3GPP TSG-SA WG6 Meeting #63	S6-244032

Hyderabad , IN, 14th – 18rd Oct 2024

Source:	ZTE Corporation

Title:	Adding Functional Descriptions for ML Model-Related Operations

Spec:	3GPP TS 23.482 v0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Yang Li (li.yang1226@zte.com.cn)

1. Introduction

This paper provides functional descriptions for ML model-related operations, such as ML model retrieval, ML model training, ML model information management.

2. Reason for Change

Based on approved procedures for AIMLE layer in clause 8, some functional description for ML model-related operations are missing in clause 6. This paper provides functional descriptions for AIMLE ML model-related operations, such as ML model retrieval, ML model training, ML model information management.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.2.0.

* * * First Change * * * *

- x	Support for ML model retrieval

This functionality enables the retrieval of ML models from the AIMLE server by AIMLE clients or VAL servers. It allows a requestor to retrieve ML models based on specified filters, such as model identifiers or analytics identifiers. The requestor can either retrieve models directly or subscribe to receive notifications when new models become available. The subscription process allows the requestor to be notified of updates or new models that meet the defined retrieval filters. This functionality supports both one-time retrieval requests and ongoing subscription-based notifications for model availability.

- y	Support for ML model training

This functionality enables the AIMLE server to support ML model training based on requests from a VAL server. The VAL server can request the AIMLE server to train a specific ML model or select an appropriate model for training based on provided requirements. The request includes information such as the type of training (e.g., VFL or HFL), the ML model to be trained, and criteria for selecting participating members. The AIMLE server validates the request and, if authorized, coordinates the training process according to the request information.

- z	Support for ML model management

This functionality enables the AIMLE server to manage ML models through interaction with the ML repository. It includes two key procedures: ML model information storage and ML model information discovery. The storage functionality allows the AIMLE server to store trained ML model information in the repository. The discovery functionality enables the AIMLE server to retrieve information about available ML models by sending discovery requests. The ML repository verifies authorization for both storage and discovery operations and responds with the appropriate success or failure indication.

* * * End of Changes * * * *