3GPP TSG-SA WG6 Meeting #63	S6-244048

Hyderabad, India, 14th – 18rd October 2024	(revision of S6-243xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0589 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for Location user profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Starting with Rel-18 (S6-231429) a common location user profile configuration in TS 23.280, appendix A.8 has been introduced. This and related CRs intend to continue consolidating location related authorizations in this common core specification. No new features are introduced. Authorization entries are moved from TS 23.379 (MCPTT), TS 23.282 (MCData) and TS 23.281 (MCVideo), appendix A.3 to appendix A.8 in this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Moved “Authorization for restricting the dissemination of the location information” from MCPTT, MCVideo and MCData to this specification. - Moved “Authorization to request location of another user in the primary MC system" AND rephrased it to state that this is in addition to the already existing authorization based on a list of users (from whom location can be requested). - Moved “List of partner MC systems for which user is authorised to request location information for another user” from MCPTT, MCVideo and MCData to this specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsisting documentation for authorization regarding location. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.379 CR 0440 |  |  |
|  |  | x |  |  |  |  |  | TS 23.281 CR 0230 |  |  |
|  |  | x |  |  |  |  |  | TS 23.282 CR 0369 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# A.8	Location user profile configuration data

The location user profile configuration data is stored in the location user database. The location management server obtains the location user profile configuration data via the configuration management server (CSC-24).

Table A.8-1 contains the location user profile configuration required to support the use of on-network location service. Data in table A.8-1 can be configured offline using the CSC-11 reference point.

Table A.8-1: location user profile data (on and off network)

| Reference | Parameter description | Location UE | Location Server | Configuration management server | Location user database |
| --- | --- | --- | --- | --- | --- |
| [R-5.11-007] of 3GPP TS 22.280 [3] Clause 10.9.3.3 | Authorization to set a trigger at a LMC |  |  |  |  |
|  | > List of MC service IDs for which a trigger is authorized to be set | Y | Y | Y | Y |
|  | >> MCPTT ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCData ID (see NOTE 1) | Y | Y | Y | Y |
| [R-5.11-008] of 3GPP TS 22.280 [3] | Authorization to cancel a trigger at a LMC |  |  |  |  |
|  | > List of MC service IDs for which a trigger is authorized to be cancelled | Y | Y | Y | Y |
|  | >> MCPTT ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCData ID (see NOTE 1) | Y | Y | Y | Y |
| Clause 10.9.3.5 Clause 10.9.3.7 | Authorization to subscribe to location information for an MC user |  |  |  |  |
|  | > List of MC service IDs for which subscription and subscription cancellation is authorized | Y | Y | Y | Y |
|  | >> MCPTT ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCData ID (see NOTE 1) | Y | Y | Y | Y |
| Clause 10.9.3.6.2 Clause 10.9.3.7 | Authorization to obtain location information for an MC user |  |  |  |  |
|  | > List of MC service IDs for which obtaining location information is authorized | Y | Y | Y | Y |
|  | >> MCPTT ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
|  | >> MCData ID (see NOTE 1) | Y | Y | Y | Y |
| Clause 10.9.3.10.6 Clause 10.9.3.11 | Authorization to modify the location user profile of an MC user |  |  |  |  |
|  | > Modification to the location user profile is permanent/temporary | Y | N | Y | Y |
|  | >> Duration of time for temporary modification | Y | N | Y | Y |
|  | > List of MC service IDs for which modifying the location reporting configuration is authorized | Y | N | Y | Y |
|  | >> MCPTT ID (see NOTE 1) | Y | N | Y | Y |
|  | >> MCVideo ID (see NOTE 1) | Y | N | Y | Y |
|  | >> MCData ID (see NOTE 1) | Y | N | Y | Y |
| [R-6.12-003] of 3GPP TS 22.280 [3] | Authorised to restrict the dissemination of the location information |  |  |  |  |
|  | > List of MC service IDs of the requesting MC service user |  |  |  |  |
|  | >> MCPTT ID (see NOTE 3) | Y | Y | Y | Y |
|  | >> MCVideo ID (see NOTE 3) | Y | Y | Y | Y |
|  | >> MCData ID (see NOTE 3) | Y | Y | Y | Y |
| Clause 10.9 | Authorised to request location information of another user in the primary MC system, who is not included in “List of MC service IDs for obtaining location information is authorized” (see NOTE 4) |  |  |  |  |
|  | > List of MC service IDs of the requesting MC service user |  |  |  |  |
|  | >> MCPTT ID (see NOTE 3) | Y | Y | Y | Y |
|  | >> MCVideo ID (see NOTE 3) | Y | Y | Y | Y |
|  | >> MCData ID (see NOTE 3) | Y | Y | Y | Y |
| Clause 10.9 | List of partner MC systems for which user is authorised to request location information for another user |  |  |  |  |
|  | > List of Identity of partner MC systems |  |  |  |  |
|  | >> Identity of partner MCPTT system (see NOTE 1) | Y | Y | Y | Y |
|  | >> Identity of partner MCVideo system (see NOTE 1) | Y | Y | Y | Y |
|  | >> Identity of partner MCData system (see NOTE 1) | Y | Y | Y | Y |
| NOTE 1: There can be zero or more. NOTE 2: The default authorization applies to all MC users not listed as specifically authorized or not authorized. The intent of this parameter is to avoid having to list all MC users as specifically authorized or not authorized and allowing the administrator to only indicate specific authorization statuses. NOTE 3: There can be zero or one. NOTE 4: Further differentiation on authorisation for requesting location information based on detailed characteristics (e.g. MC organization, MC service ID, functional alias) is left to implementation. |  |  |  |  |  |

* * * End of Change * * * *