3GPP TSG-SA WG6 Meeting #63	S6-244049

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0590 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MC gateway UE routing capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Ericsson, Sepura Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR clarifies the MC gateway UE routing capabilities for the case it hosts the MC clients and for the case it does not host the MC clients. The deployment where the MC gateway UE hosts the MC client does not correctly describe its supported routing capabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rephrase the sentence, that if the MC client is hosted on the N3GPP device IP connectivity is required behind the MC gateway UE, otherwise both IP and non-IP connectivity may be supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Required features which depend on non-IP connectivity can not be supported in case the MC gateway UE hosts the MC clients. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex E (informative): 
MC gateway UE routing capabilities

# E.1	General

When the MC client resides on a non-3GPP device the use of the MC gateway UE requires the support of an IP network behind the MC gateway UE, that a range of IP addresses is reachable over a single MC gateway UE. That enables the forwarding of signalling information and media plane between an MC client on the non-3GPP device and the MC server by the MC gateway UE.

If the MC client resides on the MC gateway UE the MC gateway UE may support IP based and non-IP based connectivity behind it.

# * * * * End of changes * * * *