3GPP TSG-SA6 Meeting #63	S6-244058

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0595 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common Recording/Logging procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Common recording procedures (which are common for MC services) are not specified in the MC specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding common recording procedures in MC specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common procedures will not be added to the specifications. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 12	Recording

## 12.1	General

MC recording service supports recording of metadata, media of MCX Service information (MCPTT, MCVIDEO, MCDATA). Procedures lists common procedure for supporting recording for MC services.

## 12.2	Procedures

Common procedures for MC Service servers supporting recording.

Pre-conditions:

- The MC service UE has successfully logged in into MC Service Server.

- MC Service procedures (MCPTT, MCVIDEO, MCDATA) have been completed.

- Recording is enabled for the MC Service.

### 12.2.1	Common recording procedure

Figure 12.2.1-1:  Common MC service recording procedure

1. MC service client shall initiate MC service request (MCPTT, MCVIDEO, MCDATA) towards MC service server.

2. MC service server shall receive MC service request from MC service client, MC service server shall start processing the request. Post handling MC service request, if recording related configuration or recording related parameters are set, then MC service server shall initiate recording context towards recording server.

3. Recording server shall honour MC service request, shall create new recording context for storing MC service specific recording information.

NOTE: MC service specific information will be audio, video, metadata, file attachments, message text, voice messages, location attachments etc.

4. MC service client shall keep sending updates (with-in the service context, this is based on the end user action participating within the context) to MC Service server, these messages will be update to existing context information.

5. MC service server shall gracefully handle update requests received from MC service client, shall filter out requests (required to be sent towards recording server), post filtering, requests are sent towards recording server.

6. Recording server shall handle (in) context information received from MC service server, shall keep updating recording information stored.

7. When MC service user decides to finish or close the MC service context, MC service client will sendout MC service (context) complete information to MC service server.

8. On receiving MC service (context) complete message (or indication) from MC service client, MC service server shall take appropriate action and shall send out stop recording indication to recording server.

9. On receiving stop recording indication, recording server shall stop recording further details into MC service recording context mantained.

10. Recording server shall synergize date stored in MC service recording context data (it includes metadata, recording content information).

11. Recording server shall transfer MC service recorded data (specific to a context) to mass storage server.

NOTE: Implementation specifics of Step 10,11 are out of scope fort this specification.

### 12.2.2	Fetching recording information

Pre-conditions for fetching recording contents from recording server:

- MC Replay client is provisioned in MC System.

- Recording content present in recording server.

Figure 12.2.2-1:  Fetching MC service recording information by MC-Replay client

1. MC replay client shall login to MC IDMS server, shall perform MC service user specific authentication procedures.

NOTE: The specific security and authentication mechanisms required for MC User is specified in 3GPP TS 33.180 [25].

2. Post MC replay client authentication procedures, MC replay client tries to fetch recording contents from recording server. This action is based on serving recording specific use-cases that MC replay client is configured to support.

3. Recording server shall receive request from MC replay client, shall serve the request.

NOTE: Replay solution to serve the request (arising from MC replay client) at recording server is out of scope for this specification.

4. Recording server shall respond to the request received from MC replay client, recording server is expected to pass recording specific fetched information to MC replay client. Required recording related information shall be received at MC replay client, MC replay client shall use the retrieved information to satisfy use-cases.

# * * * * End of changes * * * *