3GPP TSG-SA WG6 Meeting #63	S6-244062

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0442 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on initial floor request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | October 5, 2024 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The initial floor request is only meaningful between the call originator and the home and group host MCPTT servers. It is not all clear in the existing spec when the call request message is sending beteween MCPTT servers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify in the information table that the “initial floor request” IE is included when a call request is sent from a MCPTT server to the group host MCPTT server. - Add the missing “initial floor request” IE in Table 10.19.2.3-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 may get confused when this IE needs to be included in the call request message sending between MCPTT servers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.1, 10.6.2.2.5, 10.6.2.2.8, 10.6.2.2.20, 10.19.2.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.6.2.2.1	MCPTT emergency group call request

Table 10.6.2.2.1-1 describes the information flow MCPTT emergency group call request from the MCPTT client to the MCPTT server, from the MCPTT server to the group host MCPTT server, and from the MCPTT server to the MCPTT client.

Table 10.6.2.2.1-1: MCPTT emergency group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID on which the call is to be conducted |
| Emergency indicator | M | Indicates that the group call request is an MCPTT emergency call |
| Alert indicator | O | Indicates whether an emergency alert is to be sent |
| Implicit floor request (see NOTE) | O | Indicates that the originating client requests the floor. |
| Requested priority | O | Priority level requested for the call. |
| Location | O | Location of the calling party |
| NOTE: This information element shall be included only when this information flow is from the client to the MCPTT server or from the MCPTT server to the group host MCPTT server and the originating client requests the floor. |  |  |

# * * * * Next change * * * *

##### 10.6.2.2.5	MCPTT imminent peril group call request

Table 10.6.2.2.5-1 describes the information flow MCPTT imminent peril group call request from the MCPTT client to the MCPTT server, from the MCPTT server to the group host MCPTT server, and from the MCPTT server to the MCPTT client.

Table 10.6.2.2.5-1: MCPTT imminent peril group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID on which the call is to be conducted |
| Imminent peril indicator | M | Indicates that the group call request is an imminent peril call |
| Implicit floor request (see NOTE) | O | Indicates that the originating client requests the floor |
| Requested priority | O | Priority level requested for the call. |
| Location | O | Location of the calling party |
| NOTE: This information element shall be included only when this information flow is from the client to the MCPTT server or from the MCPTT server to the group host MCPTT server and the originating client requests the floor. |  |  |

# * * * * Next change * * * *

##### 10.6.2.2.8	Group call request (MCPTT server – group host MCPTT server)

Table 10.6.2.2.8-1 describes the information flow group call request between the MCPTT server and the group host MCPTT server.

Table 10.6.2.2.8-1: Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| Implicit floor request (see NOTE) | O | Indicates that the originating client requests the floor. |
| Requested priority | O | Priority level requested for the call. |
| Location information | O | Location of the calling party |
| Remotely initiated call request indicator | O | Indicates that the MCPTT group call request is a result of receiving of a remotely initiated call request and may be included only for remotely initiated call |
| NOTE: This information element shall be included only when the originating client requests the floor. |  |  |

# * * * * Next change * * * *

##### 10.6.2.2.20	Group join request

Table 10.6.2.2.20-1 describes the information flow group join request from the MCPTT client to the MCPTT server and from the (MCPTT user's primary) MCPTT server to the group host MCPTT server.

Table 10.6.2.2.20-1: Group join request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT group member joining the group communications for the group |
| Functional alias | O | The functional alias of the MCPTT group member joining the group communication for the group |
| MCPTT group ID | M | The MCPTT group ID of the group to which the group communication is requested |
| SDP offer | M | Media parameters of MCPTT client |
| Implicit floor request (see NOTE) | O | Indicates that the originating client requests the floor. |
| Requested priority | O | Application priority level requested for this call |
| NOTE: This information element shall be included only when the originating client requests the floor. |  |  |

# * * * * Next change * * * *

#### 10.19.2.3	Ad hoc group call request (MCPTT server – group host MCPTT server)

Table 10.19.2.3-1 describes the information flow ad hoc group call request between the MCPTT server and the group host MCPTT server.

Table 10.19.2.3-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| MCPTT ID list (see NOTE 1, NOTE 3) | O | MCPTT IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Offered Media parameters of MCPTT server |
| Implicit floor request (see NOTE 4) | O | Indicates that the originating client requests the floor |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| Preconfigured MCPTT group ID | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. NOTE 4: This information element shall be included only when the originating client requests the floor. |  |  |

Editor's Note: It is FFS if the server to server message is needed in a call request or response message.