3GPP TSG-SA WG6 Meeting #63	S6-244078

Hyderabad, India, 14th – 18th October 2024	(revision of S6-243115)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0569 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location reporting information obtained from the PLMN operator (LTE) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 22.280, there is the location related requirement: [R-6.12-002] The MCX Service shall support conveyance of Location information provided by 3GPP location services. Location information obtained from the network comes with several advantages to the MC operators, as discussed in S6-244077, as: - The MC service server is always aware of the location information of MC users irrespective of possible unforeseen events, such as GPS deactivation intentionally or unintentionally, GPS not working properly, UE is off during a time a trigger should have been sent. - The MC service server is independent from the LMC (including its configuration) for its operations and decision making. - Maintain network resources and offload the radio interface especially during network congestion cases. There are two main mechanisms to obtain the location information from the core network: LCS services and via PCRF/PCF notification as part of the PCC rule mechanism. The LCS network can provide location information either on demand based (immediately) or event-triggered based, according to the procedures defined in 3GPP TS 23.271, e.g., mobile terminating location request related procedures. The type of events related to the event triggered based request are the UE availability (after temporarily being unavailable due to e.g., inaction of the user, IMSI detach, radio connectivity loss), periodic reporting, changing of a predefined geographical area or UE motion once moving more than a predefined distance from a certain location. The location information provided by the LCS report includes, among others, the geographical location of the MC service UE, the time stamp of the location information, and related event based on the requested triggers as mentioned earlier. Furthermore, the MC service server can obtain the Access Network Information Reporting parameters upon subscribing to the PCRF notification. The Access Network Information Reporting includes the user location report (e.g., the cell ID, SAI, PLMN ID) and the UE time zone, as described in 3GPP TS 23.203. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - update abbreviations to include GMLC - adding the reference point Le in clause 7.3.1 - remove the note in clause 7.4.2.2.10 - update clause 10.9 to describe the two mechanisms described earlier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirement indicated above is not met, and the mechanisms how the MC service server obtains location information from the PLMN operator is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.3.1, 7.4.2.2.10 and 10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

APN	Access Point Name

BM-SC	Broadcast Multicast Service Centre

CMS	Configuration Management Server

CSC	Common Services Core

CSCF	Call Server Control Function

DPF	Direct Provisioning Function

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EPC	Evolved Packet Core

EPS	Evolved Packet System

FEC	Forward Error Correction

GBR	Guaranteed Bit Rate

GCS AS	Group Communication Service Application Server

GCSE_LTE	Group Communication Service Enabler over LTE

GMS	Group Management Server

GMLC	Gateway Mobile Location Centre

GRUU	Globally Routable User agent URI

HSS	Home Subscriber Server

HTTP	Hyper Text Transfer Protocol

I-CSCF	Interrogating CSCF

IARI	IMS Application Reference Identifier

ICE	Interactive Connectivity Establishment

ICSI	IMS Communication Service Identifier

IdMS	Identity Management Server

IM CN	IP Multimedia Core Network

IMPI	IP Multimedia Private Identity

IMPU	IP Multimedia PUblic identity

IMS	IP Multimedia Subsystem

KMS	Key Management Server

LCS	Location Services

LMC	Location Management Client

LMS	Location Management Server

MBMS	Multimedia Broadcast and Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MC	Mission Critical

MC ID	Mission Critical user identity

MCPTT AS	MCPTT Application Server

MCPTT ID	MCPTT user identity

NAT	Network Address Translation

P-CSCF	Proxy CSCF

PLMN	Public Land Mobile Network

ProSe	Proximity-based Services

PSI	Public Service Identity

QoS	Quality of Service

RAN	Radio Access Network

RF	Radio Frequency

ROHC	RObust Header Compression

S-CSCF	Serving CSCF

SIP	Session Initiated Protocol

SSL	Secure Sockets Layer

TLS	Transport Layer Security

TMGI	Temporary Mobile Group Identity

UDC	User Data Convergence

UDR	User Data Repository

USB	Universal Serial Bus

URI	Uniform Resource Identifier

WLAN	Wireless Local Area Network

For the purposes of the present document, the abbreviations given in 3GPP TS 22.280 [3] apply

MCData

MCPTT

MCVideo

# * * * * Second change * * * *

### 7.3.1	On-network functional model

Each MC service can be represented by an application plane functional model. The functional model across MC services may be similar but is described by the individual functional entities and reference points that belong to that MC service. Within the application plane for an MC service there is a common set of functions and reference points. The common set is shared across services. This common set of functions and reference points is known as the common services core.

Figure 7.3.1-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1-1: Functional model for application plane for an MC system

The common services core functions and reference points shown in figure 7.3.1-1 are shared across each MC service. The description of the functions and reference points specific to an MC service is contained in the corresponding MC service TS.

NOTE 1:	The ACM client and ACM server are not essential entities to enable MC services. The ACM client and ACM server are optional and only required if the administrative configuration management feature is deployed. In administrative configuration management case, only the UE of an authorized MC service user requires such an ACM client.

In the model shown in figure 7.3.1-1, the following apply:

-	A specific MC service server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [18].

-	The functional alias management client is an integrated functional entity of the configuration management client. The functional alias management client is described in subclause 7.4.2.2.12.

-	The functional alias management server is an integrated functional entity of the configuration management server. The functional alias management server is described in subclause 7.4.2.2.13.

Figure 7.3.1-2 shows the functional model for the signalling control plane.

Figure 7.3.1-2: Functional model for signalling control plane

Figure 7.3.1-3 shows the relationships between the reference points of the application plane of an MC service server and the signalling plane.

Figure 7.3.1-3: Relationships between reference points of MC service application plane and signalling control planes

NOTE 2:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 3:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 7.3.1-3.

NOTE 4:	CSC-5, CSC-9, and CSC-15 make use of SIP-1 and SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 7.3.1-3.

# * * * * Third change * * * *

##### 7.4.2.2.10	Location management server (LMS)

The location management server is a functional entity that receives and stores user location information, and provides user location information to the MC service server. The location management server may also acquire location information provided by PLMN operator (e.g. LCS network).

# * * * * Fifth change * * * *

### 10.9.1	General

Location information of MC service user shall be provided by the location management client to the location management server, or by the 3GPP core network (i.e., via the PLMN operator). The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

NOTE 1:	It is up to implementation whether location information reporting from the core network is considered as an optional or primary source of location information to the MC system.

The location information provided by the 3GPP core network can be obtained from the location services feature in EPS (i.e., LCS network) as described in 3GPP TS 23.271 [29], or from the PCRF as part of the PCC mechanism once the MC service server subscribes to related notifications from the PCRF as described in 3GPP TS 23.203 [8].

Regarding the LCS network, the Le reference point between the location management server which acts as an LCS client and LCS network (in specific GMLC) allows to acquire location information according to the defined procedures in 3GPP TS 23.271 [29].

The identity of the target MC service UE(s), which its location is requested from the LCS network, is identified by IMSI. The MC service client shares the IMSI information of its corresponding MC service UE during the service authentication procedure if location information is obtained from the LCS services.

Regarding the location information obtained from the PCRF, upon subscription to related notification from the PCRF, the MC service server receives the Access Network Information Reporting parameters, which includes the user location report and the UE time zone, as described in 3GPP TS 23.203 [8].

The location management client can be configured to store location information while not reporting location information to the location management server. The location management client may report stored location information to the location management server.

NOTE 2:	Configuration and location information transmission may occur while there is a communication link between the location management client and location management server.

In the information flows of clause 10.9.2, and procedures of clause 10.9.3:

-	the term "requesting MC service user" is used to refer to the MC service user which sends a request to subscribe to location information reports, to trigger a location information report or to modify existing location information triggers from another MC service user;

-	the term "requested MC service user" is used to refer to the MC service user whose location information is requested or existing location information triggers are to be modified; and

-	the term "reporting MC service user" is used to refer to the MC service user which provides location related information based on on-demand request or trigger criteria.

# * * * * End of changes * * * *