3GPP TSG-SA WG6 Meeting #63	S6-244100

Hyderabad, India, 19th – 23rd August 2024	(revision of S6-24xxxx)

Source:	Ericsson

Title:	Pseudo-CR on clause 7

Spec:	3GPP TR 23.700-09 v 0.2.0

Agenda item:	8.8

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR address KI#1 with the focus on clause 7.

2. Reason for Change

This pCR looks into having clause 7 of TS 23.180 agnostic to the network.

3. Conclusions

This pCR updates clause 7.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-09 v 0.2.0

* * * First Change * * * *

## 6.x	Solution #X: Clause 7 (Functional model) of TS 23.180

### 6.x.1	General

This solution addresses key issue #1 with the focus on clause 7 to be written in a generic way to be a generic IOPS solution.

### 6.x.2	Solution

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution.

# 7	Functional model

## 7.1	General

The functional model to support MC services in the IOPS mode of operation in case of a backhaul failure consists of a set of functions and reference points.

NOTE:	Terminology such as client and server are not meant to imply specific physical implementation of a functional entity.

## 7.2	Description of the planes

The functional model for the support of MC services in the IOPS mode of operation is defined as a series of planes to allow for the breakdown of the architectural description.

The application plane supports all the necessary functions to establish and provide MC services in the IOPS mode of operation. The application plane uses the services of the signalling control plane to establish the association of the users involved in an MC service. The signalling control plane also offers access to and control of services across MC services.

## 7.3	Functional model description

### 7.3.1	General

The application plane and signalling plane supporting the functional model for the IOPS mode of operation are described in this clause.

### 7.3.2	Functional model for the signalling control plane in the IOPS mode of operation

Figure 7.3.2-1 shows the common functional model for the signalling control plane in the IOPS mode of operation.

Figure 7.3.2-1 Functional model for the signalling control plane in the IOPS mode of operation

NOTE:	The functional model for the signalling plane in the IOPS mode of operation is described as a common functional model across MC services.

### 7.3.3	MCPTT functional model for the application plane in the IOPS mode of operation

Figure 7.3.3-1 shows the MCPTT functional model for the application plane in the IOPS mode of operation.

Figure 7.3.3-1 MCPTT functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.3-1, the following apply:

-	MCPTT-IOPS-2 carries signalling related to the support of multicast or broadcast transmissions, e.g. MBMS bearer announcement, or 5G MBS session announcement, between the IOPS MC connectivity function and the MCPTT client.

-	MCPTT-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

-	MCPTT-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

### 7.3.4	MCData functional model for the application plane in the IOPS mode of operation

Figure 7.3.4-1 shows the MCData functional model for the application plane in the IOPS mode of operation.

Figure 7.3.4-1 MCData functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.4-1, the following apply:

-	MCData-IOPS-2 carries signalling related to the support of multicast transmissions, e.g. MBMS bearer announcement, 5G MBS session announcement, between the IOPS MC connectivity function and the MCData client.

-	MCData-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

-	MCData-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

## 7.4	Functional entities description

### 7.4.1	Application plane

#### 7.4.1.1	General

Entities within the application plane of an IOPS MC system provide application specific functions to support MC services in the IOPS mode of operation.

#### 7.4.1.2	IOPS connectivity client

The IOPS connectivity client functional entity provides support for enabling that a user at the MC service client is registered and discovered by the IOPS MC system in the IOPS mode of operation. This functional entity is located in the MC service UE.

#### 7.4.1.3	IOPS MC connectivity function

The IOPS MC connectivity function entity provides supporting MC services in the IOPS mode of operation. It includes registering and discovering users within the IOPS MC system. The IOPS MC connectivity functional entity represents a specific instantiation of the GCS AS described in 3GPP TS 23.468 [4] to manage unicast and multicast transmissions for group communication related sessions.

The IOPS MC connectivity function entity is supported by the signalling application server functional entity of the signalling control plane.

#### 7.4.1.4	IOPS distribution function

The IOPS distribution function entity provides support for distributing the IP packets containing the MC service application data received from a MC service UE in the IOPS mode of operation.

By means of information provided by the IOPS MC connectivity function (e.g. IP addresses, transport layer ports), it will provide the following functionality:

-	receiving uplink IP transmissions from the MC service UE by means of the MCPTT-IOPS-3 reference point for MCPTT and the MCData-IOPS-3 reference point for MCData;

-	distributing the related IP packets via downlink IP unicast transmissions to MC service UEs by means of the MCPTT-IOPS-3 reference point for MCPTT and the MCData-IOPS-3 reference point for MCData;

-	distributing the related IP packets via downlink IP multicast/broadcast transmissions to MC service UEs by means of the MCPTT-IOPS-4 reference point for MCPTT and the MCData-IOPS-4 reference point for MCData.

#### 7.4.1.5	MC service client

The MC service client functional entity, e.g. a MCPTT client and a MCData client, supporting the IOPS 3GPP network operation acts as the user agent for the corresponding MC service transactions as well as all IOPS related application transactions.

### 7.4.2	Signalling control plane

#### 7.4.2.1	Signalling user agent client

The signalling user agent client functional entity supports at the MC service client all required transactions related to the IOPS discovery. This includes supporting registration, publication, subscription and notification events.

#### 7.4.2.2	Signalling application server

The signalling application server functional entity supports at the IOPS MC system all required transactions related to the IOPS discovery. This includes supporting registration, publication, subscription and notification events.

## 7.5	Reference points

### 7.5.1	Application plane

#### 7.5.1.1	Reference point MC-IOPS-1 (between the IOPS MC connectivity function and the IOPS connectivity client)

The MC-IOPS-1 reference point, which exists between the IOPS MC connectivity function and the IOPS connectivity client, is used for IOPS related application signalling (e.g. registration, publication, subscription and notification events). The MC-IOPS-1 reference point utilizes the MC-IOPS-X1 reference point for transport and routing of the IOPS related signalling.

#### 7.5.1.2	Reference point MCPTT-IOPS-2 (between the IOPS MC connectivity function and the MCPTT client)

The MCPTT-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCPTT client, is used to communicate signalling related to the MBMS bearers or 5G MBS sessions utilized for multicast/broadcast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.3	Reference point MCData-IOPS-2 (between the IOPS MC connectivity function and the MCData client)

The MCData-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCData client, is used to communicate signalling related to the MBMS bearers or 5G MBS sessions utilized for multicast/broadcast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.4	Reference point MCPTT-IOPS-3 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-3 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP unicast transmissions in uplink and downlink. The MCPTT-IOPS-3 reference point uses the SGi reference point or N6 reference point as defined in 3GPP TS 23.002 [7] or 3GPP TS 23.501[23.501], respectively.

#### 7.5.1.5	Reference point MCData-IOPS-3 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-3 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP unicast transmissions in uplink and downlink. The MCData-IOPS-3 reference point uses the SGi reference point or N6 reference point as defined in 3GPP TS 23.002 [7] or 3GPP TS 23.501[23.501], respectively.

#### 7.5.1.6	Reference point MCPTT-IOPS-4 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-4 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP multicast transmissions in downlink. The MCPTT-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4], or Nmb8/ Nmb6 reference point defined in 3GPP TS 23.247 [23.247], respectively.

#### 7.5.1.7	Reference point MCData-IOPS-4 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-4 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP multicast transmissions in downlink. The MCData-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4], or Nmb8/Nmb6 reference points as defined in 3GPP TS 23.247 [23.247], respectively.

#### 7.5.1.8	Reference point MC-IOPS-5 (between the IOPS MC connectivity function and the IOPS 3GPP system)

The MC-IOPS-5 reference point, which exists between the IOPS MC connectivity function and the IOPS 3GPP system, is used to obtain unicast bearers with appropriate QoS from the IOPS 3GPP system. The MC-IOPS-5 reference point utilizes the Rx interface of the EPS, or N5 interface of 5G system according to 3GPP TS 23.203 [8] or 3GPP TS 23.503[23.503], respectively.

#### 7.5.1.9	Reference point MC-IOPS-6 (between the IOPS MC connectivity function and the IOPS 3GPP system )

The MC-IOPS-6 reference point, which exists between the IOPS MC connectivity function and the IOPS 3GPP system, is used to request the allocation and activation of multicast or broadcast transport resources, i.e. MBMS bearers or 5GMBS sessions, during the IOPS mode of operation. The MC-IOPS-6 reference point uses the MB2-C interface as defined in 3GPP TS 23.468 [4], Nmb10, Nmb13, or N33, as defined in 3GPP TS 23.247 [23.247].

### 7.5.2	Signalling control plane

#### 7.5.2.1	Reference point MC-IOPS-X1 (between the signalling application server and the signalling user agent client)

The MC-IOPS-X1 reference point, which exists between the signalling application server and the signalling user agent client, is used to support the IOPS related application signalling (e.g. registration, publication, subscription and notification events).

NOTE:	The IOPS related signalling supporting, e.g., the IOPS discovery, subscription and notification events can be SIP-based. In such a case, the signalling application server acts as a SIP application server (AS) or application function (AF) and the signalling user agent client acts as the SIP user agent for all SIP related transactions.

### 6.x.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services ".

[4]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[5]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.002: "Network Architecture".

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[10]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[11]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[23.501]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[23.247]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[23.503]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: As described in 3GPP TS 23.401 Annex K [2].

IOPS 3GPP system: The collection of 3GPP network entities to enable MC services in IOPS mode of operation.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

### 6.x.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 into an access generic IOPS solution.

* * * End of Changes * * * *