3GPP TSG-SA WG6 Meeting #63	S6-244134

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0089 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new feature to support the Multi-modal flows alignment and monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SEALDD (as XR application enabler) can facilitate Multi-modal flows alignment and monitoring for XR application, as concluded in TR 23.700-23 (KI#1, KI#2, sol#6) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new feature to support the Multi-modal flows alignment and monitoring for XR application. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting new feature in Rel19 XR work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.4.1, New clause: 9.12.2.y, 9.12.3.m, 9.12.3.n, 9.12.3.o, 9.12.3.p, 9.12.3.u, 9.12.3.v, 9.12.3.w, 9.12.3.x, 9.12.3.y, 9.12.3.z, 9.12.3.m, 9.12.3.n, 9.12.4.a, 9.12.4.b, 9.12.4.c, 9.12.4.d, 9.12.4.e, 9.12.4.f |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.12.2.y	Multi-modal flows alignment and monitoring

9.12.2.y.1	General

Synchronization between different multi-modal XR application components is crucial. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that do not arrived yet.

To avoid the downlink traffic flow delay difference introduced from (R)AN and Internet, the SEALDD client can do the multi-modal traffic flow alignment, by steering associated multi-modal traffic flows to be transferred to application client at specific time.

The following clauses specify procedures, information flows and APIs about Multi-modal flows alignment and monitoring.

9.12.2.y.2	Multi-modal flows alignment

Figure 9.12.2.y.2-1 illustrates the procedures of multi-modal flows alignment.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

-	The connection has been established among the UE, SEALDD server and VAL server.

-	The SEALDD server has been provided for multi-modal XR policy including the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2].

Figure 9.12.2.y.2-1: Multi-modal flows alignment procedures

1.	VAL server sends the multi-modal flows alignment request to SEALDD server, with VAL service ID, traffic descriptor, UE ID(s), and optionally, Multi-modal Service ID, flows transmission requirement. Flows transmission requirement includes the delay requirement, number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends the multi-modal flows alignment configuration request to the SEALDD client identified by the UE ID(s), along with the Job ID identifying the alignment job, information received in step1, and the flows alignment assistance information (e.g. timestamp in the RTP header, RTCP).

If the maximum acceptable duration for traffic flow alignment is not provided, then the SEALDD server may determine the maximum acceptable duration for traffic flow alignment based on VAL service ID, flows transmission requirement, transmission quality, and the synchronization threshold. The server may translate the traffic descriptor into SEALDD flow ID.

NOTE 1: The flow alignment assistance information can be obtained by SEALDD server based on SEALDD policy.

NOTE 2: The SEALDD client performs the caching and transmission to align multi-modal flows based on the flow alignments assistance information and maximum acceptable time duration.

Editor's note: Whether and how to make use of existing CN functionality (e.g. PDU set related feature) is FFS.

NOTE 3: The NTP timestamp and RTP timestamp in RTCP sender report (SR) can be used to to identify the associated packet among muti-modal flow, and further be used to perform alignment in SEALDD client.

3.	The SEALDD client sends the multi-modal flows alignment configuration response to the SEALDD server.

4.	The SEALDD server sends the multi-modal flows alignment response to the VAL server with alignment job ID.

5.	When the traffic flows arrive, the SEALDD client initiates the multi-modal flows alignment based on the information received in step 2. The flows need to be aligned are identified by the VAL service ID, traffic descriptor, and flow alignment assistance information.

If the number of associated flows is provided, it is used to determine whether all the associated flows have arrived. If the flow alignment assistance information is provided, the SEALDD client can identify the associated packets (e.g., those with the same RTP timestamp) in the multi-modal flows. After all associated packets in the multi-modal flows have arrived, the SEALDD client sends the associated packets to the application client. If the maximum acceptable time duration is provided, once this maximum acceptable time is reached, the SEALDD client will no longer wait for the associated packets in multi-modal flows, even if they have not arrived yet.

NOTE 4:	Step3 and step5 can be executed in parallel.

9.12.2.y.3	Multi-modal flows alignment monitoring

Figure 9.12.2.y.3-1illustrates the procedures of multi-modal flows alignment monitoring.

Precondition:

-	The multi-modal flows alignment has been requested and triggered.

Figure 9.12.2.y.3-1: Multi-modal flows alignment monitoring procedure

1.	VAL server sends the multi-modal flows alignment monitoring request. Job ID identifying alignment task is provided in the request.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends the multi-modal flows alignment monitoring configuration request to the SEALDD client.

3.	The SEALDD client sends the multi-modal flows alignment monitoring configuration response to the SEALDD server.

4.	The SEALDD server sends the multi-modal flows alignment monitoring response to the VAL server, indicating whether the multi-modal flows alignment monitoring request is successful.

5.	The SEALDD client calculates the end-to-end delay difference based on the arrival time and timestamp (e.g., timestamp in the RTP header) among the flows.

Based on the traffic description, delay difference for traffic flows identified needs to be calculated. The arrival time of first and last detected flows, T1 and T2, are recorded, and the delay difference is calculated based on T1 and T2.

- SEALDD client sends the multi-modal flows alignment monitoring report to the SEALDD server.

- The SEALDD server sends the multi-modal flows alignment monitoring notification to the VAL server.

* * * Next Change * * * *

9.12.3.m	Multi-modal flows alignment request

Table 9.12.3.m-1 describes the information flow from the VAL server to the SEALDD server for requesting Multi-modal flows alignment service.

Table 9.12.3.m-1: Multi-modal flows alignment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | M | Identity of the VAL service |
| UE ID | M | Identifier of specific UE or VAL user |
| Multi-modal Service ID | O | Identifier of Multi-modal Service |
| Traffic descriptor | M | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection. |
| Flows transmission requirement | O | Flows transmission requirement includes the delay requirement, number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow. |

9.12.3.n	Multi-modal flows alignment response

Table 9.12.3.n-1 describes the information flow from the SEALDD server to the VAL server for response Multi-modal flows alignment response.

Table 9.12.3.n-1: Multi-modal flows alignment response

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | The identifier of the Multi-modal flows alignment job |
| Flows flows alignment result | M | Indicates success or failure of the request for the target traffic flows. |

9.12.3.o	Multi-modal flows alignment configuration request

Table 9.12.3.o-1 describes the information flow from the SEALDD server to the SEALDD client for requesting Multi-modal flows alignment configuration service.

Table 9.12.3.o-1: Multi-modal flows alignment configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor |
| Job ID | M | The identifier of the Multi-modal flows alignment job at SEALDD server |
| SEALDD flow ID | O(NOTE 1) | Identity of the SEALDD flows |
| Traffic descriptor | O(NOTE 1) | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection |
| VAL service ID | M | Identity of the VAL service |
| UE ID | O | Identifier of specific UE or VAL user |
| Multi-modal Service ID | O | Identifier of Multi-modal Service |
| Flows alignment assistance information | O | Assistance information to do the multi-modal flows alignment, (e.g. timestamp in the RTP header, RTCP). |
| Flows transmission requirement | O | Flows transmission requirement including the delay requirement, number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow. |
| NOTE 1: At least one of these IEs shall be present in the message. |  |  |

9.12.3.p	Multi-modal flows alignment configuration response

Table 9.12.3.p-1 describes the information flow from the SEALDD server to the VAL server for response Multi-modal flows alignment configuration response.

Table 9.12.3.p-1: Multi-modal flows alignment configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | Identifier of the Multi-modal flows alignment job |
| Flows flows alignment result | M | Indicates success or failure of the request for the target traffic flows. |

9.12.3.u	Multi-modal flows alignment monitoring request

Table 9.12.3.u-1 describes the information flow from the VAL server to the SEALDD server for requesting Multi-modal flows alignment monitoring service.

Table 9.12.3.u-1: Multi-modal flows alignment monitoring request

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | Identity of the Multi-modal flows alignment job |
| Monitoring period | O | Required monitoring period. |

9.12.3.v	Multi-modal flows alignment monitoring response

Table 9.12.3.v-1 describes the information flow from the SEALDD server to the VAL server for response Multi-modal flows alignment monitoring response.

Table 9.12.3.v-1: Multi-modal flows alignment monitoring response

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | The identifier of the Multi-modal flows alignment job |
| Result | M | Indicates success or failure of the Multi-modal flows alignment monitoring. |

9.12.3.w	Multi-modal flows alignment monitoring configuration request

Table 9.12.3.w-1 describes the information flow from the SEALDD server to the SEALDD client for requesting Multi-modal flows alignment monitoring configuration service.

Table 9.12.3.w-1: Multi-modal flows alignment monitoring configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | Identity of the Multi-modal flows alignment job |
| Monitoring period | O | Required monitoring period. |
| Reporting method | O | Required monitoring period(e.g., on time, or daily etc.). |

9.12.3.x	Multi-modal flows alignment monitoring configuration response

Table 9.12.3.x-1 describes the information flow from the SEALDD server to the VAL server for response Multi-modal flows alignment monitoring configuration response.

Table 9.12.3.x-1: Multi-modal flows alignment monitoring configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | The identifier of the Multi-modal flows alignment job |
| Result | M | Indicates success or failure of the Multi-modal flows alignment configuration monitoring. |

9.12.3.y	Multi-modal flows alignment monitoring report

Table 9.12.3.y-1 describes the information flow from the SEALDD client to the SEALDD server for Multi-modal flows alignment monitoring report.

Table 9.12.3.y-1: Multi-modal flows alignment monitoring report

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | Identity of the Multi-modal flows alignment job |
| Monitoring report | M | Result of the monitoring job. |

9.12.3.z	Multi-modal flows alignment monitoring notification

Table 9.12.3.z-1 describes the information flow from the SEALDD server to the VAL server for Multi-modal flows alignment monitoring notification.

Table 9.12.3.z-1: Multi-modal flows alignment monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Job ID | M | The identifier of the Multi-modal flows alignment job |
| Monitoring report | M | Report of the monitoring result. |

* * * Next Change * * * *

9.12.4	APIs

9.12.4.1	General

Table 9.12.4.1-1 illustrates the APIs exposed by SEALDD server for XR transmission.

Table 9.12.4.1-1: List of SEALDD server APIs for XR transmission

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_XRTransmission | Request | Request/Response | VAL server |
| Sdd_XRTransmissionConnnection | Establish | Request/Response | SEALDD client |
| Sdd_MMflows_alignment | Request | Request/Response | VAL server |
| Sdd_MMflows_alignment_Config | Request | Request/Response | SEALDD Server |
| Sdd_MMflows_alignment_Monitoring | Request | Request/Response | VAL server |
| Sdd_MMflows_alignment_MonitoringConfig | Request | Request/Response | SEALDD Server |
| Sdd_MMflows_alignment_MonitoringReport | Notification | Notification | SEALDD Server |
| Sdd_MMflows_alignment_MonitoringNotification | Notification | Notification | VAL server |

9.12.4.2	Sdd_XRTransmission Request operation

API operation name: Sdd_XRTransmission Request

Description: The consumer requests for one time for multi-modal XR transmission service.

Inputs: See clause 9.12.3.1.

Outputs: See clause 9.12.3.2

See clause 9.12.2.1.2 and 9.12.2.2.2 for details of usage of this operation.

9.12.4.3	Sdd_XRTransmissionConnection_Establish operation

API operation name: Sdd_XRTransmissionConnection_Establish

Description: The consumer requests for multi-modal transmission connection establishment.

Inputs: See clause 9.12.3.3.

Outputs: See clause 9.12.3.4.

See clause 9.12.2.1.2 and 9.12.2.2.2 for details of usage of this operation.

9.12.4.a		Sdd_MMflows_alignment

API operation name: Sdd_MMflows_alignment Request

Description: The consumer requests for Multi-modal flows alignment service.

Inputs: See clause 9.12.3.m.

Outputs: See clause 9.12.3.n

See clause 9.12.2.y.2 for details of usage of this operation.

9.12.4.b	Sdd_MMflows_alignment_Config

API operation name: Sdd_MMflows_alignment_Config Request

Description: The consumer requests for Multi-modal flows alignment configuration service.

Inputs: See clause 9.12.3.o.

Outputs: See clause 9.12.3.p

See clause 9.12.2.y.2 for details of usage of this operation.

9.12.4.c	Sdd_MMflows_alignment_Monitoring

API operation name: Sdd_MMflows_alignment_Monitoring Request

Description: The consumer requests for Multi-modal flows alignment monitoring service.

Inputs: See clause 9.12.3.u.

Outputs: See clause 9.12.3.v

See clause 9.12.2.y.3 for details of usage of this operation.

9.12.4.d	Sdd_MMflows_alignment_MonitoringConfig

API operation name: Sdd_MMflows_alignment_MonitoringConfig Request

Description: The consumer requests for Multi-modal flows alignment monitoring configuration service.

Inputs: See clause 9.12.3.w.

Outputs: See clause 9.12.3.x.

See clause 9.12.2.y.3 for details of usage of this operation.

9.12.4.e	Sdd_MMflows_alignment_MonitoringReport

API operation name: Sdd_MMflows_alignment_MonitoringReport

Description: The consumer requests for Multi-modal flows alignment monitoring notification service.

Inputs: See clause 9.12.3.y.

Outputs: None

See clause 9.12.2.y.3 for details of usage of this operation.

9.12.4.f	Sdd_MMflows_alignment_MonitoringNotification

API operation name: Sdd_MMflows_alignment_MonitoringNotification

Description: The consumer requests for Multi-modal flows alignment monitoring notification service.

Inputs: See clause 9.12.3.z.

Outputs: None

See clause 9.12.2.y.3 for details of usage of this operation.

* * * END of Change * * * *