3GPP TSG-SA WG6 Meeting #63	S6-244166

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Samsung

Title:	Pseudo-CR on enhancements to AIMLE client discovery procedure

Spec:	3GPP TS 23.482 (v0.2.0)

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath Ramamoorthy (arun.prasath@samsung.com)

1. Introduction

This pCR enhances the AIMLE client discovery procedure.

2. Reason for Change

As specified in the study, if the required number of AIMLE clients are not returned in the response, the AIMLE clinet may interact with another AIMLE server. Need to add such clarification.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 (v0.2.0).

* * * First Change * * * *

#### 8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs from the ML repository.

Editor’s Note: Whether Member UE filtering criteria is included in the request is FFS.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on AIMLE client profiles and AIMLE client discovery criteria. If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

Editor’s Note: Whether AI/ML Policy IDs is included in the request is FFS.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1. If the required number of AIMLE clients are not included in the response message from the AIMLE server (i.e., the AIMLE clients in the response message are less than the required minimum number of AIMLE clients), then the AIML enablement consumer discovers the remaining required AIMLE clients from other AIML enablement server indicating the required number of AIMLE clients.

### 8.8.3	Information flows

#### 8.8.3.1	AIMLE client discovery request

Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure.

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| Service identifier | O | An identifier for the service associated with the requestor. |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of the required AIMLE clients | O | Indicates the requested number of AIMLE clients to be discovered based on selection policies. |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as training. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type, client availability to support AIML operations at the UE). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| Required service | O | Information about the service. |
| > Required service ID | O | Required service ID that the client supports. |
| > Required corresponding service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |

Editor’s Note: Whether target area for data collection is included in the AIMLE client discovery criteria is FFS.

* * * End of Change * * * *