3GPP TSG-SA WG6 Meeting #63	S6-244181

Hyderabad, India, 14th – 18th Oct 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0332 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support the satellite S&F operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-01 v2.0.0, the KI#7 of Usage of S&F events information for the application enablement for has been concluded as follows: The study concludes with following solution considerations for the normative work: - Solution #AE4 and - Solution #AE8. So it’s needed to specify the functions, procedures and information flows for the KI#7 and this CR focuses on the Sol#AE4 and performs it in TS 23.434 for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedures and information flows for application enablers (NRM, SEALDD) provisioning and exposing the satellite S&F events information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IoT service will be impacted when it is using the satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,14.3.x(new),14.3.2.a(new),14.3.2.b(new),14.3.2.c(new) ,14.3.2.d(new) ,14.3.2.e(new),14.3.2.f(new) ,14.3.2.g(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.3.x	Satellite S&F events information

14.3.x.1	General

This following sub-clauses shows how the application enablers provisioning the satellite S&F configuration to the VAL UE and expose the satellite S&F events information to the VAL server to help minimize the service interruption (e.g., adjust the DL frequency, etc.) when the VAL UE starts to perform the satellite S&F operation.

14.3.x.2	NRM Server exposing the S&F events to the VAL server procedure

Pre-condition:

-	The NRM client, the NRM server and the VAL server all are deployed on the ground.

-	The PGW-U in 3GPP CN is deployed on the satellite.

Figure 14.3.x.2-1: NRM Server exposing the satellite S&F events to the VAL server procedure

1.	The VAL server sends the satellite S&F events subscription request to the NRM server in which the subscribed events may include the UE status in S&F mode, the expected delivery time, the maximum S&F data storage quota per UE, the trigger conditions, etc.

2.	The NRM server authorizes the subscription request from the VAL server.

3.	If the subscription request is authorized, the NRM server sends the satellite S&F events subscription request to the NRM client and/or the 3GPP CN respectively.

4.	The NRM client and/or 3GPP CN acknowledge the NRM server with the satellite S&F events subscription response respectively.

5.	The NRM server responds to the VAL server with the satellite S&F events subscription response.

6.	If the trigger condition in the subscription request is met, the NRM client and/or 3GPP CN may report the requested satellite S&F events as specified in table 14.3.2.c to the NRM server via the satellite S&F events subscription notification.

Editor's note:	The UE S&F events exposed from 3GPP CN depend on the progress of SA2 Rel-19 study and normative work.

7.	The NRM server notifies the VAL server for the received satellite S&F events in step 6. The VAL server may take them into consideration and adjust the application behaviour to minimize the service interruption.

-	The VAL server may pend the DL data transition when the UE is in S&F Mode;

-	The VAL server may pend the DL data transition when the Feeder Link is not available;

-	The VAL server may adjust the DL data volume which are smaller than maximum S&F data storage quota;

-	The VAL server may re-transmit the pending DL data when the Feeder Link is available.

NOTE:	The above behaviours from the VAL server are all out of 3GPP scope.

14.3.x.3	NRM server provisionings the S&F configuration procedure

Figure 14.3.x.3-1 shows how the NRM Server provides the S&F configuration to the NRM Client based on the service request from VAL server and the VAL UE reported capabilities as described in clause 14.3.x.4.

This procedure also applies when the NRM server provides the updated S&F configuration to the NRM client.

Pre-condition:

-	The NRM client and the NRM server are deployed on the ground.

-	The PGW-U in 3GPP CN is deployed on the satellite.

-	The NRM server has obtained the satellite related information (e.g. ephemeris).

-	The NRM Client has reported the UE capabilities related satellite information to the NRM server as described in clause 14.3.x.4.

Figure 14.3.x.3-1: NRM Server provides the S&F configuration to the NRM Client procedure

1.	The NRM server takes advantage of the service request (e.g. monitor the UE traffic volume in a limited time period) initiated by the VAL server, the obtained satellite information (e.g. ephemeris), the reported UE capabilities as described in 14.3.x.4 and its pre-configuration, etc. to determine the on-board S&F parameters for the application layer (e.g., maximum S&F data storage quota per UE/satellite, maximum S&F data retention per UE/satellite, etc.).

NOTE 1:	How the NRM server determining the satellite S&F parameters is internal implementation.

2.	After determined the satellite S&F parameters, the NRM server provides such information to the NRM client via sending a satellite S&F configuration request.

3.	The NRM client/VAL UE stores the received satellite S&F parameters and may take them into account when the VAL UE start to operate the S&F operations based on its satellite S&F configuration.

NOTE 2:	The VAL UE may consider the pre-configuration, the received parameters from the application enabled layer to determine/update its satellite S&F configuration. How the VAL UE determining the S&F parameters is internal implementation.

4.	The NRM client sends the satellite S&F configuration response to the NRM server.

14.3.x.4	VAL UE reports the UE satellite related capabilities procedure

The Figure 14.3.x.4-1 illustrates the procedure of the VAL UE/NRM client reports the UE satellite related capabilities to the NRM server.

This procedure also applies when the VAL UE/NRM client updating its UE satellite related capabilities to the NRM server.

Figure 14.3.x.4-1: VAL UE reports the UE satellite related capabilities procedure

1.	The NRM client reports the UE satellite related capabilities to the NRM server which may contain the UE identity, the UE type (e.g. IoT) and the UE satellite related capabilities, such as the serving satellite ID, the supported satellite types (LEO, MEO and GEO), the maximum data storage quota, etc.

2.	The NRM server checks the authorization for the request from the NRM client.

3.	After successful authorization, the NRM server sends a Report UE satellite capability response to the NRM client and stores the UE identity and the satellite related capabilities information.

* * * Next Change * * * *

14.3.2.a	Satellite S&F events subscription request

Table 14.3.2.a-1 describes the information flow from VAL server to NRM server or from the NRM server to NRM client for satellite S&F events subscription request.

Table 14.3.2.6-1: Satellite S&F status events subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| UE satellite S&F events | O | List the UE satellite S&F events that to be monitored. |
| > UE status in S&F mode | O | Indicates the UE is in S&F mode or not. |
| > The expected delivery time (NOTE1) | O | Indicates the estimated time to send data from the UE to gateway or from gateway to UE. |
| > The maximum S&F data storage quota | O | Indicates the maximum data storage quota for application layer per UE when it is using the satellite S&F operation. |
| Time between consecutive reports | O | Indicates the interval time between consecutive reports |
| Triggering conditions | M | Identifies when the NRM client will trigger/send the notification. |
| NOTE 1: The definition can refer to the clause 8.2 of 3GPP TR 23.700-29 [x]. |  |  |

* * * Next Change * * * *

14.3.2.b	Satellite S&F events subscription response

Table 14.3.2.b-1 describes the information flow from NRM server to the VAL server or from NRM client to NRM server for response of satellite S&F events subscription request.

Table 14.3.2.6-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Subscription status | M | It indicates the subscription result |

* * * Next Change * * * *

14.3.2.c	Satellite S&F events notification

Table 14.3.2.c-1 describes the information flow from NRM server to VAL server or from the NRM client to NRM server to notify the subscribed satellite S&F events.

Table 14.3.2.c-1: Satellite S&F status events notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| UE satellite S&F events | O | List the UE satellite S&F events that to be monitored. |
| > UE status in S&F mode | O | Indicates the UE is in S&F mode or not. |
| > The expected delivery time (NOTE1) | O | Indicates the estimated time to send data from the UE to gateway or from gateway to UE. |
| > The maximum S&F data storage quota | O | Indicates the maximum data storage quota per UE when it is using the satellite S&F operation. |
| Triggering conditions | M | Identifies the conditions that triggered the sending of the notification. |
| Timestamp | O | Timestamp of the notification report. |
| NOTE 1: The definition can refer to the clause 8.2 of 3GPP TR 23.700-29 [x]. |  |  |

* * * Next Change * * * *

14.3.2.d	Satellite S&F configuration request

Table 14.3.2.d-1 describes the information flow from the NRM server to the NRM client for satellite S&F configuration request.

Table 14.3.2.d-1: Satellite S&F configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Satellite S&F configuration information | O | Indicates the S&F configuration per UE when it is using the satellite S&F operation. |
| > Maximum S&F data storage quota | O | Indicates the maximum data storage quota per UE on the application layer when it is using the satellite S&F operation. |
| > Maximum S&F data retention | O | Indicates the maximum data retention per UE on the application layer when it is using the satellite S&F operation. |

* * * Next Change * * * *

14.3.2.e	Satellite S&F configuration response

Table 14.3.2.e-1 describes the information flow from the NRM client to the NRM server for the response of satellite S&F configuration request.

Table 14.3.2.e-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL users or VAL UEs or NRM server. |
| Result | M | It indicates the request result |

* * * Next Change * * * *

14.3.2.f	Report UE satellite capability request

Table 14.3.2.f-1 describes the information flow from the NRM client to the NRM server for reporting the UE satellite related capabilities.

Table 14.3.2.f-1: Report UE satellite capability request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| UE type | O | Indicate the type of VAL UEs (e.g. IoT). |
| Satellite related capability | O | The information of satellite related capability of VAL UE when the VAL UE supports the satellite access. |
| > Satellite ID | O | Indicates the serving satellite ID for the VAL UE. |
| > Satellite type | O | Indicates the supported satellite RAT types (LEO, MEO and GEO) for the VAL UE. |
| > Maximum data storage quota | O | Indicates the maximum data storage quota on the application layer for the VAL UE when it is using the satellite S&F operation. |
| > Maximum data retention | O | Indicates the maximum data retention on the application layer for the VAL UE when it is using the satellite S&F operation. |

* * * Next Change * * * *

14.3.2.g	Report UE satellite capability response

Table 14.3.2.g-1 describes the information flow the NRM server to the NRM client for the response of reporting the UE satellite related capabilities request.

Table 14.3.2.g-1: Report UE satellite capability response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | It indicated the request result. |

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[40]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[41]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[42]		3GPP TS 28.533: "Management and orchestration; Architecture framework".

[43]		3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[44]		3GPP TS 28.532: "Management and orchestration; Generic management services".

[45]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[46]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[48]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[49]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[50]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[51]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

[52]	3GPP TS 23.256 "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[53]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[54]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[55]	3GPP TS 23.545 "Application layer support for Factories of the Future (FF)".

[56]	3GPP TS 23.482 "Functional architecture and information flows for AIML Enablement Service".

[57]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[x]	3GPP TS 23.700-29 " Study on integration of satellite components in the 5G architecture; Phase 3 ".

* * * End of Changes * * * *