3GPP TSG-SA WG6 Meeting #63	S6-244185

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Samsung

Title:	Pseudo-CR on localization service

Spec:	3GPP TS 23.437 (v0.2.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR provides procedure for localization service.

2. Reason for Change

Localization service was studied in clause 7.8.3.5 of TR 23.700-21. Need to provide normative procedure for the same solution.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (v0.2.0).

* * * First Change * * * *

### 9.3.x	Spatial localization services

Figure 9.3.x-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to invoke spatial localization services. The service is provided by SEAL SM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 9.3.x-1: Spatial localization services

1)	The VAL server (or SEAL SM client) sends request to perform localization service in the spatial map to the SEAL SM server. The request includes identity of the map on which the action is to be performed. The request may includes list of users to localize.

2)	The SEAL SM server authorizes the requestor (e.g. VAL server or SEAL SM client). If the requestor is not authorized then the SEAL SM server sends failure response. If the request is authorized: the SEAL SM server, identifies all objects (including the humans and UEs associated with human) along with their three dimensional location in the spatial map.The SEAL SM server also assigns unique identity within maps for the identified objects. If area of interest IE is present then the SEAL SM server identifies the target users within the area of interest only, otherwise, the SEAL SM server identifies the target users for whole map (according to the area associated with spatial map identifier).

3)	The SEAL SM server sends response to the VAL server (or SEAL SM client). The response includes the object localization and identity.

#### 9.3.x.1	Information flows for spatial localization services

##### 9.3.x.1.1	Spatial localization service request

Table 9.3.x.1.1-1 describes information elements for the spatial localization service request from the SM client or VAL server to the SM server.

Table 9.3.x.1.1-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL SM client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target user identities | O | List of identity of the users to localize |

##### 9.3.x.1.2	Spatial localization service response

Table 9.3.x.1.2-1 describes information elements for the spatial localization service response from the SM server to the SM client or VAL server.

Table 9.3.x.1.2-1: Spatial localization service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the action |
| Object details | O | A list of objects or users |
| >> List of stationary objects | O | List of stationary objects for the spatial map (if available). Each object contains information as specified in Table 9.3.1.1-4 |
| >> List of moving objects | O | List of moving objects for the spatial map (if available). Each object contains information as specified in Table 9.3.1.1-4 |