3GPP TSG-SA WG6 Meeting #63	S6-244193

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0334 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for multiple UEs sharing the same location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-09-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the conclusuion of KI#6 of TR 23.700-21 “Solution #10 addressing the KI#6 can be considered in the normative work. The related function and procedure will be specified in 3GPP TS 23.434 [9] to support the solution.” Need to bring solution#10 to the normative work |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated registration table to add assocation ID and location sharing indicator - New procedure and information flow to enable/disable location reuse from the location management client |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature for multiple Ues shairng same location will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.23, 9.3.2.x (NEW), 9.3.2.y (NEW), 9.3.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.2.23	Location service registration request

Table 9.3.2.23-1 describes the information flow from the location management client to the location management server for registration of the location service.

Table 9.3.2.23-1: Location service registration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Location capability | O | The information of location capability of VAL UE for which the location service is registered. |
| > Location access type (NOTE 2) | O (NOTE 1) | Identifies the available location access type of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| >Positioning method | O (NOTE 1) | Identifies the available positioning methods of the VAL UE. E.g. as described in TS 23.273[50] and TS 29.572[51]. |
| List of association IDs | O (NOTE 3) | Identifies list of associations with other UEs. |
| Sharing location | O | Indicates that the associated UEs are allowed to share the location information when they are at the same location. |
| NOTE 1: At least one of these rows shall be present. NOTE 2: The non-3GPP access as defined in TS 23.273[50] and TS 29.572 [51] is out of scope of the present specification. NOTE 3: The association IDs are known to the UE based on offline methods and are our of scope of this specification. The requestor can be part of multiple associations and one association can have two or more UEs. |  |  |

#### 9.3.2.x	Location reuse request

Table 9.3.2.x-1 describes the information flow from the location management client to the location management server for enabling or disabling location reuse.

Table 9.3.2.x-1: Location reuse request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requestor VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Security parameter | M | Security parameters of VAL UE |
| Identity of UE-2 | M | Identity of the VAL UE-2 who will reuse the location of UE-1. |
| Location reuse | M | Indicates whether to enable or disable reuse of location |
| Current location report | O | Location report of the UE-1 (as specified in Table 9.3.2.2-1) |

#### 9.3.2.y	Location reuse response

Table 9.3.2.y-1 describes the information flow from the location management server to the location management client for the location reuse response.

Table 9.3.2.y-1: Location reuse response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

### 9.3.x	Location reuse request

Figure 9.3.x -1 illustrates the procedure of client-triggered enable location reuse request.

Pre-condition:

1.	All UEs in the association are registered to share the location.

2.	UE has configured another UEs in the association to received offline location reports as specified in clause 9.5.3.

Figure 9.3.x-1: Location reuse procedure

1.	The SEAL LMC-1 (in UE-1) receives off-network location report from the SEAL LMC-2 (in UE-2). Based on location report, if UE-2 is within certain range of UE-1, the SEAL LMC-1 determines that the UE-2 is within allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are close enough) and so UE-1's location can be used by SEAL LMS instead of UE-2's location.

Based on location report, if UE-2 is out of certain range of UE-1, the SEAL LMC-1 determines that the UE-2 is outside allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are not close enough) and so UE-1's location can not be used by SEAL LMS instead of UE-2's location.

2.	The SEAL LMC-1 (in UE-1) sends location reuse request message to the SEAL LMS to enable reuse of location of UE-1 for UE-2. If both UE1 and UE2 are clouse enough, the request includes indication to enable reuse of location of UE-1 for UE-2. If both UE1 and UE2 are not clouse enough, the request includes indication to disable reuse of location of UE-1 for UE-2. The request message also includes VAL user’s identity, security parameters, identity of VAL UE-1, identity of UE-2 and latest location report of UE-1.

3.	The SEAL LMS authenticates and authorizes the user. If authorized, the SEAL LMS identifies that if SEAL LMC-1 and SEAL LMC-2 are associated (e.g. same associated ID) or not based on the received registration information of SEAL LMC-1 and SEAL LMC-2, and if reusing of location is allowed of nor based on whether the sharing location indicator is set to true or not.

4.	If both UE-1 and UE-2 are assocaited and reusing of location is allowed (as determined in step-3), then the SEAL LMS stores the location of UE-1. If the request includes indication to enable reuse of location of UE-1, the SEAL LMS enables reuse of location of UE-1 for UE-2. If the request includes indication to disable reuse of location of UE-1, the SEAL LMS disables reuse of location of UE-1 for UE-2. The SEAL LMS sends response back to the SEAL LMC-1.

When location reuse is enabled, if location request for UE-2 is received from VAL server, the SEAL LMS reuses and provides the location of UE-1 (instead of location of UE-2) to the VAL server. When location reuse is disable, if location request for UE-2 is received from VAL server, the SEAL LMS provides the location of UE-2 only (instead of reusing location of UE-1) to the VAL server.