3GPP TSG-SA WG6 Meeting #63	S6-244205

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Evaluation and Conclusion of KI#7

Spec:	3GPP TS 23.392 v1.2.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes evaluation and conclusion of key issue#7.

2. Reason for Change

The LS which asked SA2 whether they have a plan to define an interface for application providers to manage the applications to DCAR has replied.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-92 v1.2.0.

* * * First Change * * * *

### 10.3.X	Key issue #7: Data channel application related capability exposure

The open issues of this KI are as follows:

1.	How does the eMMTel service enabler layer provide service for the DC application provider to support application upload, application upgrade, application deletion, application information retrieval etc, based on the capabilities of the IMS subsystem.

To address the open issues the following solutions has been proposed:

-	Solution #13: data channel application related capability exposure

-	The Controlling Application Server executes the operation of DC Application upload, update and deletion via eMMTel Enabler Server, which implements DC application management in DCAR.

-	The DCAR needs to open northbound interfaces for application management.

-	Solution #4: configuration of DC application profile

-	The Controlling Application Server upload the DC Application Profile to the eMMTel Enabler Server before provision it to UE.

The solution#13 covers the upload of DC application between controlling Application Server and eMMTel Enabler Server. the solution#4 covers the upload of DC application profile between controlling Application Server and eMMTel Enabler Server. The two solutions can be combined in the normative work.

* * * Next Change * * * *

## 11.X	Conclusion of KI#7

For KI#7 on " data channel application related capability exposure " the following conclusions are agreed:

1.	The DCAR needs to open northbound interfaces for DC application management.

2.	The DC application and DC application profile are simultaneously uploaded from the control application server to the emmtel server.

The Solution#13 and Solution#4 with necessary enhancements and additions will be considered as the baseline for technical specification.

* * * End of Changes * * * *