3GPP TSG-SA WG6 Meeting #63	S6-244244

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0338 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add procedure for exposing Sidelink ranging information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-72, the conclusion for KI#5 specifies solution #7 can be considered for normative work to address exposing ranging information to 3rd parties. This contribution targets adding procedure in SEAL-LM architecture to expose ranging information to 3rd parties. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for exposing Sidelink ranging information (9.3.x) Add information flows for request/response in 9.3.2.a and 9.3.2.b |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ranging information is not available to 3rd parties. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.x (new), 9.3.2.a (new), 9.3.2.b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.x	Sidelink ranging information exposure

Figure 9.3.x-1 describes a procedure to expose ranging information to third parties.

Pre-condition:

- Target and reference UEs are configured and provisioned for sidelink ranging/positioning service as described in clause Y.

Editor’s Note:

Figure 9.3.x-1: Sidelink ranging information exposure procedure

- A VAL server sends a sidelink ranging information request to SEAL LMS to initiate sidelink ranging operation for target and reference UEs. The request includes the client, target and reference UEs, the sidelink positioning and ranging information (e.g. range, direction, relative position, relative velocity, etc.) requested, reporting interval and events, QoS requirement, and expiration of ranging request.

2.	The SEAL LMS sends a request to the Client UE to initiate the sidelink positioning and ranging procedure between the target and reference UEs using the information provided in step 1.

3.	The Client UE initiates the sidelink positioning and ranging operation as described in 3GPP TS 23.586 [13]. Either UE-only operation or Network-based operation is performed.

4.	The Client UE sends the sidelink positioning and ranging results to the SEAL LMS, which uses the ranging results to derive a field of view for the target UE. The SEAL LMS uses the range and direction between the target UE and reference UEs to determine UEs that are within a field of view of the target UE.

5.	The SEAL LMS sends a response to the VAL server the requested sidelink positioning and ranging information and exposes location information of UEs within a field of view of the target UE.

* * * Next Change * * * *

#### 9.3.2.a	Sidelink ranging information exposure request

Table 9.3.2.a-1 describes the information flow from the VAL server to the SEAL LMS for the sidelink ranging information request.

Table 9.3.2.a-1: Sidelink ranging information exposure response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the requestor (e.g. VAL user or VAL UE) |
| Requested ranging information | M | Indicates the requested sidelink ranging information (e.g. range, direction, relative position, relative velocity, etc.). |
| Reference UE configurations | M | A list of reference UEs that have been configured for sidelink ranging operation, including client, target, and reference UEs. |
| QoS requirement | O | The QoS requirement for the Sidelink ranging operation. |
| Reporting interval | O | The interval to report sidelink ranging information, e.g. a reporting rate or a reporting schedule. |
| Reporting events | O | Events detected from sidelink ranging information that triggers a notification. Events may be based on absolute and relative distances, velocity, and direction. |
| Expiration | O | The expiration for the sidelink ranging information procedure. |

#### 9.3.2.b	Sidelink ranging information exposure response

Table 9.3.2.b-1 describes the information flow from the SEAL LMS to the VAL server for the sidelink ranging information response.

Table 9.3.2.b-1: Sidelink ranging information exposure response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Identity of the requestor (e.g. VAL user or VAL UE) |
| Ranging information | M | The ranging information associated with locations of UEs within the field of view of the target UE. |

* * * End of Changes * * * *