3GPP TSG-SA WG6 Meeting #63	S6-244255

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0098 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SEALDD server discovery and determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, the SEALDD client and the VAL server may ultilize different SEALDD server, then the SEALDD enabled regular data transmission connection establishment procedure will not work. For example, VAL server discovers and select SEALDD server#1, however, SEALDD client discover and select SEALDD server#2, then the procedure will not work |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added necessary clarification on SEALDD server discovery and determination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEALDD server discovery and determination will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.2.2	SEALDD enabled regular data transmission connection establishment procedure

Figure 9.2.2.2-1 illustrate the procedure for establishing regular SEALDD data transmission connection.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 9.2.2.2-1: SEALDD enabled regular data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for application traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the QoS information for the application traffic, e.g. QoS requirements.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address/port of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S data transmission connection information of the SEALDD server side. The SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S data transmission connection information of the SEALDD server side). The VAL server and SEALDD server can use SEALDD-S data transmission connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer.

The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the address or ports allocated by SEALDD server, and the QoS information for application traffic. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4].

NOTE 1:	The SEALDD-S data transmission connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server.

3.	Data transmission session information is provisioned to the VAL client by the VAL server via application signalling. The data transmission session information may contain the SEALDD address information which is discovered and selected by VAL server.

NOTE 2:	The application signalling may be transmitted via direct application layer connection or via the SEALDD layer.

4.	The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. If the data transmission session information contains the SEALDD address information, then the VAL client sends the selected SEALDD address information to SEALDD client in service request message. The response indicates that whether the SEALDD service request is successful or not.

5.	If the data transmission session information does not contain the SEALDD address information, then the VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address.

6.	The SEALDD client allocates SEALDD-UU flow ID(s) mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD-UU flow ID(s), the SEALDD traffic descriptor of the SEALDD client side (the address/port of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID. The request message also contains the selected VAL server endpoint information and UE ID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject.

NOTE 3: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic.

NOTE 4:	The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD-UU flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD.

7.	The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address/port allocated in step 2, transport layer protocol) mapping to the application traffic.

8.	If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the SEALDD traffic according to the SEALDD-S information negotiated in step 1-2.

9.	The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment.

After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address/port and SEALDD server's address/port for the SEALDD traffic transmission).

After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application traffic and SEALDD-UU flow ID. The SEALDD server gets the mapping information between the SEALDD-UU flow ID and the SEALDD-S connection. Upon receiving application traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD-UU flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly.

The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3.

* * * END of Change * * * *