3GPP TSG-SA WG6 Meeting #63	S6-244289

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0340 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN in 9.3.11.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | R19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | (1) There is one EN in cluase 9.3.11.2: Editor's note: It is FFS whether to use SL/Ranging mechanism to further figure out the target UE(s) are present or absent in the proximity range of the reference UE when the reference UE (or target UE(s)) lost the connectivity with CN but the target UE(s) (or the reference UE) can still be reached via ProSe. The target VAL UE location status still can be further determined using SL/Ranging mechanism (if they are within the Prose communication range) in the case that the reference UE lost connectity but the target UE(s) still have the connectivity, or vice versa. So if only the target UE(s) or the reference UE lost connectity, the LM server should try to request the UE still have connectivity to obtain the location information of the connectivity lost UE. Several possibile options are considered: (1) requst the connected UE/CN to obtain the absolute location information, and the LM server calculate the distance and the target VAL UE location status; (2) request the connected UE/CN to report the relative location, and the LM server determines the target VAL UE location status; (3) request the connected UE to report the target VAL UE location status, in this option, the LM server provide the proximity range to the connected UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the clause 9.3.11.2 to support the above enhancement to determine the target VAL UE location status. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The target VAL UE location status is not accurate in the case that only the reference UE lost connectity but the target UE(s) still have the connectivity, or vice versa. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.5, 9.3.2.6, 9.3.2.7, 9.3.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* First Changes *****************************/

9.3.2.5	Location information subscription request

Table 9.3.2.5-1 describes the information flow from the VAL server or location management client to the location management server, or from the location management server to the location management client for location information subscription request or for updating an existing location information subscription.

Table 9.3.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE (see NOTE 3) |
| Identities list | M | List of VAL users or VAL UEs whose location information is requested. |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Supplementary location information indication | O | Indicates that supplementary location information is required. |
| Location QoS | O (NOTE 2). | Definition of the location Quality of Service for which the location information is requested (see NOTE 1). |
| Requested location type | O (NOTE 5) | Indicate absolution or relative location information is requested |
| Requested positioning mechanism | O (NOTE 5) | Indicates the positioning mechanism for which the location information is requested, e.g., Ranging/SL. |
| Velocity information | O | Identifies the velocity of the target UE for which the location information is requested. |
| Indication for the location data statistic | O | It indicates whether the statistic/calculation of target UE location data is needed per e.g. time/location. |
| Requested time information | O (NOTE 4) | It indicates the time information when the target UE location data is calculated per time. |
| Requested location information | O (NOTE 4) | It indicates the location information when the target UE location data is calculated per location. |
| Velocity indication | O | It indicates whether velocity of the requested VAL users/UEs is needed. |
| NOTE 1: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 2: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 3: This information element shall not be updated via location information subscription update procedure in clause 9.3.7a. NOTE 4: This information element only applies when the indication for the statistic of target UE location data is included. |  |  |

9.3.2.6	Location information subscription response

Table 9.3.2.6-1 describes the information flow from the location management server to the VAL server or location management client, or from the location management server to the location management client for location information subscription response or location information subscription update response.

Table 9.3.2.6-1: Location information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Subscription status | M | It indicates the subscription result |

9.3.2.7	Location information notification

Table 9.3.2.7-1 describes the information flow from the location management server to the VAL server or the location management client, or from the location management client to the location management server.

Table 9.3.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of the VAL users or VAL UEs whose location information needs to be notified |
| Identity (see NOTE 2) | O | Identity of the VAL user or VAL UE subscribed to location of another VAL user or VAL UE (see NOTE 1) |
| Subscription ID (see NOTE 2) | O | Subscription identity related to VAL server subscription with Location management server for location information notification. |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information | M | Location information including e.g., absolution location or relative location |
| Velocity information | O | Indicates the velocity of the target UE for which the location information is requested. |
| Statistic of location data | O | Indicates the statistic result of target UE location data per temporal/spatial granularity as requested. |
| Velocity information | O | Velocity information |
| Timestamp | O | Timestamp of the location report |
| NOTE 1: This is only used for location management server sends location information notification to the VAL user or VAL UE who has subscribed the location. NOTE 2: Either Identity or Subscription ID shall be included. |  |  |

# /******************************* Second Changes *****************************/

9.3.11.2	Monitoring Location Deviation procedure

Figure 9.3.11.2-1 describes the procedure for monitoring the VAL UE's location in a given area of interest.

Pre-condition:

-	The LM server authorized to consume the 3GPP core network service (Monitoring events provided by NEF as specified in 3GPP TS 23.502 [11]).

Figure 9.3.11.2-1: Monitoring VAL UE's location at a given location

1.	The VAL server sends Monitor Location Subscription Request to LM server including target VAL UE Identifier(s), predetermined or dynamic area of interest information, notification interval and notification URI where the VAL server intends to receive the notifications from LM server regarding target VAL UE's presence in a given area, interested notification information, dynamic geofencing conditions.

-	"Area of interest" is the location information, which the VAL server wishes to monitor the target VAL UE's location adherence. This parameter can include an area of interest information and other relevant parameters. In the case of dynamic area of interest, it includes a reference VAL UE Identifier and its proximity range.

-	"Notify_Interval" represents the periodic interval in which the LM server needs to notify target VAL UE's location information to the VAL server. When the target VAL UE moves away from the "Area of interest", then the LM server ignores the "Notify_Interval" and sends the location notification to the VAL server immediately.

-	"dynamic geofencing conditions" includes a temporal condition(s) (e.g. time of day and day of week), connectivity condition(s) (e.g. operator information, roaming status, access type) and/or a spatial condition(s) (e.g. location area(s)) for the reference VAL UE. For instance, the monitoring is valid when a reference UE is served by operator A without using satellite in location area X from 7:00am to 9:00am on working days.

2.	If dynamic area of interest information and dynamic geofencing conditions are received, the LM server requests 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11]) to monitor the reference UE location change, access type change and roaming status change, in order to determine spatial dynamicity in area of interest related to the target UE(s). If spatial condition is received, the LM server also requests 3GPP CN (i.e. NEF service for area of interest monitoring as specified in 3GPP TS 23.502 [11]) to know whether the reference UE is in the VAL server's predetermined area (e.g. PLMN x). The LM server will process the monitoring considering the received dynamic geofencing conditions for the reference UE.

3.	If dynamic area of interest information is received, the LM server requests 3GPP CN (i.e. NEF service for loss of connectivity as specified in 3GPP TS 23.502 [11]) for both target UE(s) and reference UE.

4a.	LM server processes the Area of interest information in the request, and then subscribes to target UE location monitoring as specified in 3GPP TS 23.502 [11] with appropriate parameters mapping. Based on the subscription, the LM server receives the target VAL UE location information periodically from the 3GPP core network. The LM server also invokes NEF service for Area of Interest monitoring as specified in clause 5.3.3 of 3GPP TS 23.256 [52] to know whether the target VAL UE is in the VAL server's predetermined or dynamic area of interest.

4b.	LM server shall use the Location information procedures as specified in clause 9.3.7 and clause 9.3.10, to periodically obtain the target VAL UE location information. Based on the geographic information from the VAL server, the LM server may determine to additionally include the positioning methods in SEAL LMS procedures to obtain location information.

4c.	LMS may periodically obtain the target VAL UE location information from the 3rd party location management server based on the geographic information from the VAL server.

NOTE 1:	How the LMS obtains the target UE location from the 3rd party location management server is out of scope of this specification.

NOTE 2:	Step 4a to 4c are performed for each target VAL UE if multiple target VAL UE Identifiers are provided by the VAL server.

5.	LM server, after successful subscription according to steps 2~4, sends Monitor Location Subscription response, indicating that the LM server accepts VAL server's request and will monitor the location of the target VAL UE(s) to verify if the target VAL UE(s) is in the area of interest.

Either step 6 or step 7 is performed if it is notified about loss of connectivity for the reference UE and target UE(s).

6.	The LM server requests the target UE to obtain the reference UE’s location information (i.e., the absolute location or the relative location).

NOTE 3:	In this figure, it assumes the reference UE lost connectivity and the target UE still has the connectivity. If the target UE lost connectivity and the reference UE has the connectivity, LM server requests the reference UE to obtain the target UE's location information.

6a.	The LM server determines the reference UE lost connectivity and the target UE still has the connectivity, the LM server send location information subscription request to the target UE. The reference UE identity and the Ranging/SL method, location type (i.e., the absolute location or the relative location) are included. The LM client in the target UE returns a location information subscription response.

6b.	The target UE initiates the initiates the Ranging/SL based MO-LR procedure in clause 6.20 of 3GPP TS 23.273 [50] to obtain the reference UE location. The target UE obtains the reference UE’s location information.

6c.	The target UE sends the location information notification with the reference UE’s location information.

7.	Alternatively, the LM server requests the core network to the Ranging/SL based MT-LR procedure in clause 6.20 of 3GPP TS 23.273 [50] and obtains the reference UE location.

8.	LM server processes the location information received from SEAL Location Information procedures, the 3GPP core network and/or the 3rd party location management server, and validates the information. If the location information is matching, then the LM server shall check if the VAL UE's current location is within the predetermined or dynamic area of interest received in step 1.

NOTE 4:	The location information received from the 3GPP core network and from LM client does not necessarily be exactly the same, the LM server can aggregate the location information if there is no big deviation.

Further, LM server verifies its determined target VAL UE location status (i.e. presence or absence with regard to the predetermined area of interest) with the retrieved 3GPP core network result from the NEF monitoring service for Area of Interest. The LM server will continue with step 9 as applicable.

9a.	If the location information received from Location management client, the 3GPP core network and/or the 3rd party location management server do not match or the NEF reported Area of Interest monitoring result does not match with the determined target VAL UE location status (present/absence) in LM server, then the LM server shall consider the target VAL UE as outside from its specified area of interest and shall notify ("Notify Mismatch Location" message) the VAL server of the same, including target VAL UE ID and the location information from LM server, the 3GPP core network and/or the 3rd party location management server in the notification message.

9b.	If the target VAL UE's current location is from Location management client, the 3GPP core network and/or the 3rd party location management server matches, and either not in the predetermined area of interest received from VAL server in Monitor Location Subscription Request message or not within the proximity range of the reference UE, and such out of area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server considers the target VAL UE as outside from its specified area of interest and shall notify the VAL server that the target VAL UE's current location is outside of area of interest and target VAL UE ID in "Notify Absence" message.

If the target UE or the reference UE lost connectivity, the LM server determines the target VAL UE is outside the predetermined area of interest or dynamic area of interest, and shall notify the VAL server "Notify Absence" message.

- the target UE lost connectivity, and target VAL UE's current absolute location from reference UE per step 6c and not in the predetermined area of interest or not within the proximity range of the reference UE;

- the target UE lost connectivity, and target VAL UE's current relative location from reference UE per step 6c and not in the predetermined area of interest or not within the proximity range of the reference UE;

- the reference UE lost connectivity, and reference VAL UE's absolute relative location from target UE per step 6c. The target UE's location is not within the proximity range of the reference UE;

- the reference UE lost connectivity, and reference VAL UE's current relative location from target UE per step 6c. The target UE's location is not within the proximity range of the reference UE;

9c.	When the target VAL UE's current location is either in the predetermined area of interest or in the proximity range of the reference UE, and such in area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server shall notify ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the target VAL UE is within the area of interest, along with target VAL UE's current location information.

If the target UE or the reference UE lost connectivity, the LM server determines the target VAL UE is within the predetermined area of interest or dynamic area of interest, and shall notify the VAL server "Notify Presence" message.

- the target UE lost connectivity, and target VAL UE's current absolute location from reference UE per step 6c and in the predetermined area of interest or within the proximity range of the reference UE;

- the target UE lost connectivity, and target VAL UE's current relative location from reference UE per step 6c and in the predetermined area of interest or the proximity range of the reference UE;

- the reference UE lost connectivity, and reference VAL UE's absolute location from target UE per step 6c. The target UE's location is within the proximity range of the reference UE;

- the reference UE lost connectivity, and reference VAL UE's current relative location from target UE per step 6c. The target UE's location is within the proximity range of the reference UE;

9d.	If the LM server is notified by the 3GPP CN about loss of connectivity for the reference UE and target UE(s), or fails to obtains the target UE's location or the reference UE' location in step 6c (e.g., due to the target UE and reference UE are beyond the Ranging/SL range), then the LM server notifies ("Notify Unknown" message with detailed reason) the VAL server indicating that the LM server does not know whether the target UE is "Presence" or “Absence", therefore the dynamic geofencing monitoring for the impacted target UE(s) is suspended.

NOTE 5:	Upon "Notify Unknown", the VAL server can remind end user to check status for the reference/target UE, which is out of 3GPP scope.

Editor's note:	Detailed handling for “loss of connectity” in LM server is FFS. For instance, whether to address the case when reference UE and target UE(s) both lose connectivity (e.g., using UE-only operation for SL/Ranging).

NOTE 6:	Notifications in step 9 can be subject to explicit notification indication from the VAL server, for instance, the VAL server can be interested to know only "Notify Absence" and "Notify Unknown".