3GPP TSG-SA WG6 Meeting #63	S6-244392

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new solution to KI on NRM XR API

Spec:	3GPP TR 23.700-35 v0.0.0

Agenda item:	8.9

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to a new solution to KI#a on NRM XR specific service .

2. Reason for Change

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-35 v0.0.0.

* * * First Change * * * *

X.a	Solution #x: XR session with guaranteed bitrate service

X.a.1	Solution description

This solution proposes to address the KI #a: NRM XR specific service.

It is proposed to introduce a new NRM API dedicated to XR service, i.e., XR session with guaranteed bitrate service. This service provides the capability for the VAL server to request XR related network resource and handlings for XR service, e.g., guaranteed bitrate,  PDU Set handling, power saving, QoS monitoring, Round-Trip (RT) latency.

X.a.1.1	Procedures

X.a.1.1.1	XR session with guaranteed bitrate create

The figure X.a.1.1.1-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with guaranteed bandwidth service.

Pre-condition:

- The VAL server and VAL client already connected. The application traffic delivery is handled by VAL server and VAL client themselves.

Figure X.a.1.1.1-1: XR session with guaranteed bitrate service create procedure

1.	The VAL server determines to request the network resource for XR service. It sends the XR network session with guaranteed bitrate create request to the NRM server. In the request, the following parameters are included:

-	Bitrate parameters: QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of 3GPP TS 23.503 [23503]. This is used to request the desired bitrate.

The following parameters are optionally included per XR application requirements:

-	Alternative bitrate parameters set: QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of 3GPP TS 23.503 [23503]. This is the alternative bitrate parameters the XR service can accept if the primary bitrate parameters cannot be fulfilled by the network.

-	Limited network resource optimization parameters: refers to the "PDUSetHandling" feature related parameters in 3GPP TS 29.122 [29122]. This is used by the network to guarantee certain level user experience when in limited network resource scenario.

-	Multi-model parameters:  refers to the " MultiMedia" feature related parameters in 3GPP TS 29.122 [29122].

-	Power saving parameters: refers to the "PowerSaving" feature related parameters in 3GPP TS 29.122 [29122].

-	QoS monitoring parameters: refers to the "EnQoSMon" feature related parameters in 3GPP TS 29.122 [29122].

-	Round-time latency parameters: refers to the "RTLatency" feature related parameters in 3GPP TS 29.122 [29122]. This is used by the VAL server to request the round-trip latency for the XR service.

-	L4S parameter: refers to the "L4S" feature related parameters in 3GPP TS 29.122 [29122]. This is used by the VAL server to request the network to add the ECN marking for L4S support.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_create request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_create request (if the NRM server is owned by MNO). The corresponding features are selected by the NRM server based on the parameters received in step 1.

3.	The NEF or PCF process the NRM server’s request.

4.	The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_create response or Npcf+policyAuthorization_create response.

5.	The NRM server sends the XR network session with guaranteed bitrate create response with result to the VAL server.

X.a.1.1.2	XR session with guaranteed bitrate update

Compared with the XR session with guaranteed bitrate update procedure and information flows, the only differences are the message name in the update procedure is XR network session with guaranteed bitrate update request and response. Accordingly, the NRM server invokes the Nnef_AFsessionWithQoS_update or Npcf+policyAuthorization_update.

X.a.1.1.3	XR session with guaranteed bitrate delete

The figure X.a.1.1.1-1 illustrates the high-level procedure of VAL server invoking the NRM XR session with guaranteed bandwidth service to delete the requested network session.

Pre-condition:

Figure X.a.1.1.3-1: XR session with guaranteed bitrate service delete procedure

1.	The VAL server determines to release the network resource for XR service. It sends the XR network session with guaranteed bitrate release request to the NRM server.

2.	Based on the received request, the NRM further invokes the Nnef_AFsessionWithQoS_delete request (if the NRM server is owned by 3rd party and outside the MNO domain)  or Npcf+policyAuthorization_delete request (if the NRM server is owned by MNO).

3.	The NEF or PCF process the NRM server’s request.

4.	The NEF or PCF returns the corresponding Nnef_AFsessionWithQoS_delete response or Npcf+policyAuthorization_delete response.

5.	The NRM server sends the XR network session with guaranteed bitrate delete response with result to the VAL server.

X.a.1.2	Information flows

X.a.1.2.1	XR network session with guaranteed bitrate create/update request

Table X.a.1.2.1-1 describes the information flow from VAL server to NRM server for unicast resources create/update.

Table X.a.1.2.1-1: XR network session with guaranteed bitrate create/update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE. |
| Bitrate parameters | M | Indicates the bitrates requirements of the XR service. It is either QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of 3GPP TS 23.503 [23503]. |
| Alternative bitrate parameters set | O | Indicates the alternative bitrates requirements of the XR service. It is either QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of 3GPP TS 23.503 [23503]. |
| Limited network resource optimization parameters | O | Indicates requesting the network to perform optimization when in limited network resource case. It refers to the "PDUSetHandling" feature related parameters in 3GPP TS 29.122 [29122]. It |
| Multi-model parameters | O | Indicate the multi-model requirements. Refer to the "MultiMedia" feature related parameters in 3GPP TS 29.122 [29122]. |
| Power saving parameters | O | Indicate the power saving requirements. Refer to the "PowerSaving" feature related parameters in 3GPP TS 29.122 [29122]. |
| Round-time latency parameters | O | Indicates the round-trip latency for the XR service requirements. Refer to "RTLatency" feature related parameters in 3GPP TS 29.122 [29122] |
| L4S parameter | O | Indicate requesting the network to add the ECN marking for L4S support. Refer to the "L4S" feature related parameters in 3GPP TS 29.122 [29122] |
| QoS monitoring parameters | O | Indicates the interested network session events from the network. Refer to the "EnQoSMon" feature related parameters in 3GPP TS 29.122 [29122]. |

X.a.1.2.2	XR network session with guaranteed bitrate create/update response

Table X.a.1.2.2-1 describes the information flow from NRM server to VAL server for XR network session with guaranteed bitrate create/update response.

Table X.a.1.2.2-1: XR network session with guaranteed bitrate create response

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE whose network session is to be released. |

X.a.1.2.3	XR network session with guaranteed bitrate delete request

Table X.a.1.2.3-1 describes the information flow from NRM server to VAL server for XR network session with guaranteed bitrate delete request.

Table X.a.1.2.3-1: XR network session with guaranteed bitrate delete request

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource request operation. |
| Cause | O | Indicates the failure cause |

X.a.1.2.4	XR network session with guaranteed bitrate delete response

Table X.a.1.2.4-1 describes the information flow from NRM server to VAL server for XR network session with guaranteed bitrate delete response.

Table X.a.1.2.4-1: XR network session with guaranteed bitrate delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource request operation. |
| Cause | O | Indicates the failure cause |

X.a.2	Impacts on architecture, services, entities

This solution has no architecture impacts.

This solution creates new NRM XR service at NRM.

7.2.2.4	Solution evaluation

This solution provides an NRM XR service complied with the friendly API design principle.