3GPP TSG-SA WG6 Meeting #63	S6-244297

Hyderabad, India, 14th – 18th October 2024	(revision of S6-243200)

Source:	Ericsson

Title:	Support of VFL at Enablement Layer

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedure for VFL at enablement layer to TS 23.482.

2. Reason for Change

Solution#18 is recommended for normative work on procedure to support Vertical FL (VFL) at enablement layer. See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for VFL at enablement layer to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for VFL at enablement layer to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FL		Federated Learning

VFL	Vertical FL

* * * Next Change * * * *

## 8.x	VFL at Enablement Layer

This clause describes procedure for supporting VFL among application layer multiple UEs.

### 8.x.1	General

The following clauses specify procedures, information flows and APIs to support VFL among Application Layer multiple UEs.

### 8.x.2	Procedures

#### 8.x.2.1	Procedure for supporting VFL in Enablement Layer

Pre-conditions:

-	VFL member (e.g., AIMLE Client which is deployed to a UE) has dataset or can access to data sources or is able to collect data from data producers for the AI/ML operations.

-	VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server.

Figure 8.x.2.1-1: Procedure for supporting VFL in Enablement Layer

  - An AIMLE server receives an ML model training assist request or ML model training request from a VAL server as described in clause 8.3.2. If ML model training assist request is received and the training type is VFL in the requeat message, the AIMLE server generates assistance information for the VFL at the VAL server and VAL clients. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between domains), the procedure continues to step 2.

2.	The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 or be obtained through running the step 2 in the AIMLE client discovery procedure in clause 8.8.2.1). The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for ML model training capability evaluation as described in clause 8.x.2.2.

3.	The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 2 and parameters received in step 1. The criteria used by the AIMLE Server include:

-	Available data and minimum number of data samples for the same sample among the VFL members.

-	Feature alignment of the sample/datasets with data labels among the VFL members.

-	Available time of the VFL members for support the VFL training operations.

-	Capability and minimum number of the VFL members for the VFL training operations.

- AIML model information for the VFL members and for the AIMLE Server.

4.	The AIMLE Server coordinates the selected VFL members to perform VFL training operations for the VFL process using the parameters received in step 1. During VFL training, the VFL members send the AIMLE Server intermediate results, and the AIMLE Server sends the VFL members gradients to update the model parameters maintained by each VFL member.

Editor’s Note:	The procedure for AIMLE Server coordinates the selected VFL members to perform VFL training is FFS.

Editor’s Note:	Whether and how the AIMLE server monitoring the dynamic changes of the VFL member(s) during the VFL process is FFS.

- The AIMLE server sends an ML model training assist response or an ML model training response to the VAL server as described in clause 8.3.2.

#### 8.x.2.2	Procedure for training capability evaluation

Pre-conditions:

-	AIMLE server determines to use VFL training.

-	AIMLE Server knows the information of the VFL members (e.g., AIMLE Client which is deployed to an UE).

Figure 8.x.2.2-1: Procedure for training capability evaluation

1.	The AIMLE Server sends ML model training capability evaluation request to the VFL members (AIMLE Clients which are deployed on UEs). The request message includes information as described in Table 8.x.3.1-1.

2.	The VFL members evaluate their capability and availability to join the VFL training process. The VFL members (AIMLE Clients which are deployed on UEs) run the test task contained in the request in step 1 and determine if it can join the VFL process. As part of the test task, data alignment between the datasets of the different domains are determined. The VAL server may also provide data labels for the data alignment.

NOTE:	The procedures for data collection from UE need to take user consent into account.

3.	The VFL members (AIMLE Clients which are deployed on UEs) send ML model training capability evaluation response to the AIMLE Server. The response message contains the information as described in Table 8.x.3.2-1.

### 8.x.3	Information flows

#### 8.x.3.1	ML model training capability evaluation request

Table 8.x.3.1-1 shows the request sent by AIMLE server to VFL members (AIMLE Clients which are deployed on UEs) for ML model training capability evaluation.

Table 8.x.3.1-1: ML model training capability evaluation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Available time | O | Requirement on available time for supporting VFL operations. |
| Test task | O | The task for test ML model training capability. |
| AI/ML model and model parameter(s) | O | Information about the AI/ML model and model parameters for use in VFL training. |
| Requirement on dataset | O | Requirements on dataset for VFL training. |
| >Feature ID(s) | O | Identifier(s) of the required feature of the dataset from the UE. |
| >Domain(s) | O | Domain(s) of the datasets at the UE. |

NOTE:	The detail content of the IEs in Table 8.x.3.1-1, e.g. Test task and AI/ML model and model parameter(s), are up to implementation and not defined in 3GPP.

#### 8.x.3.2	ML model training capability evaluation response

Table 8.x.3.2-1 shows the response sent by VFL members (AIMLE Clients which are deployed on UEs) to AIMLE server for the ML model training capability evaluation.

Table 8.x.3.2-1: ML model training capability evaluation response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the evaluaton: success, fail. - Success means join the VFL training process. - Fail means not join the VFL training process. |
| Test result | C | The test result of the ML model training capapbility evaluation. The "test result" shall be provided when the "status" is "success". |
| Fail reason | C | The reason of the ML model training capapbility evaluation fail. The "fail reason" shall be provided when the "status" is "fail". |

* * * Next Change * * * *

### 9.Y.A	AIMLE ML Model Training Capability Evaluation API

Editor’s Note:	This clause provides the corresponding AIMLE ML Model training capability evaluation API.

# *** End of Changes ***