3GPP TSG-SA6 Meeting #63	S6-244306

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-23 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XR application interaction context information request procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Telecomunicazioni SpA |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | KI#4 describes the need for application enablement layer capabilities usage to support the XR services. Today's almost all XR devices are equipped with eye-tracking and gaze-tracking capabilities. Some are with advanced technologies like haptics. The video-based XR applications depend on where the user is looking and the associated interaction of the user's gaze with the XR application. For example in foveated streaming, the part of the video is rendered in the high-quality where the user's gaze is and peripheral parts in low-quality in the user's Field-of-view(FOV). The information related to the user's gaze is necessary to optimize the XR media transmission in the network, and thereby improving the user Quality of Service(QoS) and Quality of Experience(QoE). The SEALDD server or XRApp server handles the XR media packet processing and transmission to the XR clients or SEALDD clients. Therefore the SEALDD or XR App server needs to consider mechanisms for handling the packet transmission due to the user's interaction(like gaze) leading to the delivery of the high-resolution packet with low latency and high bandwidth. The proposed solution provides configuration and collection of the relevant information from the XR clients required for the consumer(e.g. SEALDD or XRApp server ) to efficiently manage resources and optimize performance. The solution enables the SEALDD server to collect interaction context information. The SEALDD server later makes use of this information to optimize the data delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper introduces support for the foveated delivery or viewport-based delivery of XR applications in the SEALDD server. It is only a collection information from the SEALDD client or XR client and this informaton will be for later use by different SEALDD procedures for optimization of XR data delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEALDD server would not be able to optimize the delivery of XR traffic and unable to provide the adaptive streaming of XR traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x(new), 7.x.1.1(new), 7.x.2.1(new), 7.x.2.2(new), 7.x.2.3(new),7.x.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.x	Solution #x: XR application interaction context information request procedure

#### 7.x.1.1	XR Application enablement client application interaction context information procedure

Figure 7.x.2.1-1: XR Application enablement client application interaction context information procedure

1.	The consumer(e.g. SEALDD server) sends the XR Application enablement client application interaction context information subscribe request to the SEALDD client or XR App client as per Table 7.x.2.1-1.

2.	The SEALDD client or XR App client provides the XR Application enablement client application interaction context information subscribe response to the consumer as per Table 7.x.2.2-1.

3.	The SEALDD client or XR App client sends the XR Application enablement client application interaction context information notification to the consumer as per Table 7.x.2.3-1. The SEALDD server can retrieve the viewport information, gaze/pose information as per the 3GPP TS 26.118 clause 9.2.4 and Table 9.3.2-1, Table  9.3.2-2 defined in clause 9.3.2. The SEALDD server uses the interaction context information for later use to optimize the XR media transmission QoS using quality transmission or network slicing or rate control.

Editor’s NOTE: The usage of interaction context in other SEALDD procedures is FFS.

* * * Next Change * * * *

#### 7.x.2.1	XR Application enablement client application interaction context information subscribe request

Table 7.x.2.1-1 describes the information flow from the consumer(e.g. SEALDD server) to the SEALDD client for XR Application enablement client application interaction context information subscribe request.

Table 7.x.2.1-1: XR Application enablement client application interaction context information subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor |
| VAL server ID | O | Identity of the VAL server |
| VAL service ID | M | Identity of the VAL service |
| XR client ID | O | Indentity of the XR client |
| Interaction context information configuration | M | Indicates XR Application interaction context information configuration. It includes interaction type(e.g. gaze, eye-tracking, touch, gesture), and interaction update reporting interval. |
| Reporting notification endpoint | O | Indicates the notification endpoint information for reporting. |
| Immediate notification reporting flag | O | Indicates the flag to send immediate report |

#### 7.x.2.2	XR Application enablement client application interaction context information subscribe response

Table 7.x.2.2-1 describes the information flow from the SEALDD client to the consumer(e.g. SEALDD server)  for XR Application enablement client application interaction context information subscribe response.

Table 7.x.2.2-1: XR Application enablement client application interaction context information subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the subscription request |
| > cause | O | Indicates the reason for failure (e.g. hardware not available) |

#### 7.x.2.3	XR Application enablement client application interaction context information notification

Table 7.x.2.3-1 describes the information flow from the SEALDD client to the consumer (e.g. SEALDD server)  for XR Application enablement client application interaction context information notification.

Table 7.x.2.3-1: XR Application enablement client application interaction context information notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor |
| SEALDD Flow ID | O | Identity of the SEALDD-UU flow ID |
| Multi-Flow ID | O | Identity of the multi-flow ID |
| Interaction context information | M | Indicates XR Application interaction context information. It includes viewpoint information, Gaze direction, timestamp(like pose timestamp, frame timing) |

### 7.x.3	Solution evaluation

This solution enables the XR App or SEALDD server to optimize the XR media transmission for interaction like gaze tracking-based high-resolution delivery in foveated or adaptive XR streaming. This solution enables SEALDD server to fetch the interaction context. This interaction context can be used by other SEALDD procedures like rate control, or quality transmission or slicing-based XR delivery to optimize the XR data delivery.

* * * End of Changes * * * *