3GPP TSG-SA6 Meeting #63	S6-244422

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024 					(revision of S6-244058)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0595 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common Recording/Logging procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Common recording procedures (which are common for MC services) are not specified in the MC specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding common recording procedures in MC specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common procedures will not be added to the specifications. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.18 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 10.18		Recording and Replay procedures

### 10.18.1	Recording procedures

MC recording procedures involves capturing the contents of MC services that can be call/session based MC services (e.g. MCPTT/MCVideo Call, MCData session etc.) or non-call/session based MC Services/events (e.g. MCPTT Emergency Alert etc.).

#### 10.18.1.1	Recording procedure for call/session based MC services

Following are pre-conditions before MC service recording procedures are invoked.

Pre-conditions:

- The MC service UE has successfully logged in into MC service server.

- The target group/user involved in the recording session has been configured for recording and target recording server information has been configured in MC service server.

Figure 10.18.1.1-1:  Session based MC service recording procedure

1.	MC service client shall initiate a session based MC service request towards MC service server. MC service server shall receive MC service request from MC service client, MC service server shall start processing the request.

2.	If target MC user or MC service group is configured for recording, MC service server sends start recording indication to the recording server with the session context information.

3.	Recording server shall honour MC service request, shall create new recording context for storing MC service specific recording information.

4.	MC service client continues it’s active MC service session with the MC service server and it may cause additional MC service content/meta data information be generated.

5.	Any updates on MC service specific contents or metadata are reported to the recording server.

6. 	Recording server shall handle (in) context information received from MC service server, shall keep updating recording information.

7.	When the MC Service session with the MC service client is ended, there will be session closing information generated by the MC service server.

8. 	MC service server sends stop recording indication to MC recording server with the session close context information.

9.	On receiving stop recording indication, recording server shall stop recording context and reconcile meta data and recorded contents.

10.	Session based recorded contents are made ready for replay service.

#### 10.18.1.2	Recording procedure for non-session based MC services

Non session based recording procedures is for supporting recording of non session based MC services.

Following are pre-conditions before MC service recording procedures are invoked.

Pre-conditions:

- The MC service UE has successfully logged in into MC service server.

- The target group/user involved in the non-session based recording has been configured for recording and target recording server information has been configured in MC service server.

Figure 10.18.1.2-1:  Non-Session based MC service recording procedure

1.	MC service client shall initiate non session based MC service communication towards MC service server (ex. MCPTT Emergency alerts). MC service server shall receive MC service request and process the request.

2.	If target MC user or MC service group is configured for recording, MC service server shall send the MC service communication information to recording server.

3.	On receipt of the MC service communication information, recording server shall save MC service communication information along with metadata.

4.	Recording server shall make non session based recorded contents ready for replay service.

### 10.18.2	Replay procedures

#### 10.18.2.1	Replay procedure for recording content.

Editor's note:	contents of this section is FFS.

# * * * * End of changes * * * *