3GPP TSG-SA WG6 Meeting #63	S6-244519

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244114)

Source:	China Mobile

Title:	Pseudo-CR on Updated solution evaluation and conclusion of KI#3

Spec:	3GPP TS 23.700-92 v1.2.0

Agenda item:	8.2

Document for:	Approval

Contact:	Yue Liu, liuyueyjy@chinamobile.com

1. Introduction

Update the solution evaluation and conclusion of KI#3 based on LS replied from SA2 (S6-244009)

2. Reason for Change

Update the solution evaluation and conclusion of KI#2 based on LS replied from SA2 (S6-244009)

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-92 v1.2.0.

* * * First Change * * * *

10.3.3	Key issue #3: support of eMMTel call between application with IMS capability and application without IMS capability

The open issues of this KI are as follows:

- The architecture, necessary functionalities and procedures of the eMMTel Enabler to support the eMMTel call between application with IMS capability and application without IMS capability.

- The necessary service APIs on eMMTel Enabler to support the application without IMS capability to initiate, terminate and update, etc., the eMMTel call with an application with IMS capability.

To address the open issues the following solutions has been proposed:

-	Solution #2: Application calling service between application and DCMTSI client

-	Applicable to calls from the application layer application (such as non-IMS applications) to UE. In the procedure, the eMMTel Enabler Server acts as http proxy and transfers the http traffic to the IMS Core, the IMS Core transforms http streams to data channel streams.

-	Solution #7: getting call information

-	In the procedure, application server can get the information of the call from the eMMTel Enabler Server.

Solution #2 and Solution #7 covers open issues#1 and #2 of KI#3 and can be considered in the normative work.

In addition, the IMS DC capability exposure is planned to be supported in NG_RTC_Ph2 WID in Rel-19, i.e.

-	establishing, updating, terminating of bootstrap data channels;

-	establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels.

New APIs for IMS session management will not be defined in NG_RTC_Ph2 WID. In NG_RTC_Ph2 WID, it is assumed that the existing APIs, e.g. OMA APIs provides such functionality.

* * * Next Change * * * *

11.3	Conclusion of KI#3

For KI#3 on "support of eMMTel call between application with IMS capability and application without IMS capability" the following conclusions are agreed:

1.	Based on the gap analysis of OMA OneAPI framework in clause 4.2, IMS data channel is not supported by OMA API, therefore, a set of new APIs for MMTel enabler will be considered to be specified for technical specification based on Solution#2, and such new APIs needs to be aligned with existing OMA APIs and also provide data channel capability (based on 3GPP TS 23.228 [3]) in Rel-19.

a.	The eMMTel Enabler Server acts as a DC AS based on SA2’s IMS exposure capability architecture, according to the session creation API invoked by the eMMTel Enabler Server, the IMS network initiates an IMS session.

b.	The eMMTel Enabler hides the complexity of underlying the IMS core capabilities, the eMMTel Enabler Server handles all the necessary API requests/responses related to the session creation on behalf of the application without further interaction with the application.

c.	The eMMTel Enabler can also update or terminate the IMS session.

Editor’s note:	How to use the existing IMS session management APIs provided by OMA in MMTel service, without impacting SA2 architecture and considering potential security requirement is FFS and needs to be solved in the normative phase.

2.	The Solution#7 will be used for get the information of call and considered as the basis for technical specification with necessary enhancements and additions.

* * * Next Change * * * *

<Proposed change in revision marks>