3GPP TSG-SA WG6 Meeting #63	S6-244543

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244323)

Source:	Samsung, ZTE

Title:	Pseudo-CR on group user consent

Spec:	3GPP TR 23.700-22 V1.0.0

Agenda item:	8.7

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This pCR is to address the open issue related to group user consent.

## 5.1 Key issue #1: Managing resource owner consent ### 5.1.1 Description With the introduction of support for Subscriber-aware Northbound API access in 3GPP Rel-18 a number of requirements were added at stage 1, one of which was motivated by the desire for a UE to be able to control whether or not to provide information considered private to a 3rd party entity. Specifically, the requirement is that the 5G system shall be able to allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party. At stage 2 the CAPIF-8 reference point was introduced to CAPIF where the aspect of consent was highlighted through the statement that the resource owner communicates with the authorization function in the CAPIF core function to manage resource owner consent, with such communication being expected to be performed over CAPIF-8. However, the mechanism for managing such consent was not specified with the functionalities over CAPIF-8 being is FFS and out of scope of the Rel-18 of the specification. Authentication of resource owner is required over CAPIF-8. Further, user consent may also be required during group communication to access the information related to the members of the group. For example, the VAL server hosting dynamic platooning group requires access to the location of the members e.g. for performing operations of a group, joining the group. ### 5.1.2 Open issues The CAPIF does not address management of resource owner consent in the context of supporting RNAA. The open issues are: 1. How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function 2. Whether (and how) “purpose of data processing” will be captured and where will it be stored. 3. How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer. 4. How to enable CAPIF resource owner’s consent in the context of group. 5. How to authenticate the resource owner via CAPIF-8 NOTE: Aspects pertaining to the definition of resource owner consent/authorization over CAPIF-8 are in the scope of SA3, noting that the R18 security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [3].

2. Reason for Change

To address the open issue enabling CAPIF resource owner’s consent in the context of group, the proposal is to consider the group context during provisioning of user consent on CCF. The group context is composed of Subject which includes server and other group members invoking the user consent and Scope includes e.g. location, service APIs etc.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 V1.0.0.

* * * First Change * * * *

## 6.X	Solution #X: Enabling resource owner’s consent in the context of group

### 6.X.1	Solution description

#### 6.X.1.1	General

This paper is for KI #1 to enable resource owner’s consent in the context of group.

To address the open issue enabling CAPIF resource owner’s consent in the context of group, the solution proposes to consider the group context information during provisioning of user consent on CCF. The group context information is composed of Subject which includes group service server and other group members information invoking the user consent and Scope which includes e.g. location, service APIs etc.

#### 6.X.1.2	Impact to existing CAPIF procedures

The CAPIF procedure in 3GPP TS 23.222 can be enhanced (highlighted in bold italics) as follows.

### 8.31.3 Procedure Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner. Pre-conditions: 1. The resource owner function can communicate with the API invoker. 2. The service API access requires obtaining authorization from resource owner. Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner - The API invoker requests to obtain resource owner authorization information to invoke the service API exposed by the API exposing function. The authorization function provides the authorization by interacting with the resource owner via the resource owner function. For enabling resource owner’s consent for the group, the Resource owner function (ROF) and CAPI Core Function (CCF) communicate for managing the provisioning of resource owner consent. The group context information (composed of Subject which includes group service server, group ID and other group members information and Scope which includes e.g. location, service APIs etc.) is sent to CCF. And CCF will include this group context information to obtain the user consent. The Resource owner function (ROF) indicates the user consent success or failure, for e.g. success if the user has given consent for the requested information. NOTE 1: The detailed procedure to obtain the resource owner's authorization information is specified in TS 33.122 [12]. NOTE 2: Coordination with SA3 is required on the step 1. NOTE 3: CCF obtaining group information from SEAL GMS is as per clause 10.3.4 of 3GPP TS 23.434. 2. The API invoker sends service API invocation request to the API exposing function with the resource owner authorization information received in step 1. 3. The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the authorization information.

### 6.X.2	Architecture Impacts

This solution is based on architecture of CAPIF as described in 3GPP TS 23.222 [2].

### 6.X.3	Corresponding APIs

No new APIs are introduced.

### 6.X.4	Solution evaluation

This solution addresses the open issue of group user consent in KI #1, the impacted CAPIF procedure is the API invoker obtaining authorization from the resource owner based on the group context information.

* * * Next Change * * * *

## 9.1	Evaluation of key issue#1

Clause 5.1 captures the key issue on managing resource owner consent. The following open issues are listed:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored?

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

4.	How to enable CAPIF resource owner’s consent in the context of group.

Solution#2 specified in clause 6.3 proposes a solution to address open issue#1 in nested API scenario. This solution requires coordination and feedback on the security aspects for the appropriate user consent at AEF.

Solution#3 specified in clause 6.4 proposes a solution to address open issue#3 on finer granular access control for service API by enhancing the existing access control by introducing the resource level access.

Solution#9 specified in clause 6.10 proposes a solution to address open issue#1 and open issue#2 by enabling more granular information about API invoker being used in the context of RNAA.

Solution#10 specified in clause 6.11 proposes a solution to address open issue#3 by emphasing the need for authorization based on granular service access (e.g., per service API or per service operation or per service resource). The impact on stage 2 aspects of this procedure are covered by Solution#3 and the specific impact for authorization requires further study.

Solution#11 specified in clause 6.12 proposes a solution to address open issue#1 on communication between resource owner and Authorization function (CCF) for user consent revocation.

Solution#16 specified in clause 6.17 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for obtaining user consent.

Solution#12 specified in clause 6.13 addresses open issue#1 to manage consent of resource owner through communication between ROF and CCF (Authorization Function) for processing user consent requests related to one or more API invokers.

Solution #13 specified in clause 6.14 addresses open issue#1 on obtaining resource owner consent via the CAPIF-8 reference point, whereby a request for such consent can be on behalf of one or more API invokers and relate to one or more API exposing functions to reduce the required number of interactions between the CCF (Authorization Function) and resource owner via the ROF.

Solution#13 specified in clause 6.14 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for bulk user consent requests processing for pre-configured API invokers.

Solution#14 specified in clause 6.15 proposes a solution to address open issue#2 on inclusion of purpose for data processing in the mechanim for API invoker requesting/obtaining authorization information.

Solution#15 specified in clause 6.16 proposes a solution to address open issue#1 by describing the role of Authorization function in storage of authorization related information from the resource owner.

Solution#X specified in clause 6.X proposes a solution to address open issue#1 by describing the usage of group context information for enabling CAPIF resource owner’s consent in the group.

Editor's note:	Further evaluation of KI#1 considering the related solutions is FFS.

* * * Next Change * * * *

## 10.2	Conclusions of key issue #1

Solution#3 and Solution#9 can be considered as candidate for normative work for addressing the aspects of granular access and granular API invoker information.

Solution#11, Solution#16, Solution#12 and Solution#13 can be considered as candidate for normative work for addressing the communication between resource owner and Authorization function (CCF) over CAPIF-8. The authentication and authorization procedures for CAPIF-8 are required.

Solution#14 can be considered as candidate for normative work for addressing the inclusion of purpose for data processing in the mechanism for API invoker requesting/obtaining authorization information.

Solution#15 can be considered as candidate for normative work for enhancing the description of the Authorization function role.

Solution#2 requires coordination and feedback on security aspects for the appropriate user consent at AEF to be considered for stage 2 impacts in normative work.

Solution#X can be considered as candidate for normative work for enabling resource owner’s consent in the context of group.

NOTE 1:	Security aspects are in scope of SA3 and further coordination with SA3 is required.

NOTE 2:	Solution#10 is related to security aspect and is in SA3 scope.

NOTE 3:	A consistent terminology for the terms "user consent", "resource owner consent", "resource owner authorization" is to be defined during normative work.