3GPP TSG-SA WG6 Meeting #63	S6-244547

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244322)

Source:	Samsung

Title:	Pseudo-CR on CAPIF discovery enhancements

Spec:	3GPP TR 23.700-22 V1.0.0

Agenda item:	8.7

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This pCR is to enhance Discovery Service APIs to support improvement in the API invoker on-boarding experience.

2. Reason for Change

We have the following KI to support enhancing the API invoker experience:

## 5.5 Key issue #5: Enhancing support to API Invoker on-boarding ### 5.5.1 Description Currently in 3GPP TS 23.222 clause 8.1 On-boarding of API Invoker to the CAPIF procedure, there is an assumption that API Invoker has sufficient API information to make decision of making an on-boarding request to CCF. However, in reality the API Provider exposes limited or only information that can be shared commonly to the API Invoker. The reason for this is beacause the trust relationship is yet to be established between the two parties (i.e. API Invoker and the API Provider). In another scenario, after the API Invoker successfully on boards to the CAPIF, the API Invoker realizes that certain features or services that the API Invoker wishes to consume with assistance of CAPIF, are not supported. Then the API Invoker may off board from the CAPIF or take certain other actions like not consuming northbound APIs via the registered CCF. In such a scenario, it is waste of resources for the API invoker and the CCF to perform the on boarding and maintain the API Invoker profile information. The features that API Invoker may be looking to consume from CCF and not supported by CCF, may include, the AEFs serving certain set of service API(s), availability of set of service APIs, support for certain security methods, support for certain security methods for certain AEFs / service APIs, interconnection with a given set of CCFs etc. To address these cases, it is essential to study enhancing support to API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources. ### 5.5.2 Open issues This key issue will study the following aspects: 1. How to enhance the support of API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources? 2. Any enhancements required to other CAPIF procedures e.g. Registering the API provider domain functions, Publish Service APIs to CCF?

Also, there are below architectural requirements in CAPIF TS 23.222, to restrict dissemination of service API information to the entities depending on the entity and the scenario, for e.g. un-authenticated API Invoker, interconnected CCF, type of API publishing entity, API serving area).

[AR-4.2.2-c] The CAPIF shall provide a mechanism to restrict the discovery of the published service API information by the API invokers, based on configured policies.

[AR-4.2.2-d] The CAPIF shall provide a mechanism to configure policies to restrict the discovery of the published service API information.

The use case for supporting yet to be onboarded API invoker for discovery is to enable anyone to discover publicly exposed API information, which might result in more onboarding requests and consumption of APIs in the API provider’s expectation.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 V1.0.0.

* * * First Change * * * *

## 6.x	Solution #x: Enhancing discover service APIs for API Invoker on-boarding

This solution is related to Key issue #5: Enhancing support to API Invoker on-boarding.

Currently in 3GPP TS 23.222 clause 8.1 On-boarding of API Invoker to the CAPIF procedure, there is an assumption that API Invoker has appropriate API information to make decision of making an on-boarding request to CCF. Also, there are architectural requirements in CAPIF to restrict discovery of service API information to the entities depending on the entity and the scenario, for e.g. un-authenticated API Invoker, interconnected CCF, type of API publishing entity, API serving area).

### 6.x.1	Solution description

This solution is related to Key issue #5, enhancing discover service APIs for better API Invoker on-boarding experience.

#### 6.x.1.1	Enhancing Discover service APIs

The Service API discover request and Service API discover response information flows in TS 23.222 clause 8.7 Table 8.7.2.1-1 and Table 8.7.2.2-1 respectively are enhanced (bold text) as proposed below.

Table 8.7.2.1-1: Service API discover request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information (NOTE 1) | M | Identity information of the API invoker discovering service APIs |
| Query information | O | Criteria for discovering matching service APIs (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), required API provider name (optional), UE IP address (optional), interfaces, protocols, Service KPIs (optional), and Network Slice Info (optional)). (see NOTE 2) |
| NOTE 1: If the API invoker is yet to be onboarded, this IE is not necessary. NOTE 2: It should be possible to discover all the service APIs. |  |  |

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1) | List of service APIs corresponding to the request, including API description such as service API name, service API type, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version, data format, Service KPIs (optional), and Network Slice Info (optional). Filtered service APIs information that can be shared to the API Invoker yet to be onboarded. |
| CAPIF core function identity information | O (see NOTE 1) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. |  |  |

The Discover service APIs procedure in TS 23.222 clause 8.7.3 is enhanced (bold text) as proposed below:

8.7.3	Procedure enhancements:

Figure 8.7.3-1 illustrates the procedure for discover service APIs.

The service API discovery mechanism is supported by the CAPIF core function.

Pre-conditions:

1.	The API invoker is onboarded and has received an API invoker identity or an API invoker yet to be onboarded.

2.	The CAPIF core function is configured with a discovery policy information (e.g. to restrict discovery to category of APIs) for API invoker(s).

3.	The CAPIF core function is aware of the Service API information that can be disseminated, depending on the requesting entity and the criteria.

NOTE:	CCF dissemination of service API information may be, for example, service API information that is accessible to all, service API information that is accessible to a specific requesting entity (e.g. API invoker, CCF), service API information that is allowed to be shared in specific criteria (API discovery prior to onboarding, API publish to interconnected CCF).

Figure 8.7.3-1: Discover service APIs

1.	The API invoker sends a service API discover request to the CAPIF core function. It includes the API invoker identity if already onboarded or API invoker information, and may include query information.

2.	Upon receiving the service API discover request, the CAPIF core function verifies the identity of the API invoker (via authentication), if already onboarded or not. The CAPIF core function retrieves the stored service API(s) information from the CAPIF core function (API registry) as per the query information in the service API discover request. Further, if the API invoker is yet to be onboarded, the CAPIF core function considers the entity requesting and applies the criteria to perform filtering of service APIs information,.

3.	The CAPIF core function sends a service API discover response to the API invoker with the filtered list of service API information as per the API invoker authorization.

### 6.X.2	Architecture Impacts

This solution is based on architecture of CAPIF as described in 3GPP TS 23.222 [2].

### 6.X.3	Corresponding APIs

This solution impacts the CAPIF_Discover_Service_API API as specified in clause 10.2 of 3GPP TS 23.222 [2].

NOTE:	During normative a new API may be introduced instead of enhancing existing CAPIF_Discover_Service_API API.

### 6.X.4	Solution evaluation

This solution addresses the open issue of enhancing the support to API Invoker on-boarding in KI #5, the impacted CAPIF procedure is the procedure of discover service APIs for limiting the information to be provided to the API Invoker prior to on-boarding.

* * * Next Change * * * *

## 9.5	Evaluation of key issue#5

Clause 5.5 captures the key issue on Enhancing support to API Invoker on-boarding. The following open issues are listed:

1.	How to enhance the support of API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources?

2.	Any enhancements required to other CAPIF procedures e.g. Registering the API provider domain functions, Publish Service APIs to CCF?

Solution#5 specified in clause 6.6 provides a proposal to address open issue#1 and open issue#2 by enhancing the API invoker onboarding with onboarding criteria information from API invoker and onboarding is completed based on meeting the onboarding criteria information. Further, the solution proposes CCF sending notification to the API invoker when CCF cannot meet the onboarding criteria information at later point of time after the API invoker is onboarded. This enables API Invoker with information to either renew enrolment or offboard from CCF.

Solution#17 specified in clause 6.18 provides a proposal to address open issue#1 and open issue#2 by re-using existing CAPIF event notification procedures and introducing new events to support the onboarded API invokers. The events related to security are to be coordinated with SA3.

Solution#x specified in clause 6.x proposes a solution to address open issue#5 by enhancing the discover service APIs for limiting the information to be provided to the API Invoker prior to on-boarding.

* * * Next Change * * * *

## 10.6	Conclusions of key issue #5

Solution#5 and Solution#17 can be considered as candidate for normative work for addressing the CAPIF event notifications for API invoker onboarding. The events related to security are to be coordinated with SA3.

Solution#5 can be considered as candidate for normative work for enhancements related to API invoker onboarding.

Solution#x can be considered as candidate for normative work for enhancements related to discover service APIs for improving the API Invoker experience prior to on-boarding.