3GPP TSG-SA WG6 Meeting #63	S6-244548

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244142, merge 4034)

Source:	Nokia, ZTE

Title:	Resolving Editor Notes in Solution#6

Spec:	3GPP TR 23.700-22 v1.0.0

Agenda item:	8.7

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Resolving Editor Notes in Solution#6

2. Reason for Change

There are 2 ENs which are to be resolved.

## 6.7 Solution #6: API instantiation based on service discovery ### 6.7.1 Solution description Figure 6.7.1-1 illustrates the procedure for Service API discovery procedure. Figure 6.7.1-1: Procedure to trigger service API instantiation 1-2. Same as step 1 and 2 in clause 8.7.3 of 3GPP TS 23.222 [2]. 3. If the CCF determines that the service API being discovered is not instantiated, then the CCF invokes AMF to perform service/AEF instantiation. The AMF is responsible for triggering service/AEF instantiation. Editor's Note: If service API level instantiation is not available by SA5 mechanisms, AMF would ask for the instantiation of the AEF providing such a service API. it is FFS whether SA5 can support service API level instantiation. 4. Same as clause 8.3.3 of 3GPP TS 23.222 [2]. 5. Same as step 3 in clause 8.7.3 of 3GPP TS 23.222 [2]. ### 6.7.2 Architecture Impacts Existing CAPIF architecture in 3GPP TS 23.222 [2] is used to accommodate new interactions between CCF and AMF in this solution. ### 6.7.3 Corresponding APIs This solution impacts the CAPIF-5/5e interface with a new AMF API for triggering service instantiation. ### 6.7.4 Solution evaluation Editor's note: This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v1.0.0.

* * * First Change * * * *

## 6.7	Solution #6: API instantiation based on service discovery

### 6.7.1	Solution description

Figure 6.7.1-1 illustrates the procedure for Service API discovery procedure.

Figure 6.7.1-1: Procedure to trigger service API instantiation

1-2.	Same as step 1 and 2 in clause 8.7.3 of 3GPP TS 23.222 [2].

3.	If the CCF determines that the service API being discovered is not instantiated, then the CCF invokes AMF to perform service/AEF instantiation. The AMF is responsible for triggering service/AEF instantiation.

NOTE:	Whether support for AMF request for service API level instantiation or for instantiation of AEF is to be resolved in coordination with SA5 during normative work.

4.	Same as clause 8.3.3 of 3GPP TS 23.222 [2].

5.	Same as step 3 in clause 8.7.3 of 3GPP TS 23.222 [2].

### 6.7.2	Architecture Impacts

Existing CAPIF architecture in 3GPP TS 23.222 [2] is used to accommodate new interactions between CCF and AMF in this solution.

### 6.7.3	Corresponding APIs

This solution impacts the CAPIF-5/5e interface with a new AMF API for triggering service instantiation.

### 6.7.4	Solution evaluation

This solution provides a proposal to address the open issues listed in key issue#7 by enhancing the service API discover mechanism to enable service API level instantation where CAPIF's AMF will interact with the OAM system specified by SA5 for such service API level or AEF level instantiation. Coordination with SA5 is required for normative work progress.