3GPP TSG-SA WG6 Meeting #63	S6-244556

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244329)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0214 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invoker onboarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per conclusion of key issue #5 in TR 23.700-22, solution #5 can be considered as candidate for normative work to enhance API invoker onboarding. This CR proposes to enhance the API invoker onboadring procedure with onboard criteria information, aliging to solution #5 in TS 23.700-22. Also, CAPIF events is updated to support new event from CCF to API invoker, when the onboading criterial from API invoker is no longer met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | API invoker onboarding enhanced to support criteria information. New CAPIF event to notify API invoker by CCF, when onboarding criteria is not met, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The study conclusions are not captured in normative specification. API invoker onboarding is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1, 8.1.3, 8.8.2.1, 8.8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

#### 8.1.2.1	Onboard API invoker request

Table 8.1.2.1-1 describes the information flow onboard API invoker request from the API invoker to the CAPIF core function.

Table 8.1.2.1-1: Onboard API invoker request

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding information | M | The information of the API invoker including enrolment details, required for onboarding |
| > Onboarding criteria | O | Indicates that the API Invoker wishes to on board itself to the CCF only if the CCF supports the features mentioned in criteria information. |
| >> Criteria information | M | This information includes atleast one of the criteria listed (List of AEFs information serving a certain set of service API(s), availability of AEFs serving a set of service APIs, supported security methods, supported security methods for given AEFs / AEF types/ service APIs/ Service API categories, interconnection with a given set of CCFs) for the CCF to onboard the API invoker. |
| APIs for enrollment | O | List of APIs being enrolled for. |
| Proposed expiration time | O | Proposed expiration time for the onboarding. |

* * * Next Change * * * *

### 8.1.3	Procedure

Figure 8.1.3-1 illustrates the procedure for onboarding the API invoker to the CAPIF. The security aspects of this procedure are specified in subclause 6.1 of 3GPP TS 33.122 [12].

Pre-conditions:

1.	The API invoker is not a recognized user of the CAPIF.

2.	The API invoker has visibility to APIs information (e.g., API catalogue or dashboard - central place for the API provider to manage which APIs are displayed, giving API invokers the ability to enroll for).

Figure 8.1.3-1: Procedure for onboarding the API invoker to the CAPIF

1.	For enrollment of the API invoker to be a recognized user of the CAPIF, the API invoker triggers onboard API invoker request towards the CAPIF core function, providing the information as required for the API management and criteria information as required to onboard the API invoker.

2.	The CAPIF core function begins the onboarding process by verifying whether all the necessary information has been provided to onboard the API invoker, and further initiates a grant process. If the onboard API invoker request message includes onboard criteria information, then the CCF verifies whether the CCF can support the onboard criteria information. The CCF further proceeds with API invoker onboarding procedure, only if the CCF supports the onboard criteria information from the API Invoker. Successful onboarding results in provisioning API invoker profile which includes identity for the API invoker. The authorization information and the list of APIs and the categories of APIs that the API invoker can access subsequent to successful onboarding may also be created. The CAPIF core function may create access control policy (see Table E-1) for the onboarded API invoker considering the network slice information.

NOTE 1:	Completion of onboarding process can require explicit grant by the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the grant process internally without the need of explicit grant by the CAPIF administrator.

NOTE 2:	The API invoker profile consists of at least the identity information for the API invoker, information required for the authentication and authorization by the CAPIF and the CAPIF identity information.

3.	If the API invoker has triggered the onboard API invoker request and is granted permission, the onboard API invoker response provides success indication including information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs.

4.	As a result of successful onboarding process, the CAPIF core function is able to authenticate and authorize the API invoker.

* * * Next Change * * * *

#### 8.8.2.1	Event subscription request

Table 8.8.2.1-1 describes the information flow for event subscription request from the subscribing entity to the CAPIF core function.

Table 8.8.2.1-1: Event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | The information to determine the identity of the subscribing entity |
| Event criteria | M | The event criteria include event type information like failure API invocation event, new API available event, API version change event, API location change event, API invoker status etc and other query information like service API identifier, service API name, onboarding criteria etc. |
| Notification reception information | O | The information of the subscribing entity for receiving the notifications for the event. |

* * * Next Change * * * *

### 8.8.6	List of CAPIF events

Table 8.8.6-1 provides a non-exhaustive list of CAPIF events.

Table 8.8.6-1: List of CAPIF events

| Events | Events Description |
| --- | --- |
| Availability of service APIs | Availability events of service APIs (e.g. active, inactive) |
| Service API updated | Events related to change in service API information |
| Monitoring service API invocations | Events corresponding to service API invocations |
| API invoker status | Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met.) |
| API topology hiding status | Events related to API topology hiding status in CAPIF (created, revoked) |
| System related events | Alarm events providing fault information |
| Performance related events | Events related to system load conditions |

* * * End of Changes * * * *