3GPP TSG-SA WG6 Meeting #63	S6-244569

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244169)

Source:	ETRI, Convida Wireless LLC, Nokia, Samsung

Title:	Spatial map procedure updates and Information Flows

Spec:	3GPP TS 23.437 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	Byung Jun AHN (bjahn@etri.re.kr)

1. Introduction

Spatial map procedures are updated to reflect “In-progress” create spatial map response and spatial map layering information. And information flow IEs and SEAL APIs are provided.

2. Reason for Change

“In-progress” response for create spatial map procedure and spatial map layering information has been added to the latest TR 23.700-21, which are not reflected to TS 23.437 v0.2.0 yet. Also, information flow IEs and SEAL APIs for spatial map procedures are required to be provided.

3. Conclusions

Updated procedure text, information flow IEs and SEAL APIs for spatial map procedures are provided.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437.

* * * First Change * * * *

9	Spatial Map (SM) management

9.1	General

The Spatial Map (SM) management is a SEAL service that offers the spatial map related capabilities to vertical application(s).

* * * Next Change * * * *

9.2.2.1.1	Spatial map (SM) server

The SM server acts as a SEAL server and it is used to provide services to create/update/delete/discover spatial map for the VAL application (i.e. metaverse application) and also provides spatial localization service. It can communicate with other SM server in the distributed SEAL deployments.

* * * Next Change * * * *

9.3	Spatial map management procedures

9.3.1	Create spatial map

Figure 9.3.1-1 depicts the procedure for creating a spatial map. For the request from VAL server or SM client, as a spatial map consumer, SM server creates a spatial map. A spatial map can be structured in layers where each layer signifies a specific aspect of the spatial information (e.g. three dimensional space of area of interest, objects).

Figure 9.3.1-1: Create spatial map procedure

1)	The VAL server (or SM client) sends a request message to the SM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map. The request may include spatial map layering information parameters (e.g., layer 1-basic space, etc.). The request may also include augmented layer information (e.g., VAL users, Spatial anchors, etc.) to be added on the top of the spatial map.

NOTE:	SM server may include the requested augmented layer information in the response message, but SM server does not store nor manage the augmented layer information further as a part of spatial map information.

2)	The SM server authorizes VAL server (or SM client), and validates the request. The SM server produces a requested spatial map using layering information and processed sensor data.

a)	If the augmented layer information indicates to include VAL users, the SM server fetches the list of VAL UEs in the area of interest from LM server using clause 9.3.10 of 3GPP TS 23.434 [4] and/or from NEF as specified in 3GPP TS 23.502 [5].

b)	If the augmented layer information indicates to include spatial anchors within area of interest, the SM server fetches the list of spatial anchors as specified in clause 8.3.4.

NOTE:	For the current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SM server gets access to such database to create a spatial map via application specific way which is out of SA6 scope.

3)	The SM server sends response message to the requestor with a indication of success, in-progress or fail. If the requested spatial map has been created sussesfully, the response message includes assigned spatial map ID and information which includes three-dimensional space defined by the spatial map and a list of spatial map layers with their corresponding layer ID, objects belong to the layer, etc. If the response indicates "in-progress", an assigned spatial map ID is included in the message, then the requestor subscribes spatial map event to receive notification when the spatial map is ready for the utilization (e.g. for localization service). Otherwise, the response includes an indication of failure and may include a reason for the failure.

Editor’s Note:	The final response (i.e. "success" or "failure” notification) after "in-progress" response in this procedure is FFS.

9.3.1.1	Information flows for Create spatial map

Table 9.3.1.1-1 descibes the information elements from VAL server (or SM client) to SM server for create spatial map request.

Table 9.3.1.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Information to include | O | Information to be included in the spatial map |
| > Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| > List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >> Spatial map layer information | O | A container to specify the corresponding spatial map layer specific information |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS; |

NOTE 1:	Further elements of the information to include can be defined.

Editor’s note: Details of Access control rules along with new terminology is FFS.

Table 9.3.1.1-2 descibes the information elements from SM server to VAL server (or SM client) for create spatial map response.

Table 9.3.1.1-2: Create spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Information on the created spatial map. It contains information as specified in Table 9.3.1.1-3. |
| > Spatial map information | M | ID of the created spatial map |
| > Augmented layer information | O | A container to carry the requested augmented layer information |
| > Spatial map ID | M | Assigned spatial map ID |
| Failure response | O (NOTE) | Indicates failed create spatial map |
| > Cause | O | Cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE 2:	Further elements of the spatial map information can be defined.

Table 9.3.1.1-3: Spatial map information

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the created spatial map |
| Spatial map coverage space | M | Three-dimensional space coordinates information which defines the spatial map |
| Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| List of spatial map layers | M | List of spatial map layer information. At least one or more spatial map layers shold be included. Each element includes the information described below. |
| > Spatial map layer ID | M | ID of the corresponding spatial map layer. |
| > Spatial map layer information | O | A container to carry the corresponding spatial map layer specific information |
| >> Layer status | O | Indicates whether to hide or show the layer. If this IE is not present, the layer will be shown by default. |
| >> area | O | Area covered by this layer (This area is same or subset of the Spatial map coverage space). |
| >> associated spatial map | O | Detailed spatial map associated with this layer. For example, if the layer is for a particular floor of the building, the detailed spatial map of that particular floor can be associated with this layer. |
| > List of objects | O | List of object information. Each element includes the information below. |
| >> Object ID | M | ID of the corresponding object |
| >> Object type | M | Type of object (stationary or moving) |
| >> Object position | O | Current 3D position of the object in the layer |
| >> Object specific information | O | A container to carry the corresponding object specific attribute information |

9.3.2	Discover spatial map

Figure 9.3.2-1 depicts the procedure for an authorized spatial map consumer (VAL server or SM client) to discover spatial map(s). The service is provided by SM server.

Figure 9.3.2-1: Discover spatial map procedure

1)	The VAL server (or SM client) sends a request message to the SM server to discover spatial map(s). The request includes requestor ID, security credentials, three-dimensional area of interest or localization information to discover spatial maps in the area, optional discovery filters, i.e., a set of characteristics (e.g., a list of specific spatial map layers) to search the matching spatial maps when multiple spatial maps are allowed in the area of interest.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this document.

2)	The SM server authorizes VAL server (or SM client), and validates the request. Then the server finds spatial map(s) which matches requested discovery requirements.

3)	The SM server sends the discover spatial map response message to the requestor with spatial map ID(s) when matching spatial map is found, otherwise it sends failure indication with reason.

9.3.2.1	Information flows for Discover spatial map

Table 9.3.2.1-1 descibes the information elements from VAL server (or SM client) to SM server for discover spatial map request.

Table 9.3.2.1-1: Discover spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Filtering criteria | M | Criteria to get spatial map(s) |
| > Area of interest | O (NOTE) | Three-dimensional area information to discover spatial maps |
| > Localization information | O (NOTE) | A known location to discover spatial maps in the three-dimensional space, including an orientation |
| > Optional discovery filters | O | A set of characteristics to search the matching spatial maps |
| >> List of spatial map layers | O | A list of spatial map layers to discover spatial map(s) with matching layers. Each element includes the information described below. |
| >>> Spatial map layer information | O | A container to specify the corresponding spatial map layer information |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE:	Further optional discovery filters can be defined.

Table 9.3.2.1-2 descibes the information elements from SM server to VAL server (or SM client) for Discover spatial map response.

Table 9.3.2.1-2: Discover spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Indicates successful discovery |
| > List of discovered spatial maps | M | List of discovered spatial anchor(s). Each element includes the information described below |
| >> Spatial map ID | M | ID of a spatial map discovered |
| Failure response | O (NOTE) | Indicates failed discovery |
| > Cause | O | Cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

9.3.3	Get spatial map information

Figure 9.3.3-1 depicts the procedure for an authorized spatial map consumer (VAL server or SM client) to get the spatial map. The service is provided by SM server.

Figure 9.3.2-1: Get spatial map information procedure

1)	The VAL server (or SM client) sends a request message to the SM server to get the spatial map information. The request includes requestor ID, security credentials, spatial map ID to get the information for it.

2)	The SM server authorizes VAL server (or SM client), and validates the request. Then the server collects the requested spatial map information.

3)	The SM server sends the get spatial map information response message to the requestor with requested spatial map information when successful, otherwise it sends failure indication with reason.

9.3.3.1	Information flows for Get spatial map information

Table 9.3.3.1-1 descibes the information elements from the VAL server (or SM client) to the SM server for get spatial map request.

Table 9.3.3.1-1: Get spatial map information request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Spatial map ID | M | ID of the spatial map to get information on it |
| > List of spatial map layers | O | A list of spatial map layers to get information. Each element includes the information described below. |
| >> Spatial map layer information | M | A container to specify the corresponding spatial map layer information including the ID of the corresponding layer. This IE may include layer visibility criteria which indicates to show or hide this layer. |

Table 9.3.3.1-2 descibes the information elements from the SM server to the VAL server (or SM client) for get spatial map response.

Table 9.3.3.1-2: Get spatial map information response

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the requested spatial map to get information |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Indicates successful get information |
| > Spatial map information | M | Information on the requested spatial map. It contains information as specified in Table 9.3.1.1-3. |
| Failure response | O (NOTE) | Indicates failed get information |
| > Cause | O | Cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

9.3.4	Update spatial map

Figure 9.3.4-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SM client), SM server updates the spatial map as requested.

Figure 9.3.4-1: Update spatial map procedure

1)	The VAL server (or SM client) sends a request message to the SM server to update a spatial map. The request includes requestor ID, security credentials, spatial map ID, information on what to update such as updated spatial map layering information, access control rules and updated spatial map coverage area, etc. As a part of update, the request may include augmented layer information (e.g., VAL users, Spatial anchors, etc.) to add an augmented layer on the top of the spatial map. SM server may include the requested augmented layer information in the response message, but SM server does not store nor manage the augmented layer information further as a part of spatial map information.

2)	The SM server authorizes VAL server (or SM client) and validates the request. The SM server updates the spatial map as requested.

3)	The SM server sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID.

9.3.4.1	Information flows for Update spatial map

Table 9.3.3.1-1 descibes the information elements from VAL server (or SM client) to SM server for update spatial map request.

Table 9.3.4.1-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Spatial map ID | M | ID of the spatial map to update |
| Requested update information | M | Information on what to update, e.g., access control rules or spatial map coverage area, etc. |
| > Spatial map coverage space | O (NOTE) | Three-dimensional space coordinates information which defines the spatial map |
| > Access control rules | O (NOTE) | Access control rules defining which entities are permitted to discover and access the spatial map |
| > List of spatial map layers to add | O (NOTE) | A list of spatial map layers to add. Each element includes the information described below. |
| >> Spatial map layer information | M | A container to specify the corresponding spatial map layer specific information |
| > List of spatial map layers to delete | O (NOTE) | A list of spatial map layer IDs to delete. Each element includes the information described below. |
| >> Spatial map layer ID | M | ID of the spatial map layer to delete |
| > Augmented layer information | O | A container to specify the requested augmented layer information to add. |
| NOTE: At least one of the information elements shall be provided. |  |  |

Table 9.3.4.1-2 descibes the information elements from the SM server to the VAL server (or SM client) for update spatial map response.

Table 9.3.4.1-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the requested spatial map to get information |
| Application service ID | O | ID of the requesting VAL application service |
| Success response | O (NOTE) | Indicates successful update |
| > Spatial map information | M | Updated spatial map information. It contains information as specified in Table 9.3.1.1-3. |
| > Augmented layer information | O | A container to carry the augmented layer information requested to add. |
| Failure response | O (NOTE) | Indicates failed update |
| > Cause | O | Cause of failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

9.3.5	Delete spatial map

Figure 9.3.5-1 depicts the procedure to delete the spatial map. For the request from the spatial map consumer (VAL server or SM client), SM server deletes the spatial map as requested.

Figure 9.3.5-1: Delete spatial map procedure

1)	The VAL server (or SM client) sends a request message to the SM server to delete the spatial map. The request includes requestor ID, security credentials, a spatial map ID to delete and optionally the reason to delete.

2)	The SM server authorizes VAL server (or SM client) and validates the request. The SM server deletes the spatial map as requested.

3)	The SM server sends response message to the requestor with the result of success or failure.

9.3.5.1	Information flows for Delete spatial map

Table 9.3.5.1-1 descibes the information elements from VAL server (or SM client) to SM server for delete spatial map request.

Table 9.3.5.1-1: Delete spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | Spatial map ID to delete |
| Reason to delete | O | Reason to delete |

Table 9.3.5.1-2 descibes the information elements from SM server to VAL server (or SM client) for delete spatial map response.

Table 9.3.5.1-2: Delete spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates successful delete |
| > Spatial map ID | O | Deleted spatial map ID |
| Failure response | O (NOTE) | Indicates failed delete |
| > Cause | O | Cause of failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

* * * Next Change * * * *

9.x	SEAL APIs for spatial map management

9.x.1	General

Table 9.x.1-1 illustrates the SEAL APIs for spatial map management.

Table 9.x.1-1: List of SEAL APIs for spatial map management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SmManagement | Create | VAL server, SM client | Request/Response |
|  | Discover | VAL server, SM client | Request/Response |
|  | Get | VAL server, SM client | Request/Response |
|  | Update | VAL server, SM client | Request/Response |
|  | Delete | VAL server, SM client | Request/Response |
|  | Subscribe | VAL server, SM client | Request/Response |
|  | Notify | VAL server, SM client | Notify |
|  | Unsubscribe | VAL server, SM client | Request/Response |

9.x.1	SS_SmManagement API

9.x.1.1	General

Service description: This API enables the VAL server or SM client to manage spatial maps on SM server.

9.x.1.2	Create operation

API operation name: SS_SmManagement_Create

Description: Create a spatial map in the given three-dimensional area of interest.

Known Consumers: VAL server, SM client.

Inputs: Refer Table 9.3.1.1-1

Outputs: Refer Table 9.3.1.1-2

See subclause 9.3.1 for the details of usage of this API operation.

9.x.1.3	Discover operation

API operation name: SS_SmManagement_Discover

Description: Discover a spatial map in the given three-dimensional area of interest.

Known Consumers: VAL server, SM client.

Inputs: Refer Table 9.3.2.1-1

Outputs: Refer Table 9.3.1.2-2

See subclause 9.3.2 for the details of usage of this API operation.

9.x.1.4	Get operation

API operation name: SS_SmManagement_Get

Description: Provide requested spatial map information to the authorized consumers.

Known Consumers: VAL server, SM client.

Inputs: Refer Table 9.3.3.1-1

Outputs: Refer Table 9.3.3.1-2

See subclause 9.3.3 for the details of usage of this API operation.

9.x.1.5	Update operation

API operation name: SS_SmManagement_Update

Description: Update the information on a spatial map as requested.

Known Consumers: VAL server, SM client.

Inputs: Refer Table 9.3.4.1-1

Outputs: Refer subclause 9.3.4.1-2

See subclause 9.3.4 for the details of usage of this API operation.

9.x.1.6	Delete operation

API operation name: SS_SmManagement_Delete

Description: Delete spatial map.

Known Consumers: VAL server, SM client.

Inputs: Refer Table 9.3.5.1-1

Outputs: Refer Table 9.3.5.1

See subclause 9.3.5 for the details of usage of this API operation.

9.x.1.7	Subscribe operation

API operation name: SS_SmManagement_Subscribe

Description: Process the authorized consumer’s subscription to the spatial map events.

Known Consumers: VAL server, SM client.

Inputs: Refer Table 9.3.6.1-1

Outputs: Refer Table 9.3.6.1-2

See subclause 9.3.6 for the details of usage of this API operation.

9.x.1.8	Notify operation

API operation name: SS_SmManagement_Notify

Description: Notify the information on the detected triggering events to which the authorized consumers are subscribed.

Known Consumers: VAL server, SM client.

Inputs: None

Outputs: Refer Table 9.3.7.1-1

See subclause 9.3.7 for the details of usage of this API operation.

9.x.1.9	Unsubscribe operation

API operation name: SS_SmManagement_Unsubscribe

Description: Revoke the authorized consumer’s subscription to the spatial map events.

Known Consumers: VAL server, SM client.

Inputs: Refer Table 9.3.8.1-1

Outputs: Refer Table 9.3.8.1

See subclause 9.3.8 for the details of usage of this API operation.

* * * End of Change * * * *