3GPP TSG-SA WG6 Meeting #63	S6-244579

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244123)

Source:	Convida Wireless LLC, Nokia

Title:	Digital Avatar Discovery

Spec:	3GPP TS 23.438 V0.0.0

Agenda item:	9.11

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com

1. Introduction

This contribution adds a procedure for discovering digital avatars.

2. Reason for Change

Solution #9 of TR 23.700-21 addresses key issues 3 and 5 for support of digital avatar and digital avatar discovery, respectively and was concluded for normative work. The solution has been adapted to use the A-DACM architecture. A user (e.g. represented by the requesting avatar identifier) via A-DACM client can perform the digital avatar discovery procedure to find avatars of other users.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.438 V0.0.0.

* * * First Change * * * *

## 8.X	Digital Asset Discovery

### 8.X.1	General

The following clauses specify procedures, information flows, and APIs for digital asset discovery.

### 8.X.2	Procedure

Figure 8.x.2.1-1: Digital Asset Discovery

1.	A requestor (e.g. DA client or VAL server) sends a digital asset discovery request to the DA server. The request includes information listed in Table 8.x.3.1-1.

2.	The DA server authenticates and authorizes the request. If authorized, the DA server applies the discovery criteria to discover digital assets from the local repository.

3.	The DA server generates and sends a response to the requestor. The response includes information listed in Table 8.x.3.1-2.

### 8.X.3	Information flows

#### 8.X.3.1	Digital Asset Discovery

Table 8.x.3.1-1 describes the information elements for a digital asset discovery request sent from a requestor (e.g. DA client or VAL server) to a DA server.

Table 8.x.3.1-1: Digital asset discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Requesting asset identifier | M | The identifier of the asset of the requestor. |
| Asset discovery filters | M | Set of characteristics to determine matching asset (e.g., asset identifier, asset name, asset description, location, VAL client type, Asset type, Owner user, Allowed user, Digital asset profile). |

Table 8.x.3.1-2 describes the information elements for the digital asset discovery response sent from a DA server to a requestor (e.g. DA client or VAL server).

Table 8.x.3.1-2: Digital asset discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Discovery results | M | List of discovery results matching the discovery criteria. |
| >Asset identifier | O (NOTE) | Identifier of the asset matching the discovery criteria. |
| >Asset name | O (NOTE) | Name of the asset matching the discovery criteria. |
| >Discovered asset profile identifier | O (NOTE) | Profile identifier of the digital asset matching the discovery criteria. |
| >URL | O | A URL of the service provider the asset is associated with. |
| >Asset status | M | Status of the asset. |
| >Asset descriptions | O | Description of the asset. |
| NOTE: At least one of the information elements must be present. |  |  |

Editor’s Note: Media aspects associated with digital assets are FFS.

### 8.X.4	APIs

* * * End of Change * * * *