3GPP TSG-SA WG6 Meeting #63	S6-244601

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244315)

Source:	KPN N.V.

Title:	Update to AIMLE client selection subscription and notification

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution includes a critrea for AIMLE cleint selection and also clarifies way in which the AIMLE Server determine the QoS parameters.

2. Reason for Change

The following Editors Notes are resolved with this contribution:

Editor’s Note: Further details in how AIML polices can be used for selection is FFS.

Editor’s Note: How the AIMLE Server determine the QoS parameters is FFS.

Editors Note: Further information elements and criterias are FFS.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * * First Change * * * *

## 8.13	AIMLE client selection subscription and notification

### 8.13.1	General

AIMLE client selection subscription request and notification enable VAL Servers to subscribe for monitoring and selection of AIMLE clients and receive notification when there is an update on the selected and re-selected AIML client’s status when re-selection is performed according to AIML member selection criteria.

The AIMLE server interacts with the NEF and/or SEAL services to monitor AIML members who meet the selection criteria and obtain their identifiers and configuring the AIML traffic session(s) between the VAL Server and the selected AIMLE Client(s) who meet the criteria. When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

### 8.13.2	Procedure

Figure 8.13.2-1: AIMLE client selection subscription and notification

1.	A VAL server sends an AIMLE client selection subscription request to the AIMLE Server. The AIMLE client selection subscription request includes information as described in Table 8.13.3-1 which includes selection criteria.

2.	The AIMLE server validates the AIMLE client selection subscription request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AIMLE client selection subscription request.

3.	The AIMLE server sends the AIMLE client selection subscription response to the VAL server.

4.	The AIME server monitors AIMLE clients whether they fulfil the selection criteria as provided in step 1. The AIMLE server interacts with the NEF and/or SEAL services (including SEALDD) to establish monitoring.

5.	The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients which do not fulfil the selection criteria. The AIMLE Server may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

The AIMLE server may determine the application QoS parameters based on the VAL Service ID or by using policies proviosioned by VAL Server.

NOTE:	In case of AIMLE server determining the application QoS parameters based on the VAL service ID, how it does this is upto implimention.

6.	The AIMLE Server notifies the VAL server about the selected and re-selected AIMLE clients e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B.

Editor’s Note: Service operation definition in the API clause is FFS.

Editor’s Note: The procedure for provioning policies by the VAL server is FFS.

### 8.13.3	Information flows

Table 8.13.3-1 shows the request sent by a VAL server to an AIMLE server for the Selected AIMLE Client selection subscription.

Table 8.13.3-1: AILME client selection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| VAL service ID | O | An identifier for the VAL service. |
| AIMLE client selection criteria | M | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | M | Represents the endpoint at the VAL server for receiving the notifications on the selected AIMLE Client’s status update. |

Table 8.13.3-2 shows the response sent by the AIMLE server to the VAL server for the selected AIMLE client selection subscription response.

Table 8.13.3-2: AIMLE client selection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |

Table 8.13.3-3 shows the notification sent by the AIMLE Server to the VAL server for the AIMLE client selection update notification.

Table 8.13.3-3: AIMLE client selection update notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor. |
| List of the selected AIMLE Client status update events | M | Represents the list of selected AIMLE Client status update events, e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B. |

* * * Next Change * * * *