3GPP TSG-SA WG6 Meeting #63	S6-244644

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244211)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Application calling service API

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the Application calling service exposure procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#1, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.4.1	Application calling service API with Data Channel capability

#### 8.4.2.1	General

The MMTel Enabler Server exposes the Application calling service API with Data Channel capability to the Applications in order to support establishing a call between Application and DCMTSI client.

#### 8.4.2.2	Procedure

##### 8.4.2.2.1	Establishing an Application Call with Data Channel capability

Figure 8.4.2.2.1-1 illustrates the operation to establish an Application Call with Data Channel capability between the Application and the MMTel Enabler Server.

Pre-conditions:

1.	The Application is authorized to use Application calling service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in SA2.

3.	The MMTel Enabler Server is authorized to use OMA AudioCall API as specified in OMA-TS-REST_NetAPI_AudioCall [x].

Figure 8.4.2.2.1-1: Establishing a Application Call with Data Channel capability

1.	The Application sends an Application Call with DC capability establishment request to the MMTel Enabler Server.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then:

a)	the MMTel Enabler Server can utilize the OMA OneAPI to establish a Call session with audio or video media as specified in OMA-TS-REST_NetAPI_AudioCall.

b)	the MMTel Enabler Server obtains the Session ID from the IMS session established in step a.

c)	the MMTel Enabler Server utilizes the DC update capability of the IMS core network to add DC media to the existing Application Call Session as specified in 3GPP TS 23.228 [x].

d)	the IMS core network returns the update result.

3.	The MMTel Enabler Server sends an Application Call establishment response to the Application.

Editor's note:	The detailed information elements included in the request/response are FFS and needs to be aligned with the parameters in the existing OMA APIs.

Editor's note:	How to use the existing IMS session management APIs provided by OMA in MMTel service, without impacting SA2 architecture and considering potential security requirement is FFS.

Editor's note:	The requirements to deploy the MMTel Enabler Server in a trusted 3rd party call service provider domain is FFS.

* * * End of Changes * * * *