3GPP TSG-SA WG6 Meeting #63	S6-244731

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244287/S6-244513/S6-244717)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0109 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEALDD enabled S&F transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Samsung, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-01, Solution#AE4 and #AE8 is concluded as the candidate solution to KI#7 Usage of S&F events information for the application enablement. When SEALDD is used to deliver the application traffic, the SEALDD server acts as the SEAL server in solution #AE4, AE#8 and is responsible for the data DL data control. In order to provide a to provide a differentiated transmission with S&F operation, a new transmission service is provided at the SEALDD server, i.e., SEALDD enabled transmission for S&F Satellite operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Introduce a new SEALDD transmission service; 2) Add a new clause to capture the procedures, information flows, service APIs corresponding to the new transmission service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VAL can not use SEALDD to deliver data when the UEs are in S&F Satellite operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {9.x, 9.x.1, 9.x.2, 9.x.2.1, 9.x.3, 9.x.3.1, 9.x.3.2, 9.x.3.3} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* Second Changes *****************************/

10	SEALDD services over Satellite Access

10.1	General

This clause provides clarifications on the procedures and information flows for enabling SEALDD services using satellite access.

10.2	SEALDD enabled transmission for S&F Satellite operation

10.2.1	General

This clause provides the services to support the transmission for delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) based on SEALDD capabilities including adapt the DL data delivery according to the S&F events from the 3GPP core network.

10.2.2	Procedures

10.2.2.1	SEALDD enabled data transmission for delay-tolerant satellite services

Figure 10.2.2.1-1 illustrate the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit its data between the VAL client and VAL server.

Pre-condition:

-	The VAL server discovers and select the SEALDD server by CAPIF functions.

Figure 10.2.2.1-1: SEALDD enabled data transmission for delay-tolerant satellite services

1.	The VAL server decides to use SEALDD service for delay-tolerant traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_DTwithS&FTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates the SEALDD-S data transmission connection information (e.g., address/port) of SEALDD server side to receive the application data packets from the VAL server to be delivered to the VAL UE. The SEALDD server responds with a Sdd_DTwithS&FTransmission response.

NOTE 1:	The SEALDD-S data transmission connection information of the SEALDD server side is optional, if the SEALDD server uses the downlink pull mode to fetch the application data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the application data to the address provided by VAL server.

3.	Same as step 3-9 of clause 9.2.2.2.

4.	The SEALDD server subscribes the UE S&F status events from the 3GPP CN and is notified about the UE S&F status events information e.g., whether UE is registered in S&F Mode, the estimated delivery time indicating the DL estimated time to send data from the UE to gateway or from gateway to UE. The SEALDD server stores or the received UE S&F status events information for further control the downlink application data transmission.

5.	The VAL server sends downlink application data to the SEALDD server which further delivers it to the VAL UE.

NOTE 2:	The SEALDD server may receive the DLapplication data from the VAL server in step 10 before receiving the UE S&F status events notification from the 3GPP CN in step 9.

6.	The SEALDD server controls the downlink application data delivery to the SEALDD client based on the UE S&F status events information, when the UE is in S&F mode:

-	pending the DL data transmission when the Feeder Link is not available;

-	(re-)transmitting the pending DL data when the Feeder Link is available.

-	DL data volume to be transmitted is smaller than maximum S&F data storage quota per application per UE.

-	If DL estimated delivery time is received from 3GPP CN, it may be used by the SEALDD server to estimate whether the transmitted DL application data is successfully delivered to the UE and its corresponding storage is available for new DL data. For each Feeder Link available period, the SEALDD server starts an estimated delivery timer based on the DL estimated delivery time which is corresponding to the current feeder link. The estimated delivery timer may be greater than or equal to the DL estimated delivery time. When in next Feeder Link available period, the SEALDD server checks the previous estimated delivery timer is expired or not before transmitting the DL data. Also, the maximum S&F data storage quota per application per UE is also considered.

a.	If timer expires, the SEAL server calculates the new DL data volume (e.g., increasing the volume by certain data size (e.g., size of data previously delivered to the SEALDD client)).

b.	If timer not expire, the SEALDD server also checks the available data volume within the maximum S&F data storage quota. The SEALDD server may send DL data limited within the available data volume. If no available data volume, the SEALDD server pends the DL data transmission until the timer expires.

Editor’s note:	whether the SEALDD server will consider the forwarding with higher priority policy for the DL data delivery is FFS.

7.	The SEALDD server sends the DL data to the VAL UE via the 3GPP network as indicated in step 11.

8.	Optionally, the SEALDD server may send Sdd_DTwithS&FTransmission notification to the VAL server to influence the DL application data transmission from VAL server if the VAL UE connection status changes or the DL data transmission from the VAL server needs to be adjusted.

10.2.3	Information flows

10.2.3.1	SEALDD enabled DTwithS&F transmission request

Table 10.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the delay-tolerant with S&F application transmission service.

Table 10.2.3.1-1: SEALDD enabled DTwithS&F transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | M | Identifier of specific UE or VAL user |
| SEALDD-S Data transmission connection information | M | Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. |
| VAL UE connection status subscription indication | O | Indicates the VAL server expected to receive the connection satus notification |

10.2.3.2	SEALDD enabled DTwithS&F transmission response

Table 10.2.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the delay-tolerant with S&F application transmission.

Table 10.2.3.2-1: SEALDD enabled DTwithS&F transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S information Data transmission connection information | O | Address/port and/or URL of the SEALDD server to receive the packets from the VAL server for application traffic transfer |
| Cause | O See NOTE | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE: The IE is only present if the Result is failure. |  |  |

10.2.3.3	SEALDD DTwithS&F transmission notification

Table 10.2.3.3-1 describes the information flow from the SEALDD server to the VAL server to notify S&F delivery related events.

Table 10.2.3.3-1: SEALDD connection status notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (NOTE) | Identifies event SEALDD client connection status e.g., reachable, unreachable, sleeping. |
| DL data delivery instructions | O (NOTE) | Indicates the intructions to the VAL server regarding the DL data delivery |
| > Adjust DL data volume | O | Indicates adjusting DL data volume with suggested traffic volume |
| > Pending DL data delivery | O | Indicates pending DL data delivery due to e.g., feerder link is not available |
| > Resume DL data delivery | O | Indicates pending DL data delivery due to e.g., feerder link is available |
| Identity | M | Identifier of VAL UE or VAL user. |
| VAL service ID | O | Identity of the VAL service. |
| NOTE: Either Event ID IE or the DL data delivery instruction ID is present. |  |  |