3GPP TSG-SA WG6 Meeting #64	S6-245334

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245016)

Source:	FirstNet

Title:	Pseudo-CR on Generic IOPS – Correctrion to 6.6.2, 6.9.2 (Section 10.4 of TS 23.180)

Spec:	3GPP TR 23.700-09, v0.3.0

Agenda item:	8.8

Document for:	Approval

Contact:	Mark Lipford, mark.lipford@firstnet.gov

1. Introduction

This pCR adds a correction in section 7.5.1.8 (in TR section 6.6.2) and 10.4.5.2.1 (in section 6.9.2) of TS 23.180 w of the TR.  Agreed at SA6 #63 it was agreed to add “or PDU session” with any occurance of “active PDN connection,” and to change “unicast bearers” to “unicast sessions”  This pCR makes these corrections where they were missed.

2. Reason for Change

To complete the agreed changes

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to section 6.6.2 and 6.9.2 of 3GPP TR 23.700-09, v0.3.0.

* * * First Change * * * *

#### 7.5.1.8	Reference point MC-IOPS-5 (between the IOPS MC connectivity function and the local IOPS 3GPP system)

The MC-IOPS-5 reference point, which exists between the IOPS MC connectivity function and the local IOPS 3GPP system, is used to obtain unicast sessions with appropriate QoS from the local IOPS 3GPP system. The MC-IOPS-5 reference point utilizes the Rx interface of the EPS, or N5 interface of 5G system according to 3GPP TS 23.203 [8] or 3GPP TS 23.503[23.503], respectively.

* * * Second Change * * * *

##### 10.4.5.2.1	Group Session connect over broadcast/multicast session

In figure 10.4.5.2.1-1 the MC service client 1 is the client that initiates a group session, e.g. an MCPTT client initiating a group call. The MC service client 1 may be within the broadcast/multicast  service area. The MC service clients 2 … n represent MC service clients, e.g. MCPTT clients, receiving the group session packets over an MBMS bearer. There may be other receiving MC service clients both over unicast sessions and over this or other broadcast/multicast session(s), however, they are not illustrated in this figure.

Pre-conditions:

-	The MC service user profile used for the IOPS mode of operation is pre-provisioned in the MC service UEs

-	The IOPS MC service group ID and its associated IOPS group IP multicast address are pre-configured in the MC service clients

-	The MC service users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality

-	The MC service users are discovered by the IOPS MC connectivity function to support MC services based on the IP connectivity functionality

* * * End of Changes * * * *